SA WG2 Meeting #142E (e-meeting)	S2-2008700

Nov 16 – Nov 20, 2020, Elbonia

Source:	CATT

Title:	KI#7 conclusion update

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes the conclusion update for Key Issue #7.

- Discussion

In S2-2006044 "LS on RAN impact of FS_5MBS Study", one of the questions is as follows:

"4.	Some solution suggests the 5GC sends MBS assistance information to RAN for PTP/PTM delivery method decision and switching.

SA2 would appreciate RAN2 and RAN3 feedback on the above and comments, if any."

Discussion in RAN WG2/3 on the LS including feedback to this question is still ongoing, where many companies seem to agree that MBS assistance information from the 5GC is needed for RAN to decide on selecting/switching between PTP and PTM delivery methods. Assuming that the feedback from RAN WG2/3 is yes (i.e. MBS assistance information from the 5GC to RAN is needed), an update to the conclusion for KI#7 on this aspect is proposed.

Regarding "switching between 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery method", currently it is only addressed in the conclusion for "delivery method switching due to mobility". However, switching between 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery method shall also be performed based on network policy, service requirements, etc., triggered by the MB-SMF, PCF and/or MBSF, as proposed in solution #23, #24, #25, #28 and #31. So it is proposed to also address this in the conclusion for KI#7.

# 2. Proposal

It is proposed to include the following changes in TR 23.757.

***** Start of 1st Change *****

# 8	Conclusions

## 8.X	Key Issue #7: Reliable delivery method switching between unicast and multicast

For delivery method switching due to mobility, the following principle are agreed,

- When the UE moves from a NG-RAN node that supports 5MBS to a RAN node that does not support 5MBS, the network and UE shall support switch from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- When the UE moves from a RAN node that does not support 5MBS to a NG-RAN node that supports 5MBS, the network and UE shall support switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

Editor’s Note: It is FFS before the handover procedure, whether the associated PDU Session needs to be activated and the UE and NG-RAN need to be aware of the linkage between the MBS Session and the associated PDU Session before mobility.  If yes, 5GC needs to provide this information to UE and NG-RAN.

- During the handover from RAN not supporting 5MBS to NG-RAN supporting 5MBS, PDU sessions, including the one associated with MBS session, are handed over to target RAN. After the handover, the switch is triggered at the 5GC from the 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

Editor’s Note: For the switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method, it is FFS which the trigger NF is since it is solution dependent.

- For the handover from NG-RAN supporting 5MBS to RAN not supporting 5MBS, the MBS Session flow is converted to the QoS flow within the associated PDU session at the target RAN.

Editor’s Note: It is FFS whether the support for lossless handover with data forwarding from source NG-RAN to the target NG-RAN is needed, which needs confirmation by RAN.

For delivery method switching not due to mobility, the following principle are agreed,

- Switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery shall be supported. NG-RAN is the decision point for of switching the PTP and PTM delivery methods. The NG-RAN may use the MBS assistance information provided by the 5GC for decision on selecting/switching between PTP and PTM delivery methods.

- If the NG-RAN node support 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer.

NOTE:  	When a UE is handed over from an NG-RAN node not supporting 5MBS, Individual delivery may still be used.

- Switching between 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery method shall be supported based on network policy, service requirements, etc.

Switching between multicast delivery and unicast delivery using individual UE and application server addresses is not specified in normative work.

***** End of Changes *****