SA WG2 Meeting #142e	S2-2009001

# Nov16th – 20th , 2020 ; Elbonia                   		 	(revision of S2-2004841)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2385 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of the mapping from IP addressing information provided to an AF to the user identity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2020-28-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | (relates with LS in S2-2008358) An AF may only be able to identify the UE target of an AF request for Data Provisioning or for Event Exposure about a UE by providing UE addressing information (e.g. IP address and port information). |  |  |  |  |  |  |  |  |
| Summary of change: |  | In this case the 5GC first needs to retrieve the Permanent identifier of the UE before trying to fulfil the AF request. The 5GC may to determine the Permanent identifier of the UE based on: - the UE addressing information (e.g. IP address and port information) of the UE as provided by the AF: this may correspond to an UE IP address as allocated by 5GC or to an IP address (and port) that has been NATed (Network and Port Address Translation) by an entity controlled by the 5GC operator. In the latter case the 5GC (NEF) needs to first get from the NATF (5GC NF that has carried out NAT at the user plane) the translation back from the IP addressing information provided by the AF to the IP address that the 5GC has allocated to the UE. - the corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or determined by the NEF using the identity of the AF; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SA6 requirements are not supported. Not possible to request data provisioning or data retrieval about an individual UE that the application can only identify by its IP addressing information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 23.502 CR2329 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR is only provided for technical endorsement. No creation of R17 version of the specification is meant |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 5.20	External Exposure of Network Capability

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability and Analytics reporting capability. The Monitoring capability is for monitoring of specific event for UE in 5G System and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5G System. The Policy/Charging capability is for handling QoS and charging policy for the UE based on the request from external party. The Analytics reporting capability is for allowing an external party to fetch or subscribe/unsubscribe to analytics information generated by 5G System.

Monitoring capability is comprised of means that allow the identification of the 5G network function suitable for configuring the specific monitoring events, detect the monitoring event, and report the monitoring event to the authorised external party. Monitoring capability can be used for exposing UE's mobility management context such as UE location, reachability, roaming status, and loss of connectivity. AMF stores URRP-AMF information in the MM context to determine the NFs that are authorised to receive direct notifications from the AMF. UDM stores URRP-AMF information locally to determine authorised monitoring requests when forwarding indirect notifications.

Provisioning capability allows an external party to provision the Expected UE Behaviour or the 5GLAN group information or service specific information to 5G NF via the NEF. The provisioning comprises of the authorisation of the provisioning external third party, receiving the provisioned external information via the NEF, storing the information, and distributing that information among those NFs that use it. The externally provisioned data can be consumed by different NFs, depending on the data. In the case of provisioning the Expected UE Behaviour, the externally provisioned information which is defined as the Expected UE Behaviour parameters in TS 23.502 [3] clause 4.15.6.3 or Network Control parameter TS 23.502 [3] clause 4.15.6.3a consists of information on expected UE movement, Expected UE Behaviour parameters or expected Network Configuration parameter. The provisioned Expected UE Behaviour parameters may be used for the setting of mobility management or session management parameters of the UE. In the case of provisioning the 5GLAN group information the externally provisioned information is defined as the 5GLAN group parameters in TS 23.502 [3] clause 4.15.6.7, and it consists of some information on the 5GLAN group. The affected NFs are informed via the subscriber data update as specified in TS 23.502 [3] clause 4.15.6.2. The externally provisioned information which is defined as the Service Parameters in clause 4.15.6.7 of TS 23.502 [3] consists of service specific information used for supporting the specific service in 5G system. The provisioned Service Parameters may be delivered to the UEs. The affected NFs are informed of the data update.

Policy/Charging capability is comprised of means that allow the request for session and charging policy, enforce QoS policy, and apply accounting functionality. It can be used for specific QoS/priority handling for the session of the UE, and for setting applicable charging party or charging rate.

Analytics reporting capability is comprised of means that allow discovery of type of analytics that can be consumed by external party, the request for consumption of analytics information generated by NWDAF.

An NEF may support CAPIF functions for external exposure as specified in clause 6.2.5.1.

An NEF may support exposure of NWDAF analytics as specified in TS 23.288 [86].

An AF may only be able to identify the UE target of an AF request for Data Provisioning or for Event Exposure about a UE by providing IP addressing information (IP address and port information) about this UE. In this case the 5GC first needs to retrieve the Permanent identifier of the UE before trying to fulfil the AF request.  The 5GC may to determine the Permanent identifier of the UE based on:

- the IP addressing information (IP address and port information) of the UE as provided by the AF: this may correspond to an UE IP address as allocated by 5GC or to an IP address (and port) that has been NATed (Network and Port Address Translation) by an entity controlled by the 5GC operator. In the latter case the 5GC (NEF) needs to first get from the NATF (5GC NF that has carried out NAT at the user plane) the translation back from the IP addressing information provided by the AF to the IP address that the 5GC has allocated to the UE.

- the corresponding DNN and/or S-NSSAI information: this may have been provided by the AF or determined by the NEF using the identity of the AF;

NOTE:	When the AF has been triggered by an application running on an Edge Application Server that only identifies an UE by its IP addressing information, the AF is only be able to identify the UE by providing IP addressing information for this UE;

NEXT CHANGE (2)

NEXT CHANGE (5)

END OF CHANGES