3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008609

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1: Conclusion update

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution introduces the conclusion of KI#1.

- Introduction

  - Background

The conclusion proposal for KI#1 was agreed in SA2#141E, and several open issues is to-be-studied. This document is trying to address these open issues and finalize the conclusion part.

  - On the Residual Open Issues

After SA2#141E, there are still some open issues, namely:

- NF that Handling of MBS Session context for multicast (including knowledge about UE participation in multicast session): Whether it should be AMF, SMF, MBSF-C, or some of them.

The AMF is the NF that used for handling RM/CM related issues, and when comes to MBS, the AMF is not assumed to participant in such SM related handling. Moreover, in transport only mode, there could be not MBSF-C (as mentioned in A.3), therefore, it would be not necessary to involve MBSF-C in the handling of multicast session.

- PDU Session association timing: Whether the association shall be done when UE joins, or it would be done just before the Handover to the non-supporting RAN for MBS, or it would be done after the Handover procedure.

Such issue is related to KI#7 and in summary, association the PDU Session with multicast session when the UE join shall be supported, since establishing the association relationship would be time-consuming and the service continuity could not be efficiently supported. Therefore, the PDU session associated with the MBS session and used for individual delivery method need to be established and the user plane of the associated PDU session need to be activated before the inter RAN node handover procedure is triggered. Also the UE and NG-RAN need be aware of the linkage between the MBS Session and the associated PDU Session.

- The MBS Session anchor for multicast: Whether it should be MB-UPF, or it should be MBSF-U.

In A.3, there could be no MBSF in the deployment, therefore, at least for that case the MB-UPF should be used as the MBS Session anchor. For striking the unified deployment, it is suggested to use MB-UPF as the MBS Session anchor for multicast.

Note that the UP-based UE join option is another open issue and the analysis could be found in clause 1.5.

Proposal 1: Reflecting the above-mentioned analyses in the TR.

  - Analyses on ENs

Here are the summary of the ENs included in clause 7.5 for KI#1.

# Table 1.3-1 Summary of the ENs

| EN description | Analyses | Way to resolve this EN |
| --- | --- | --- |
| It is FFS whether or not the evaluation criteria should include the requirement that the application should be not aware of 5GS specific or internal information. | There could be different scenarios/deployment options for MBS, and the application could be aware/unaware the 5GC internal information, like LTE MBMS’s design. | Clarify that depending on deployment option, it shall be possible that the 5GC specific or internal information is agnostic to the application. |
| Further conclusion continues. | Since it is considered that the open issues are already addressed; therefore, it would be OK to delete this EN. | Remove this EN. |
| It is FFS whether or not to use the Native MBS Transport ID in addition. The Native MBS Transport ID (Similar as PDU session ID) could be assigned by the NEF when neither TMGI nor IP multicast address are used. It is FFS whether the TMGI is only used on external interfaces and mapped towards a source specific IP multicast address for communication within the 5GC. | If DN cannot configure IP Multicast, the tunnelling on N6 interface shall be supported. Since TMGI is made up by PLMN ID + PLMN specific temp ID, while Native MBS ID is Even though we adopt various MBS Session ID, we need to provide a single format MBS Session ID to the RAN, at least for the broadcast scenario since RAN shall broadcast the information. | The following proposal could be a way forward: - For broadcast scenario, TMGI shall be used. - For multicast scenario, TMGI/IP multicast address/Native MBS Transport ID could be used, for the interaction related with RAN, it is suggested to use only IP multicast address. - The TMGI could be allocated by either NEF or MB-SMF. |
| This depends on solution. | This EN is related to “The Multicast Service Context applies when the MBSF is used”. Last meeting it was agreed the baseline architecture depicted in A.3, while in Configuration Option 1, there is no MBSF and the potential usage is for Transport Only Mode, or Broadcast service does not require service layer interworking with LTE MBMS. Considering this option, and given that the multicast service context is for Full Service Mode, it is reasonable to confirm this assumption. | Remove the EN. |
| It is FFS whether other criteria is needed, e.g. handling of legacy UEs. | This issue is related to the details of call flow of the Session join. Could be resolved during the normative phase. | Remove the EN and add a note for this part. |
| Whether an MBS session deactivation and activated is supported relies on RAN WG feedback. | Possible update according to RAN WGs. | Add a new general section mentioning, “Any Editor's notes relevant for the conclusions, e.g. related to other WGs dependency, will be resolved in the normative phase.” |
| This list of conclusions is non-exhaustive. | Similar as the 2nd bullet. | Similar as the 2nd bullet. |
| Coordination with RAN WGs are needed. | Possible update according to RAN WGs. | Similar as “Activation/Deactivation” part. |

Proposal 2: Address the ENs in the document as suggested above.

  - On Broadcast aspect

We need to finalize the conclusion on broadcast.

Among the four solutions on the table, the differences are not that significant: the basic call flows follow the same paradigm in MBMS, and all the solutions assumes that the SMF handling broadcast session could interact with PCF for fetching the dynamic PCC rule for the MBS Session for broadcast. Moreover, among the solutions, two of them (i.e., Solution #5 and Solution #44) specify that the UE may not need to register to the network, or interact with AF via PDU Session.

Besides the already-mentioned aspects in the document, some more requirements similar as multicast need to be considered for broadcast, e.g., authorization by 5GC for the AF request, QoS related issues, etc.

Therefore, it is proposed to update the conclusion part for broadcast.

Proposal 3: Adding bullets for broadcast for recording the principles as mentioned-above.

  - On the User-Plane Support

Companies have been discussing whether the UP-based join shall be supported for MBS for multicast since 139E.

UP-based join is an option of UE joining the multicast service, by sending the IGMP Join/MLD report request to the Gateway NF (i.e., PSA UPF) via a certain established PDU Session. The PSA UPF notifies the SMF handling the PDU Session after snooping the join request. Hereafter referred to as “UP-based join” for simplicity.

There are some ENs in the evaluation/conclusion part saying that

- “Further text updates to reflect the possibility of tunnelling multicast through the public internet using RFC 8777 are required.”

- “It is FFS if the network and UE shall support multicast session join/leave operation via UP e.g. IGMP Join/Leave.”

- “UP Join is FFS.”

Those EN were added in 141E because companies wanted more time to check the details on the IP multicast usage in public network. Note that the AMT technology is not aiming for resolve the issue in Private network only (see 4.1.4.1 in 7450), and RFC 7450 and RFC 8777 give some examples on deploying the AMT relays in the public network (e.g., 2.3.2.2 in RFC 8777 and 4.1.4.1 in 7450). The on-going moderated email discussion is also trying to resolve this issue.

It is reasonable to add new content in clause 7.5.1.2 and 8.2.1 on RFC 8777 to resolve the ENs respectively.

Moreover, The EN in architecture baseline A.3.3.6 could be resolved as well.

Proposal 4: Add new content on UP-based join in clause 7.5.1.2 and 8.2.1, and remove the ENs in A.3.3.6, 7.5.1.2, and 8.2.1.

  - Miscellaneous clarification

It shall be notified that:

- Any solutions or other descriptions as far as not relevant for the conclusions are not further updated. Also any Editor's notes not relevant for the conclusions may remain in the TR.

- Any Editor's notes relevant for the conclusions, e.g. related to other WGs dependency, will be resolved in the normative phase.

Proposal 5: Document such clarification in a separate clause for avoiding confusion.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.757.

# * * * * First change * * * *

8.X	General

- Any solutions or other descriptions as far as not relevant for the conclusions are not further updated. Also any Editor's notes not relevant for the conclusions may remain in the TR.

- Any Editor's notes relevant for the conclusions, e.g. related to other WGs dependency, will be resolved in the normative phase.

# * * * *Second change* * * *

8.1	Key Issue #1: MBS session management

8.1.1	Interim requirements for conclusions

Conclusions will take into account the following agreed system requirements:

-	For multicast solutions, signalling from the UE to the network to join a multicast session shall be supported by UE and network. Join/leave operation via CP (NAS) signalling and UP (IGMP join or MLD) shall be supported.

-	For N3 transport of the shared delivery method, GTP-U tunnelling using a transport layer IP multicast method and shared N3 (GTP-U) Point-to-Point tunnel shall be supported with support for QoS.

-	Both 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method shall be standardized for multicast data delivery.

-	The network shall be able to prepare and start the multicast traffic transmission for a MBS session after MBS service is started.

-	The network shall support selection of MB-SMF or SMF (depending on solution) at session join.

-	For N3 transport of the 5GC shared MBS delivery method, for unicast transport there shall be 1-1 mapping between MBS Session and GTP-U tunnel towards a RAN node, and for multicast transport there shall be 1-1 mapping between MBS Session and the GTP-U tunnel.

-	Depending on deployment option, the 5GC specific or internal information could be agnostic to the application.

8.1.2	Conclusions

- For MBS session management the following conclusions are reached as baseline for normative work: The MBS session is identified throughout the 5G system transport on external interface towards AF and between AF and UE, and towards the UE with an MBS Session ID.

-	MBS Session ID can have the following types: TMGI, source specific IP multicast address.

-	Source specific IP multicast address can be assigned by 5GC or external network.

-	If the MB service is broadcast, TMGI shall be used; if the MB service is multicast, TMGI/source specific IP multicast address/Native MBS Transport ID is used, and only the Source specific IP multicast address will be used for the interactions with RAN.

-	The MBS Session ID could be allocated by MB-SMF or NEF.

-	The Multicast session model is depicted in Figure 8.1.2-1, with the following conclusions:

-	The Multicast Service Context applies when the MBSF is used.

-	For support of 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method:

-	Both 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method shall be standardized for multicast data delivery. 5GC Shared MBS traffic delivery method is always mandatory, and 5GC Individual MBS traffic delivery is required to support UE mobility to/from non MBS-capable NG-RAN nodes, but otherwise optional.

-	The network shall be able to support selection of 5GC Shared MBS traffic delivery method or 5GC Individual MBS traffic delivery method based on criteria of whether RAN node supports 5MBS or not.

NOTE:	5GC individual MBS traffic delivery could be used for the legacy UE, and the details will be defined in normative phase.

-	MB-UPF acts as the MBS session anchor when 5GC shared MBS traffic delivery method is used, and UPF acts as the unicast session anchor when 5GC individual MBS traffic delivery method is used.

-	Establishment of the associated PDU Session for 5GC Individual MBS traffic delivery method is based on service requirements, networking configuration, local policy, etc.

-	It shall be possible to establish an Associated PDU session for cases where mobility to non-5GMBS-supporting cells happens.

-	It shall be possible to establish the Associated PDU session when the UE joins the MBS Session.

Figure 8.1.2-1: Merged MBMS session model

-	For multicast session establishment/join/leave/release:

Editor's Note: The support for the mechanism of AF requested MBS session establishment is FFS.

-	UE shall support multicast session join/leave operation via CP (NAS signalling for SM procedure, i.e., PDU Session Modification) and UP (IGMP join or MLD).

-	The network shall support selection of MB-SMF or SMF at session join.

-	For N3 transport of the shared delivery method, GTP-U tunnelling using a transport layer IP multicast method and shared N3 (GTP-U) Point-to-Point tunnel shall be supported with support for QoS.

-	For N3 transport of the 5GC shared MBS delivery method, for unicast transport there shall be 1-1 mapping between MBS Session and GTP-U tunnel towards a RAN node, and for multicast transport there shall be 1-1 mapping between MBS Session and the GTP-U tunnel.

-	For N9 transport of the 5GC Individual MBS traffic delivery method, GTP-U tunnelling using a transport layer IP multicast method and shared N9 (GTP-U) Point-to-Point tunnel shall be supported.

-	For N9 transport of the 5GC Individual MBS traffic delivery method, for unicast transport there shall be 1-1 mapping between MBS Session and GTP-U tunnel towards a UPF, and for multicast transport there shall be 1-1 mapping between MBS Session and the GTP-U tunnel.

-	The network may support indicating of N6 tunnel information for receiving traffic of a MBS session to the AF.

-	For multicast service parameters storage, the UDR shall be able to store the AF provisioned or preconfigured service parameters per MBS session, the PCF shall be able to provide policy and QoS requirement per MBS session to the MB-SMF.

-	For UE receiving MBS traffic moving from one RAN node to another in CM-CONNECTED and RRC-CONNECTED state, handover procedure with MB context shall be supported by UE and network.

-	When MBS session is released, the N3 transport of the 5GC shared MBS delivery method is released and the radio resource associated with the MBS QoS Flows are released, or the N3/N9 transport of the 5GC Individual MBS traffic delivery method is released and the radio resource associated with the QoS Flows are released.

-	MB-UPF is used as the MBS Session anchor for multicast.

-	For the support of broadcast:

-	UE does not need to request to join an MBS session for receiving MBS data.

-	To receive the data of broadcast communication service, the UE is either preconfigured with needed configuration (e.g. USD) for the UE to receive MB service, or provisioned with the configuration of broadcast session which may be performed using SIP signalling or methods.

-	When the needed configuration is pre-configured, the UE does not need to interact with 5GC or AF.

-	The network shall support the authorization of the AF request for MBS session establishment.

Editor's Note:	Whether an MBS session deactivation and activated is supported relies on RAN WG feedback.

Editor's Note: Coordination with RAN WGs are needed.

# * * * * Third change * * * *

7.5.2.1	Multicast solutions

Trigger for establishment of multicast sessions in the PLMN

Many solutions (e.g. Solution 2, 3, 4, 6) suggest that a multicast session is created in the PLMN when the AF requests a reservation of resources for a multicast session. Some solutions also allow the creation of a multicast session based on configuration. Some solutions (e.g. solution 3, 4) in addition allows for a dynamic creation of multicast sessions in the PLMN without any AF interaction and configuration when it is observed that a UEs joins a multicast session in an external network (e.g. via IGMP join using a source specific multicast address of the external network). All options seem to offer advantages.

Regarding the dynamic trigger for a creation of multicast sessions without any configuration:

-	It is not enough that clients activate the reception of a Multicast group, there needs to be a sender. Thus, application level service announcement and/or service negotiation is assumed. The Multicast sender may not be aware whether there are zero or more receivers.

-	Support of IP Multicast on the public internet (external Data Network) is not widespread since IGMP is not used between routers and PIM or other routing protocols are used. IP Multicast is primarily used in private or enterprise networks, i.e. within one autonomous domain where there is coordination between Multicast sender and receiver, since IP Routing must be configured. However new technologies for support of multicast routing in the public internet are emerging, for instance automatic multicast tunneling can be securely established using revers DNS using RFC 8777.

# * * * * Fourth change * * * *

A.3.3.6	UE

The UE needs to support the MBS operations and receive MBS data packets from NG-RAN. The UE performs the following functions:

-	Reception of multicast data using PTM /PTP.

-	Reception of broadcast data.

-	Handling of incoming MBS QoS flows.

-	Support UP-based joining and leaving.

Editor's Note:	Management of MBS QoS Flows depends on solution evaluation.

-	Signalling for joining and leaving multicast session.

-	MBS support at AS layer.

# * * * * Fifth change * * * *

A.3.3.13	UPF

The UPF performs the following MBS related functions:

-	Interacts with SMF for receiving multicast data from MB-UPF for individual delivery.

-	Send multicast data to NG-RAN for Individual delivery

-	Support UP-based joining and leaving.

NOTE: UPF and MB-UPF may be co-located or deployed separately.

# * * * * End of changes * * * *