SA WG2 Meeting #142E e-meeting	S2-2008625

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	China Mobile

Title:	KI #11 Conclusion updates on Data collection coordination aspect

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to update the conclusion for Key Issue 11 regarding Data collection coordination aspect.

1. Discussion

Based on the KI#11 Conclusion on Data collection coordination aspect, this proposal aims to increase some principles for DCCF to make it can apply more use cases.

2. Text Proposal

It is proposed the following changes to TR 23.700-91.

# * * * * First change * * * *

8.11	Key Issue #11: Increasing efficiency of data collection

For KI#11 for Data collection aspects, it is proposed list of principles are agreed for normative work as follows:

-	A Data Collection Coordination Function (DCCF) is used to coordinate collection of data from one or more NF(s) based on data collection requests from one or more Consumer NF(s)

-	Data Collection Coordination Function (DCCF) and Data Repository Function (DRF) are to be standardized

NOTE 1:	DCCF and DRF can be standalone NFs, possibly co-located with NWDAF, or can be hosted by NWDAF, or can be functionalities are part of NWDAF.

-	Data Collection notification to one or more Consumer NF(s) may be supported via a Messaging Framework. Adaptors supporting 3GPP services allow NFs to interact with the Messaging Framework.

-	Data Collection notifications may also be supported directly from data provider to one or more Consumer NF(s) or via DCCF and not via the Messging Framewrok.

NOTE 2:	The Messaging Framework is outside the scope of 3GPP. Adaptors are not expected to be standardized by 3GPP, only the interface between 3GPP entities and the adaptors is under 3GPP scope. This includes 3GPP services offered by adaptors to allow NFs to interact with the Messaging Framework.

-	The NWDAF analytics function interacts directly with the DCCF to request the collection of data. The NFs interact directly with the DCCF to request analytics to an NWDAF.

NOTE 3:	Based on the outcome of KI#1, decomposed NWDAF functions can also use the above new functions.

-	The DCCF interacts with the NFs to collect data.

-	Data is collected in a standardized manner from one or more NF(s) (including NWDAF) and OAM system.

-	The collected data and/or produced data can be stored in the DRF, which exposes the standardized interface for storage. In this case, DRF act as data source for the stored data.

-	Consumers NF(s) (e.g. NWDAF) access the data from DRF either directly or via a request to DCCF. The data that the NWDAF obtains directly from the DRF will be determined in the normative phase.

-	To act as data source for the stored data, NWDAF or DRF can register/update the data profile of collected data to the DCCF, then DCCF can interact with the NWDAF and DRF to collect the stored data based on consumer NF(s) request and the data profile.

-	If a consumer NF makes a request for data via DCCF and that data are already available at the DRF or some NWDAF, the DCCF forwards the request to DRF or NWDAF instead of forwarding the request to NF.

-	It is possible to use the NRF for discovery of new DCCF and DRF.

NOTE 4:	Additional authorization for Consumers to access data from a Data Source via the DCCF and to access data from DRF (directly or via DCCF) needs to be coordinated with SA3.

As shown in Solutions 9 and 15, NWDAF can increase efficiency of data collection as requested by Key Issue 11 using DCCF to coordinate siganlling and data collection and DRF to reuse the historical data. Hence, it is proposed to use both Solution 9 and Solution 15 as the basis for normative work. Solution 15 is a simple version of solution 9. The normative work of solution 9 should be taken as a full application solution, that is, introducing DCCF, DRF and messaging framework. The normative work of solution 15 should be regarded as a simple application, that is, introducing DCCF and DRF.

# * * * * End of changes * * * *