3GPP TSG-WG SA2 Meeting #142E e-meeting	S2-2008679

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Samsung

Title:	KI#4, update on conclusion

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes to update conclusion on KI #4.

Discussion

Broadcast communication services have no different aspects from multicast services especially about QoS level control, except that Join/leave procedures and individual delivery mode etc. are not considered for broadcast services.

Proposal

It is proposed to add the following changes into TR 23.757.

******************************** START OF CHANGES*********************************

8.4	Key Issue #4: QoS level support for Multicast and Broadcast communication services

The following principles are applied for normative work for multicast and broadcast communication services:

-	The network shall support QoS control per MBS session instead of per user.

-	The network shall support one or multiple QoS flow for an MBS session.

-	The network may use dedicated QoS flows for multicast sessions in a PDU session if 5GC individually delivery is use to deliver the 5MBS data packet.

-	The 5G QoS model and parameters as defined in TS 23.501 [2] clause 5.7 also apply to MBS service with the following differences:

-	Reflective QoS is not applicable;

-	Wireline access network specific 5G QoS parameters do not apply to MBS services;

-	Alternative QoS Profile is not applicable;

-	QoS Notification Control is not applicable;

-	UE AMBR is not applicable.

-	There is support for both GBR and non-GBR MBS flows.

-	AF provides the MBS session information description including QoS requirements to the 5GC.

-	The MB-SMF obtains QoS information and configures the MB-UPF accordingly.

-	The method for the MB-SMF to obtain the QoS information will be determined based on conclusions for Key Issue #1 and the overall architecture.

NOTE:	Depending on KI#1 conclusions, MB-SMF may obtain QoS information in different ways, e.g.:

-	MB-SMF may get QoS info from PCF: MB-SMF uses policy rules provided by the PCF for the MBS session QoS determination, which includes QoS parameters for multicast/broadcast mode and sends these to MB-SMF.

-	MB-SMF may also get QoS info from AF (via NEF/MBSF).

-	Interactions between MB-SMF and SMF and signalling of QoS profile to NG-RAN have dependencies with other Key Issues e.g. key issue #1 and the overall architecture and will be determined based on conclusions for Key Issue #1 and the overall architecture.

******************************** END OF CHANGES **************************************