SA WG2 Meeting #S2-142E	S2-2009065

16 - 20 November, 2020, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.203 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft 23.203 Multimedia Priority Service (MPS) Phase 2 support for Data Transport Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, CISA ECD, AT&T, T-Mobile USA, Nokia, Nokia Shanghai-Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS2 |  |  |  |  |  | Date: |  |  | 09 Nov 2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | MPS for DTS provides the Service User with priority for applications using the default bearer in the EPS, to one or more selected active Packet Data Networks (PDNs), in periods of severe network congestion during which normal commercial data service is degraded. This CR supports stage 1 requirements, TS 22.153, clause 9.3.1: The system shall support: - MPS for DTS for an authorized Service User using a UE with a subscription for MPS, and - MPS for DTS for an authorized Service User using a UE that does not have an MPS subscription. TS 23.203 currently does not support MPS for DTS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes to five clauses. - Added DTS abbreviation; - Added clarifications in various MPS clauses with respect to the existing Priority EPS Bearer Service and IMS Priority service; and - Added new clause on Bearer Priority for MPS for Data Transport Service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for MPS for DTS in Release 17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.1.11.2, 6.1.11.3, 6.1.11.4, 6.1.11.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.401 CR – 23.401 MPS for DTS on PEBS |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [8] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [8].

ADC	Application Detection and Control

AF	Application Function

AMBR	Aggregated Maximum Bitrate

ARP	Allocation and Retention Priority

ASP	Application Service Provider

BBERF	Bearer Binding and Event Reporting Function

BBF	Bearer Binding Function

BBF AAA	Broadband Forum AAA

BNG	Broadband Network Gateway

BPCF	Broadband Policy Control Function

BRAS	Broadband Remote Access Server

CSG	Closed Subscriber Group

CSG ID	Closed Subscriber Group Identity

DRA	Diameter Routing Agent

DTS	Data Transport Service

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

H-PCEF	A PCEF in the HPLMN

H-PCRF	A PCRF in the HPLMN

HRPD	High Rate Packet Data

HSGW	HRPD Serving Gateway

IP-CAN	IP Connectivity Access Network

MPS	Multimedia Priority Service

NB-IoT	Narrowband IoT

NBIFOM	Network-based IP flow mobility

NSWO	Non-Seamless WLAN Offload

OAM	Operation Administration and Maintenance

OFCS	Offline Charging System

OCS	Online Charging System

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCRF	Policy and Charging Rules Function

PFDF	Packet Flow Description Function

PRA	Presence Reporting Area

QCI	QoS Class Identifier

RAN	Radio Access Network

RCAF	RAN Congestion Awareness Function

RLOS	Restricted Local Operator Services

RUCI	RAN User Plane Congestion Information

RG	Residential Gateway

SCEF	Service Capability Exposure Function

vSRVCC	video Single Radio Voice Call Continuity

SPR	Subscription Profile Repository

TDF	Traffic Detection Function

TSSF	Traffic Steering Support Function

UDC	User Data Convergence

UDR	User Data Repository

V-PCEF	A PCEF in the VPLMN

V-PCRF	A PCRF in the VPLMN

WB-E-UTRAN	Wide Band E-UTRAN

SECOND CHANGE

#### 6.1.11.2	PCC rule authorization and QoS rule generation

For MPS service, the PCRF shall generate the corresponding PCC/QoS rule(s) with the ARP/QCI parameters as appropriate for the prioritized service.

For non-MPS service, the PCRF shall generate the corresponding PCC/QoS rule(s) as per normal procedures, without consideration whether the MPS Priority EPS Bearer Service including MPS for DTS is active or not, but upgrade the ARP/QCI values suitable for MPS when the Priority EPS Bearer Service including MPS for DTS is invoked. When the priority EPS Bearer Service including MPS for DTS is revoked, according to PCRF decision, the PCRF shall change the ARP/QCI parameters to appropriate values modified for Priority EPS bearer service including MPS for DTS.

Whenever one or more AF sessions of an MPS service are active within the same PDN connection, the PCRF shall ensure that the ARP priority level of the default bearer is at least as high as the highest ARP priority level used by any authorized PCC rules belonging to an MPS service. If the ARP pre-emption capability is enabled for any of the authorized PCC rules belonging to an MPS service, the PCRF shall also enable the ARP pre-emption capability for the default bearer.

NOTE 1:	This ensures that services using dedicated bearers are not terminated because of a default bearer with a lower ARP priority level or disabled ARP pre-emption capability being dropped during mobility events.

NOTE 2:	This PCRF capability does not cover interactions with services other than MPS services.

THIRD CHANGE

#### 6.1.11.3	Priority EPS Bearer Service

Priority EPS Bearer Service provides priority to all flows mapped/bound to the default bearer either through subscription provisioning or under control of a particular AF via Rx.

The MPS Priority EPS Bearer Service targets the ARP and/or QCI of bearer(s), enabling the prioritization of all traffic on the same bearer.

The PCRF shall, at the activation of the Priority EPS Bearer Service:

-	modify the ARP of the default bearer as appropriate for the Priority EPS Bearer Service under consideration of the requirement described in clause 6.1.11.2;

-	if modification of the QCI of the default bearer is required, modify the QCI of the default bearer as appropriate for the Priority EPS Bearer Service;

-	modify the ARP of PCC/QoS Rules installed before the activation of the Priority EPS Bearer Service to the ARP as appropriate for the Priority EPS Bearer Service under consideration of the requirement described in clause 6.1.11.2; and

-	if modification of the QCI of the PCC/QoS Rules is required, modify the QCI of the PCC/QoS Rules installed before the activation of the Priority EPS Bearer Service to the QCI as appropriate for the Priority EPS Bearer Service.

The PCRF shall, at the deactivation of the Priority EPS Bearer Service:

-	modify the ARP of the default bearer to an appropriate value according to PCRF decision under consideration of the requirement described in clause 6.1.11.2;

-	if modification of the QCI of the default bearer is required, modify the QCI of the default bearer to an appropriate value according to PCRF decision;

-	for PCC/QoS rules modified due to the activation of Priority EPS bearer service:

-	modify the ARP to an appropriate value according to PCRF decision under consideration of the requirement described in clause 6.1.11.2; and

-	if modification of the QCI of PCC/QoS Rules is required, modify the QCI to an appropriate value according to PCRF decision.

FOURTH CHANGE

#### 6.1.11.4	Bearer priority for IMS Multimedia Priority Services

IMS Multimedia Priority Service provides priority to a set of IMS IP flows under control of the P-CSCF (AF).

In addition to the mechanism specified in clause 6.1.11.2, IMS Multimedia Priority Services may require upgrade of the dedicated IM CN signalling bearer and the default bearer, e.g. in order to mitigate the IP-CAN session termination due to resource limitation at a location change the default bearer and dedicated IM CN signalling bearer may need an upgraded ARP.

At reception of the indication that the IMS Signalling Priority is set for the IP-CAN Session or at reception of service authorization from the P-CSCF (AF) including an MPS session indication and the service priority level the PCRF shall under consideration of the requirement described in clause 6.1.11.2:

-	modify the ARP of the default bearer as appropriate for IMS Multimedia Priority Service; and

-	if upgrade of the dedicated IM CN signalling bearer is required, modify the ARP in all the PCC/QoS rules that describe the IM CN signalling traffic to the value appropriate for IMS Multimedia Priority Services.

When the PCRF detects that the P-CSCF (AF) released all the MPS session and the IMS Signalling Priority is not set for the IP-CAN session the PCRF shall under consideration of the requirement described in clause 6.1.11.2:

-	modify the ARP of the default bearer to an appropriate value according to PCRF decision; and

-	modify the ARP in all PCC/QoS Rules that describe the IM CN signalling traffic to an appropriate value according to PCRF decision.

FIFTH AND LAST CHANGE

#### 6.1.11.X	Bearer priority for MPS for Data Transport Service

MPS for DTS is the means for an AF to invoke/revoke Priority EPS Bearer Service for the default bearer of a specific APN supporting/enabling MPS for DTS traffic.

Editor´s Note: The relationship between "priority EPS bearer service" and "MPS for DTS" requires further clarifications.

MPS for DTS provides priority to all flows that are carried on the default bearer for a specific APN. The QoS of the default bearer is modified to receive MPS for DTS treatment. MPS for DTS does not impact bearers other than the default bearer.

For MPS for DTS, the PCRF shall keep the AF informed about the active/deactive state of the service.

The AF uniquely identifies to the PCRF the MPS for DTS invocation and MPS for DTS revocation requests over Rx. The PCRF shall inform the AF that it successfully acted upon the MPS for DTS invocation/revocation request. If the service is deactivated by the IP-CAN, the AF shall be notified.

The PCRF shall, at the activation of MPS for DTS for a specific APN, request the PCEF to:

-	modify ARP and/or QCI of the default bearer to an appropriate value subject to regional/national regulatory requirements and operator policy and according to PCRF decision under consideration of the requirement described in clause 6.1.11.2; and

-	ensure that SDFs, presently mapped and/or bound to the default bearer, receive MPS for DTS treatment.

The PCRF shall, at the deactivation of MPS for DTS for a specific APN, request the PCEF to:

-	modify ARP and/or QCI of the default bearer to an appropriate value according to PCRF decision under consideration of the requirement described in clause 6.1.11.2; and

-	ensure that SDFs, presently mapped and/or bound to the default bearer, do not receive MPS for DTS treatment.

END OF CHANGES