SA WG2 Meeting #142E (e-meeting)	S2-2009078

# Nov 16 – Nov 20, 2020, Electronic, Elbonia

Source: 	Nokia, Nokia Shanghai Bell

Title: 	KI#1, Update on conclusions for remote update of lists

Document for: 	Agreement (P-CR)

Agenda Item: 	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This contribution proposes to update the conclusions for KI#1 related to remote update of lists used for onboarding.

# References

[1]	3GPP TS 24.501, v16.6.0, "Non-Access-Stratum (NAS) protocol for 5G System (5GS)"

[2]	3GPP TS 23.122, v16.7.0, "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode"

[3]	3GPP TS 33.501, v16.2.0, "Security architecture and procedures for 5G system"

[4]	3GPP TS 31.115, v16.0.0, "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications"

# 1 Discussion

As per endorsed conclusion for KI#1, the list of allowed SNPN IDs and Group Ids shall be associated with a specific SNPN credential, or in other words the lists are part of a SNPN subscriber profile and should be persistent within this SNPN subscriber profile.

Therefore, any update message related to these lists (or in the future potentially related to other elements of a subscriber) shall be sent to a termination point in the UE, which is responsible for the hosting of a SNPN Subscriber Profile. Details of how a UE hosts a NPN Subscriber Profile and how such a termination point looks like are beyond the scope of this study and may depend on the way, how the ME in general accesses and uses a SNPN Subscriber Profile.

Following principles are proposed:

- The 5GS shall provide a mechanism based on the usage of the control plane procedure, which enables the update of the lists [refer to S2-200xxxx] used for SNPN selection.

- The control plane mechanism is based on a NAS DL Transport transparent container.

- The transparent container is sent to the UE using UPU as defined in 3GPP TS 23.502, 3GPP TS 33.501.

# 2 Proposal

It is proposed to update TR 23.700-07 as follows

*** Start First Change ***

### 8.2	Conclusions on SNPN selection

### 8.2.1	Conclusions on update of lists on UE

The 5GS shall provide a mechanism based on the usage of the control plane procedure, which enables the update of the lists [refer to S2-200xxxx] used for SNPN selection.

The control plane mechanism is based on a NAS DL Transport transparent container.

The transparent container is sent to the UE using UPU as defined in 3GPP TS 23.502, 3GPP TS 33.501.

Editor's Note: Final decision on how to protect the transparent container is up to SA3.

Editor’s note: Whether there is a specific termination point in the UE (when there is no UICC) for the new container with the list is FFS.

*** End Second Change ***