SA WG2 Meeting #142E e-meeting	S2-2009359

Elbonia, November 16 – 20, 2020	(revision of S2-2008648r01)

Source:	Huawei

Title:	KI#2, Evaluation: Update for specific aspects of NWDAF interactions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to update the evaluation for KI#2 on specific aspects of NWDAF interaction.

1		Discussion

There has been an approved evalution for KI#2 on specific aspects of NWDAF interaction, in this part, Solution #13 and #19 have been evaluated.

Besides these two solutions, Solution #20 also belongs to this aspect, which is to address UE abnormal behaviour analytics interaction in the multiple NWDAF instances.

- Solution #20: UE abnormal behaviour analytics interaction between multiple NWDAF instances

This paper is to update the evaluation for KI#2 on specific aspects of NWDAF interaction, adding an evalution for Solution #20.

2	Proposal

It is proposed the following changes to TR 23.700-91.

# * * * * First change * * * *

7	Overall Evaluation

### 7.2.4	Specific aspects of NWDAF interactions

Solutions agreed focusing on parameter coordination across multiple NWDAFs.

-	Sol #13: Time Coordination for Multiple NWDAFs

-	Sol #19: Solution #19: Multiple NWDAFs interactions for analytics consumption and composition related to large areas

Below observations can be made about above solutions:

-	Sol#13: a tight relation may exist on an action taken by a NWDAF service consumer and the timing of analytics collected from multiple NWDAFs (e.g. in order to adopt a correct AM/ SM policy)

-	Without time coordination between multiple NWDAFs, an un-intended outcome policy may be adopted for a given a NWDAF service consumer due to consuming data analytics from each NWDAF independently.

-	Sol#13 is compatible with both distributed and hierarchical NWDAF architecture.

-	The main impact is on error response from each NWDAF to include revised waiting time.

Sol #19:

-	NWDAFs are configured with the baseline parameters allowed per type of analytics type (i.e. analytics IDs);

-	Baseline parameters are divided into two classes: dataset statistical properties that defines requirements for the selection and preparation of the collected data; and output strategy.

-	NWDAF may use the dataset statistical properties information in order to influence the data selection mechanisms to be used for the generation of an analytics IDs, assuring that the generated analytics ID reflects the statistical characteristics of the data that are relevant for the consumer in all different NWDAFs;

-	Consumers indicate the output strategy to coordinate the multiple NWDAFs under the following criteria: if preferred level of accuracy is more important than providing an output, the binary strategy shall be used by the consumer NF in the request to guarantee that all analytics outputs have equivalent confidence in the prediction; if having an analytics output is more important than reaching the preferred level of accuracy, then the consumer should use the gradient output strategy to guarantee that each NWDAF will timely provide the output indicating the level of accuracy at the moment of the output generation.

-	Sol#19 is compatible with both distributed and hierarchical NWDAF architecture.

Solutions agreed focusing on UE abnormal behaviour analytics interaction across multiple NWDAFs.

-    Sol #20: UE abnormal behaviour analytics interaction between multiple NWDAF instances

Sol #20:

- NWDAF in one network may provide the specific type of analytics e.g. UE abnormal behaviour analytics to other NWDAF(s), which may not support this certain type of analytics.

- This UE abnormal behaviour analytics interaction can be used by other NWDAF(s) to generate new statistics/predictions for certain Analytics ID during the time of the abnormal situation and provide the updated analytics to the analytics consumer.

- Sol#20 is compatible with both distributed and hierarchical NWDAF architecture.

# * * * * End of changes * * * *