SA WG2 Meeting #142E e-meeting	S2-2009360

Elbonia, November 16 – 20, 2020	(revision of S2-2008649r02)

Source:	Huawei

Title:	KI #2, Conclusion: Update for specific aspects of NWDAF interactions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes to update the conclusion for KI#2 on specific aspects of NWDAF interaction.

1		Discussion

There has been an approved conclusion for KI#2 on specific aspects of NWDAF interaction, in this part, Solution #13 and #19 have been concluded.

Besides these two solutions, Solution #20 also belongs to this aspect, which is to address the issue related to UE abnormal behaviour analytics interaction in the multiple NWDAF instances.

- Solution #20: UE abnormal behaviour analytics interaction between multiple NWDAF instances

Since this solution is useful for some NWDAFs to improve the accuracy of generated analytics for certain Analytics ID, this paper recommends to adopt the method in Sol #20 for normative work.

2	Proposal

It is proposed the following changes to TR 23.700-91.

# * * * * First change * * * *

8.2	Key Issue #2: Multiple NWDAF instances

8.2.3 Specific aspects of NWDAF interactions

As a conclusion below recommendations are proposed to be adopted for normative phase:

- The error response to "time when analytics are needed" parameter (clause 6.1.3, TS 23. 288 [5]) is revised such that when each NWDAF sends an error response to the NWDAF Service Consumer, it may include a revised waiting time.

- The dataset statistical properties and output strategy parameters shall be used by consumers of NWDAF services when multiple NWDAFs are involved in the generation of single analytics output in order to prevent un-intended outcomes due to consuming data analytics from each NWDAF independently.

- An NWDAF can consume the UE abnormal behaviour analytics provided by another NWDAF to trigger updating the applied ML model, and then based on the updated model provide updated analytics to the analytics consumer if necessary.

# * * * * End of changes * * * *