3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009474

Elbonia, November 16 – 20, 2020, 2020	(revision of S2-2008917r04)

Source:	Nokia, Nokia Shanghai-Bell, Ericsson, Huawei, HiSilicon

Title:	KI#1: Evaluation updates

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution provides updates to the evaluation of key issue 1 to reflect already drawn conclusions and agreements from the combined architecture and to address an editor´s note.

# Text Proposal

It is proposed to agree the following changes for TR 23.757.

In r03: considering merging info from 8931, and reflected the issues raised around SMF-centric approach:

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 22.101: "Service aspects; Service principles".

[4]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[5]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[6]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[7]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[8]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[9]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[10]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[11]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network (5GC)".

[12]	RP-193248: "New Work Item on NR support of Multicast and Broadcast Services", RAN#86.

[13]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[14]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[15]	3GPP TS 24.379: "Mission Critical Push To Talk (MCPTT) call control; Protocol specification".

[16]	3GPP TS 23.003: "Numbering, addressing and identification".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[18]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[19]	IETF RFC 4867: "RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs".

[20]	IETF RFC 5795: "The RObust Header Compression (ROHC) Framework".

[21]	IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed".

[22]	IETF RFC 6363: "Forward Error Correction (FEC) Framework".

[23]	3GPP TS 29.116: "Representational state transfer over xMB reference point between content provider and BM-SC".

[x]	IETF RFC 8777: "DNS Reverse IP Automatic Multicast Tunneling (AMT) Discovery".

[x+1]	IETF RFC 7450: "Automatic Multicast Tunneling".

[x+2]	IETF RFC 3552: "Guidelines for Writing RFC Text on Security Considerations."

## 7.5	Key Issue #1: MBS session management

### 7.5.1	Overview over available solutions

For Key Issue #1 "MBS session management", there are 16 solutions in the TR:

-	#2,#3,#4,#5,#6, #8, #9, #10, #11, #12, #13, #14, #15, #16, #32, #33.

1)	General MBS Session Management (MBS Session configuration, join/leave, release, etc.)

A comparison of solutions covering general MBS Session Management procedures is shown in Table 7.5.1-1.

Table 7.5.1-1: Comparison of multicast solutions for KI#1 MBS Session Management (Part 1)

|  | Procedures |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol | MBS Session (/Service) configuration | MBS Session Join | MBS Session Start | MBS Session Modification/Update | MBS Session Leave | MBS Session Stop | MBS Session Delete (/Release) |
| 2 | Y. Dedicated MBS session signalling procedures | Y. Dedicated MBS session signalling | Y. Dedicated MBS session signalling |  | Y. Dedicated MBS session signalling | Y. Dedicated MBS session signalling. | Y. Dedicated MBS session signalling. |
| 3 | Y. Dedicated MBS session procedures. | Y. Enhanced PDU session mod./est. procedure | Y. See MBS Session Join. |  | Y. Enhanced PDU session modification procedure |  |  |
| 4 | Y. Dedicated MBS session procedures. | Y. (MB/PDU) session procedure | Y. Dedicated MBS session signalling. | Y. (MB/PDU) session procedure | Y. (MB/PDU) session procedure | Y. Dedicated MBS session signalling. | Only release per RAN node |
| 6 | Y. Dedicated MBS session procedures. | Y. Enhanced PDU session est./mod. procedure | Y. See MBS Session Join. |  |  |  |  |
| 10 |  | Y. Enhanced PDU session mod./est. procedure |  |  | Y. Enhanced PDU session mod. procedure |  | Y. Enhanced PDU session mod. procedure |
| 14 |  | Y. Dedicated MBS session signalling | Y. Dedicated MBS session signalling |  | Y. Dedicated MBS session signalling | Y. Dedicated MBS session signalling |  |
| 16 |  | Y. Enhanced PDU session est./mod. procedure |  |  |  |  |  |

NOTE:	Dedicated MBS session signaling may also use existing message enhanced with 5MBS functionality, e.g. in Sol#2, UL NAS TRANSPORT is enhanced to support UE join.

2)	Solution addressing specific aspects

MBS Session activation and deactivation

Sol#13 and #32 propose procedures for MBS Session activation and deactivation, which are both based on Sol#3 and share much similarity. Both solutions suggest that MBS service activation is triggered via detection of UP activity by MB-UPF. Sol#32 adds control plane (i.e. multicast group configuration update by AF) triggered MBS Session activation and deactivation.

Interactions between MBSF and MBSU

Sol#33 proposes enhancements on N4/PFCP procedures to support interactions between MBSF(MSF-C) and MBSU (MSF-U). This is related to the ongoing discussion in S2-2007287 and LS S2-2006981 expected to be sent to SA WG4/SA WG6.

Editor's note:	Feedback from SA WG4 is required on proposed stage 2 procedures for media processing of MBS service data and related work split.

multicast session parameters based on analytics

Solution 8: AF derives multicast session parameters based on analytics and provides them via NEF/UDR to PCF. PCF uses them to derive policies.

3)	Broadcast specific MBS Session Management

Sol#5, and #9 propose Broadcast Session start/activation procedures, which are similar though MSF-C and MBSF are used respectively based on different baseline architectures.

Editor's note:	Usage of names "MBS-C", "MSF" and "MBSF" needs to be aligned.

4)	UE mobility / Inter-RAN Handover related MBS Session Management

Related solutions are either assigned to key issue 1 or key issue 7

A comparison of solutions for UE mobility/Inter-RAN HO related MBS Session Management procedures is shown in Table 7.5.1-2.

Most of these solutions have aspects dependent on the feedback from RAN WGs and also the conclusion of solution(s) selected for general MBS Session Management.

Table 7.5.1-2: Comparison of solutions for UE mobility/Inter-RAN handover related MBS Session Management

|  | Sol#11/#12 | Sol#15/#29 | Sol#26 | Sol#27 | Sol#39 | Sol#40 (dual RANs case) |
| --- | --- | --- | --- | --- | --- | --- |
| Procedures for delivery method switching | Enhanced Xn HO: MB Session resource setup during Xn Handover preparation or execution phase Enhanced N2 HO: MB Session resource setup during N2 Handover preparation phase | Enhanced Xn/N2 HO with UE initiated NAS multicast session join. | Enhanced Xn/N2 HO: Forwarding multicast data via mapped QoS flow over PDU session from source to target RAN | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, depending on whether target RAN supports MBS. - Forwarding MBS data to Target RAN supporting MBS. | Multicast->Uni-cast:UE initiated PDU Session establishment or activation; Unicast->Multi-cast: UE initiated PDU session release or deactivation. | Enhanced Xn/N2 HO: - Multicast or unicast transport established in target RAN, based on UE request of unicast or multicast mode in application layer. - Forwarding multicast data over PDU session |
| Based on which solution | 2 | 4 | - | 3 |  |  |

NOTE:	Solutions for UE mobility/Inter-RAN HO related delivery method switching, i.e. solutions #26, #27, #29, #39 and #40, are also listed here for a common evaluation.

These solutions will be further evaluated as part of the KI#7 evaluation.

Following list illustrates alternatives per technical aspect for evaluation.

Consideration on MB session network operations (e.g. start, stop, delete, suspend, resume, configuration, etc.):

Alternatives:

Alt 1: AMF triggers RAN for session start notification (Sol#2, 4, 5, 13, 32);

Editor's note:	Session start notification needs RAN coordination.

Alt 2: UE initiates session join after session start time if known, no session start notification (Sol#4)

Initial try of session join may be rejected by network with MB session information provisioned

Consideration on MB session user operations (e.g. join, leave, etc.):

Alternatives:

Alt 1: Operation via CP using NAS message

> Alt 1.1: NAS message does not include associated PDU session information (Sol#2, 4, 14);

> Alt 1.2: NAS message includes associated PDU session information (Sol#3, 4, 6, 10, 16, 32);

> Alt 1.3: NAS message includes application layer container (Sol#4);

Allow application specific MB session join/leave operation

Alt 2: Operation via UP (Sol#2, 3, 4, 32)

Alt 3: Operation via CP using RRC message for UE in RRC-INACTIVE state (Sol#4)

Alt 4: Support aggregation of multiple session join/leave requests on UE (Sol#4)

Uses multiple MB session IDs in NAS message

Alt 5: Support session leave silently (Sol#4)

Alt 6: Support AF initiated MB session user operations (Sol#3, 8)

AF provides information of UE and MB session to 5GS for transmission resource setup to a user

Consideration on MB-SMF selection for MB session user operations:

Alternatives:

Alt 1: AMF queries NRF (Sol#2, 16);

Alt 2: AMF queries UDM/UDR if session join via CP without PDU session information (Sol#4);

Alt 3: AMF selects MB-SMF based on S-NSSAI and DNN (Sol#6, 10, 14);

Alt 4: UPF notifies SMF of session join via UP, SMF informs AMF to select MB-SMF (Sol#2, 16);

Alt 5: SMF selects MB-SMF (Sol#3, 4, 32)

UPF notifies SMF of session join via UP, SMF selects MB-SMF, or SMF receives signalling of session join with PDU session information and selects MB-SMF

Consideration on selection of 5GS Shared or Individual MBS traffic delivery method:

Alternatives:

Alt 1: AMF determines the delivery method (Sol#2);

Alt 2: SMF determines the delivery method (Sol#3)

Alt 3: MB-SMF determines the delivery method (Sol#4)

Alt 4: Criterial for delivery method determination

> Alt 4.1: RAN node 5MBS capability (Sol#2, 4)

> Alt 4.2: UE count under a RAN node (Sol#4)

> Alt 4.3: UE count of a MB session (Sol#4)

> Alt 4.4: AF indication of individual delivery disablement (Sol#2)

Consideration on session context management in 5GC:

Alternatives:

Alt 1: AMF stores the session context (Sol#2);

Alt 2: MB-SMF stores the session context (Sol#2, 4, 6, 9, 10);

Alt 3: SMF stores the session context (Sol#3);

Alt 4: MBSF stores the session context (Sol#2);

Consideration on MB session ID (e.g. TMGI, HL MC address etc.) for MB session identification:

Alternatives:

Alt 1: 5GC internally assigned ID (Sol#2, 4)

> Alt 1.1: With 1-1 mapping to an IP multicast address or a source specific multicast address (Sol#2);

> Alt 1.2: With m-1 mapping to an IP multicast address or a source specific multicast address (Sol#4)

AF uses same IP multicast address for multiple MB sessions (e.g. distinguished by source port) to avoid frequent maintenance of IP multicast routing over internet

> Alt 1.3: With 1-1 mapping to N6 tunnel for non-IP multicast service (Sol#4);

Alt 2: AF assigned ID (Sol#3)

5GS does not assign internal ID, while uses IP multicast address or source specific multicast address as MB session ID

Consideration on N3 tunnel operation for 5GC Shared MBS traffic delivery method:

Alternatives:

Alt 1: AMF forwards signalling for shared N3 tunnel setup/release from RAN to MB-SMF (Sol#2, 3, 4);

> Alt 1.1: RAN indicates MB-SMF ID to AMF for signalling forwarding (Sol#3, 4)

> Alt 1.2: RAN indicates MB session ID to AMF for signalling forwarding (Sol#2)

AMF manages MB session context and determines the MB-SMF accordingly (Sol#2)

Alt 2: Keep shared N3 tunnel when session stopped and release during session delete (Sol#2)

Alt 3: Release shared N3 tunnel during session stop (Sol#4)

Consideration on N9 tunnel operation if needed:

Alternatives:

Alt 1: SMF gets N9 DL tunnel information (similar as C-TEID) from MB-SMF (Sol#4)

SMF provides the N9 DL tunnel information to UPF, and UPF associates the PDU sessions with the N9 DL tunnel information

Alt 2: SMF provides N9 DL tunnel information to MB-SMF for multicast flow redirection (Sol#3, 4)

SMF provides the N9 DL tunnel information to UPF, and UPF associates the PDU sessions with the N9 DL tunnel information

Consideration on PSA-UPF joining into multicast tree for 5GC Individual MBS traffic delivery method if needed:

Alternatives:

Alt 1: SMF gets LL MC address from MB-SMF (Sol#2, 4)

Alt 2: SMF gets HL MC address from MB-SMF (Sol#2, 4)

Consideration on service parameters provisioning:

Alternatives:

Alt 1: UDR is the storage place (Sol#3, 4, 8);

Alt 2: MBSF is the storage place (Sol#6)

Alt 3: MB-SMF is the storage place (Sol#2)

Consideration on UE mobility in CM-CONNECTED and RRC-CONNECTED state:

Alternatives:

Alt 1: Support handover for MB session with forwarding option (Sol#15, 27)

Editor's note:	Handover with forwarding option needs RAN coordination.

> Alt 1.1: Source gNB forwards data of MB session to target gNB if needed

> Alt 1.2: Uses SN number of packet assigned by CN for packet loss/duplication determination

Alt 2: Support handover for MB session without forwarding option (Sol#11, 12)

Consideration on Xn handover:

Alternatives:

Alt 1: Target gNB interacts with 5GC during handover preparation phase if needed (Sol#11, 15)

> Alt 1.1: Target gNB responses to the source gNB after exchange messages with AMF for adding the target gNB into the MB session (Sol#11)

> Alt 1.2: Target gNB simultaneously responses to the source gNB and exchanges messages with AMF for adding the target gNB into the MB session (Sol#15)

Alt 2: Target gNB adds into the distribution tree during handover execution phase if needed (Sol#11, 15)

Consideration on N2 handover:

Alternatives:

Alt 1: Target gNB adds into the distribution tree during handover preparation phase if needed (Sol#12, 15)

> Alt 1.1: MB-SMF setups transmission resource to target gNB via AMF if needed (Sol#12)

> Alt 1.2: Target gNB sends N2 message to AMF after receiving the instruction from 5GC if needed (Sol#15)

Alt 2: Target gNB adds into the distribution tree during handover execution phase if needed (Sol#15)

### 7.5.2.	Discussion

#### 7.5.2.1	Multicast solutions

Trigger for establishment of multicast sessions in the PLMN

Many solutions (e.g. Solution 2, 3, 4, 6) suggest that a multicast session is created in the PLMN when the AF requests a reservation of resources for a multicast session. Some solutions also allow the creation of a multicast session based on configuration. Some solutions (e.g. solution 3, 4) in addition allows for a dynamic creation of multicast sessions in the PLMN without any AF interaction and configuration when it is observed that a UEs joins a multicast session in an external network (e.g. via IGMP join using a source specific multicast address of the external network). All options seem to offer advantages.

Regarding the dynamic trigger for a creation of multicast sessions without any configuration:

-	It is not enough that clients activate the reception of a Multicast group, there needs to be a sender. Thus, application level service announcement and/or service negotiation is assumed. The Multicast sender may not be aware whether there are zero or more receivers.

-	Support of IP Multicast on the public internet (external Data Network) is not widespread since IGMP is not used between routers and PIM or other routing protocols are used. IP Multicast is primarily used in private or enterprise networks, i.e. within one autonomous domain where there is coordination between Multicast sender and receiver, since IP Routing must be configured. However, newer technology such as RFC 8777 [x] enables an automatic establishment of tunnels for multicast traffic through the public internet; the source address of the source-specific multicast address is used to discover tunnel endpoints for the network domain where the multicast data source is located.

NOTE:	Different IP Multicast tunneling solutions, e.g. GRE or AMT (see RFC 7450 [x+1] and RFC 8777 [x]), have been specified so that the IP packets of a remote IP Multicast Server (e.g. IPTV Channel provider) can be forwarded via not-IP-Multicast-enabled-networks. The MNO may make an agreement with the remote tunnel end-point on the tunneling solution and potentially on access rights, but this is not required for AMT. Furthemore, agreements with transit IP network are beneficial to ensure a well-defined connectivity performance (IPTV streams require a certain bandwidth due to timing of the video and audio). There is also security concern against RFC 7450 [x+1] that many of the threats and vectors described in RFC 3552 [x+2] may be employed against the protocol to launch various types of denial-of-service attacks that can affect the functioning of gateways or their ability to locate and communicate with a relay; RFC 8777 [x] addresses some of those concerns.

- If AMT is not used, IP multicast between MNO and Content Provider requires SLA and IP network configurations, thus the dynamic creation may not work as expected.

- RFC 8777 [x] uses a trusted DNS server and a secure connection. 5GC can validate using that DNS server whether the SSM address is valid, and thus check whether the UE joins a valid SSM and protect against  DoS attack as well as unexpected radio resource usage.

- Without AF configuration, there is no possibility to provide QoS differentiation to different MB service data flows.

Join/Leave Operation:

All solutions for multicast contain an operation where the UE indicates to the network the desire to receive contents of a multicast session.

Most solutions suggest that control plane signalling can be used for that purpose. In addition, many solutions (e.g. 2, 3 and 4) allow for a user plane based join/leave operation. Solution 16 suggests using the control plane PDU session establishment signalling. For control plane signalling, solution 4 suggests aggregation of multiple session join/leave requests on UE and allow application specific MBS session join/leave operation. Solution 4 also suggests that UE can leave an MBS session silently. Solution 3 and 8 suggest AF can interact with 5GS for a user to join/leave an MBS session.

| How the UE join/leave the MBS session. | Both CP and UP | CP only |
| --- | --- | --- |
| solution | #2, #3, #4, #10, #16 | #6, #8 |

For CP join, there are further differences in what signalling is used:

| CP join signalling | PDU session related signalling | Dedicated signalling |
| --- | --- | --- |
| solution | #3, #4, #10, #16 | #2, #4, #8 |

Using control plane signalling offers the advantage that it works independent of an established PDU session and is easily accessible in the AMF, and that is more flexible.

Different view whether IGMP Join is really needed is expressed (e.g. in S2-2007266):

-	Join/Leave operation via UP signalling can be avoided by means of the 5G-RG snooping the UP Join/Leave from the STB and convert it to CP(NAS) signalling. With this, the system impact for 5MBS can be minimized as far as join/Leave is concerned.

-	For applications which expect to use IGMP/MLD Join to receive content, the 5MBS capable UE (including 5G RG) snoops the IGMP/MLD message and converts IGMP/MLD Join/Leave to NAS Join/Leave.

Using user plane signalling (IP IGMP join) offers the advantage that it is well aligned with IP technologies. It avoids double signalling if a UE wants to join an external multicast session and is also well suited as dynamic trigger for a creation of multicast sessions.

Using PDU session related signalling can achieve an integrated MBS and PDU session management as well as an easier upgrade from unicast support to multicast support, but always need a PDU session for the purpose.

Using dedicated signalling has advantage that does not mandate UE to have a PDU session associated with MBS session if not required at the time of join.

Multicast Session ID and selection

Many solutions suggest reusing the TMGI allocated by the PLMN as multicast session identifier. Other solutions (e.g. solution 3) suggest using a source specific IP multicast address as identifier.

| MBS session ID | TMGI | IP multicast address | unknown |
| --- | --- | --- | --- |
| solution | #2, #4, #10, #11, #12, #14, #15, #16 | #3, #13, #16, #32 | #5, #6, #8, #9, |

The source specific IP multicast address is globally unique and a source specific IP multicast address used as identifier for an external multicast session and selected by an external network can also be used as identifier within the PLMN. However, the identifier can also be allocated by the PLMN as it is globally unique. There is also view whether it is suitable to use an external source specific IP multicast address as an identifier within 5GC.

For UP (IP IGMP) join, only IP multicast address is supported.

As TMGI is used as identifier in existing MBMS applications e.g. for public safety and may simplify migration of such applications to 5G as well as dynamic fallback from 5MBS to 4G MBMS due to coverage restrictions which is further clarified in the KI#9.

Solution 3 allows for usage of TMGIs in addition to multicast addresses by mapping TMGIs to multicast addresses.

Solution 4 suggests an IP multicast address could have multiple TMGI associated, which are distinguished by packet filters.

Handling of multicast session context including knowledge about UE participation in multicast session:

Solution 2 and 16 proposes the AMF for that purpose.

Other solutions (e.g. 3, 4) suggest that the SMF used also for unrelated unicast PDU sessions: is used for that purpose.

| NF that stores the multicast context | AMF | SMF |
| --- | --- | --- |
| solution | #2, #16 | #3, #4, #6, #8, #10, #16 |
| Notes | - UE sends an UL NAS MB Session Join Request to AMF including the MBS group information. - AMF selects the MB-SMF and/or further fetches the MB Session Context from MB-SMF if needed. - AMF provides the MB Session Context to RAN during session start procedure. - 5GC Individual MBS traffic delivery only used when RAN doesn't support MBS and application can apply 5GC individual MBS traffic delivery. | - UE sends PDU Session establishment/ modification request to 5GC. - SMF (i.e., handling unicast PDU session) fetches the MB Session Context from MB-SMF if needed. - SMF handling unicast PDU session deals with the request and provides the unicast PDU session ID together with multicast session info to RAN. |

Discussion of SMF to handle multicast context:

-	Some solutions (e.g., #3, #10 and #16) assume QoS flow level association to guarantee the network level lossless handover. For the related solutions, RAN coordination may be required to verify the proposed solution for not using the above unicast QoS Flows when NG RAN and UE support 5G MBS, and for reserve resource when they are used. Such "unicast QoS counterpart" context will be stored at RAN side, and in the Radio bearer level, some dynamic scheduling is possible and there could be no radio resource wasted for the reservation (note that such part depends on the output of RAN WGs).

-	Since the unicast PDU Session associates with multicast session, reusing the unicast PDU Session resources when UE handovers from the MBS-capable RAN to the RAN without MBS capability, to transmit/forward the MBS data from M-UPF/S-RAN is possible, and the session continuity can be handled in the network level.

Discussion of AMF to handle multicast context:

-	The AMF is enhanced to support the management of Multicast session, including 1) UE authorization for accessing MBS Session, 2) determining the delivery mode of MBS Session, 3) maintaining MBS Session Context, 4) controlling activation/deactivation of the MBS Session, 5) sending notification to UE to trigger unicast PDU Session establishment, 6) selecting (MB-)SMF for the MBS session, and so on. Such design assumes AMF with the SM functionalities and is lack of separation between MM and SM.

-	Using the AMF may have the advantage of allowing a participation in a multicast session for a UE with no PDU session and to limit the number of impacted network entity types for a basic multicast support. It may also allow a UE to join a bit quicker and with less load on the network. However, fallback to individual delivery and UP Join via IGMP still requires a PDU session and also impacts the SMF. Further, if multicast support in a network is not homogenous, the information about UEs participating in a multicast session may be lost when a UE is handed over to an AMF not supporting MBS. Also, checking the UE authorization to participate in a multicast session based on policies may be more complicated compared to SMF controlling UE participation in a multicast session.

-	For the UE that handovers from an MBS-capable RAN to the RAN only support unicast, considerable latency could be experienced: Since the target RAN cannot support MBS, unicast PDU Session is chosen to transmit the MBS data (i.e., 5GC shared MBS traffic delivery mode), if the resources (e.g., unicast PDU Session or QoS Flow) to transfer the MBS data has not been established yet, network has to trigger the related SM procedures (e.g., PDU Session Establishment/ Modification) to enable MBS data reception, but such SM procedures have to be held on because of the ongoing MM procedure (i.e., Handover), which may lead to a large latency.

During the inter-AMF mobility, the source AMF shall send the UE MBS context to target AMF.

Root of multicast distribution tree for shared delivery and ingress point for multicast data

All solutions assume that there is a multicast distribution tree for shared delivery having a single root. Many solutions allocate the root in the MB-UPF controlled by a MB-SMF (e.g. solution 2, solution 3, solution 4). The MB-SMF may be a SMF also handling PDU sessions or a separate entity. Solution 6 suggest using the MBSF and thus requires that the MBSF is used for all multicast sessions.

| Root of multicast distribution tree | MB-UPF | MBSF-U |
| --- | --- | --- |
| solution | #2 #3, #4, | #6 |

Many solutions assume that there is a single ingress point towards an external network for multicast data that can be collocated with the root of the multicast distribution tree. However, to allow for an optional media processing in a dedicated entity it is also proposed by many solutions that use the MB-UPF as root of the multicast distribution tree that the ingress point can optionally be allocated in a MBSF-U

Solution #6 and #10 assumes the SMF handling unicast PDU Session will be the same as MB-SMF controlling MB-UPF However, other such as Solution #3 consider separating the two SMFs and the unicast SMF finds MB-SMF (multicast SMF) when UE joins.

It was agreed that the MBSF-U will only be deployed in certain configurations, so an MBSF-U based solution will not address all scenarios.

Selection of root of multicast distribution tree selection

Most solutions suggest that the root of the multicast distribution tree for shared delivery and ingress point are assigned when the multicast session is established (see above)

There is a need to subsequently discover the root of the multicast distribution tree (e.g. by an AMF when a UE joins). Solution 2 suggest that TMGI ranges are assigned by configuration to specific roots of multicast distribution trees for that purpose. Other solutions suggest storing the multicast distribution tree root for a multicast session in a database (UDR e.g for solutions 3 and 4, and NRF e.g. for solution 16).

In Sol#3 and solutions based on Sol#3, the SMF interacts with UDR to retrieve the MB-SMF info, however currently SMF does not interact with UDR, and example consumers of UDR services are only UDM, PCF and NEF. It can be determined in the normative phase whether the SMF uses the PCF to interact with the UDR.

As it is agreed to support also source specific multicast addresses, a selection based on TMGI ranges is not sufficient to address all cases.

MBMS session start:

Many solutions suggest that an AF can request the start of an MBMS session that would result in resource reservation and a possible paging of UEs within a MBMS session. (e.g. solution 2, solution 32).

But it is also suggested that the MBMS session start can be combined with the MBMS session establishment (e.g. solution 3) or that the MBMs session start can be triggered by the reception of multicast data (e.g. solutions 13 and 32).

The need for a session start to trigger UE paging depends on whether the UE can become CN IDLE while in a multicast session. Coordination with RAN is necessary,

Some solutions suggest that there is a single AF request for reserving resources for the multicast session and starting it (e.g. solution 3), whereas other solutions suggest separate interactions. Solution 3 suggests that RAN establishes the MB Session Context during the session join procedure triggered by the UE.

A first interactions may be required prior to service announcements to request TMGIs to be announced from the network that allocates them; this is not required if a multicast session ID allocated by the AF is used instead. The implication is that the UE join may be rejected if the MBS session is not yet started and the UE needs to reattempt.

Solution 2 suggest a group-based RAN paging mechanism is used and additional clarifications (e.g., how could the AMF figure out the group paging area) are provided in S2-2008518. Moreover, after receiving the group paging, UE sends MB Session join request to AMF. If such MB Session join message is not Service Request, UE needs to be enhanced accordingly (i.e., triggering another procedure rather than Service Request upon the reception of paging message), if the message is Service Request, the NAS message needs to include TMGI information.

Service announcements:

Most solutions assume an application level service announcement from AF to UE. However, Solution 14 suggest extending URSP for that purpose.

Announcement over application level may have impact on multicast application client and OS layer on UE, announcement over 3GPP layer is also an alternative that has less impact on UE.

However, application level service announcements offer large flexibility and enable rich and human-readable description of the contents of a multicast session in a customized user interface offered by the application provider that could be rendered by an API or Webpage on the device. New multicast sessions can be easily added without or with minimal network impact only, and the user can actively select information sources for service announcements.

In contrast, for URSP only limited content could be provided in a service announcement and selective download of service announcements based on user preferences and interactions would hardly be possible.

Existing MBMS applications such as public safety also use application level service announcements.

5GC allowing AF to disable the usage of 5GC individual delivery

Sol#2 allows the AF to disable the usage of 5GC individual delivery when 5MBS capable UE camping on NG-RAN not supporting 5MBS joins a multicast session. The main consideration is, compare 5GC individual delivery with unicast delivery, the former has some drawbacks (e.g. 5GC individual delivery does not provide the possibility of rate adaption), therefore such delivery method is not suitable to be used as the norm for application that expects to use rate adaptation for possible improvement of user experience.

On the other hand it is unclear how seamless an application level fallback to unicast delivery can be and there is a risk of service loss.

#### 7.5.2.2	Comparison between AMF centric vs SMF centric approach

AMF centric approach refers to the solution that AMF is responsible for setting up the shared delivery (e.g. Sol#2), while the SMF centric approach refers to the solution that SMF responsible for setting up the shared delivery (e.g. Sol#3).  Related evaluation is also provided under subheading " Handling of multicast session context including knowledge about UE participation in multicast session" in subclause 7.5.2.1,

Both AMF-centric and SMF-centric approaches can fulfill the basic requirement of shared delivery, and the optional requirement of Individual delivery.

For interworking with EPS, Individual delivery may be possible for SMF-centric approach assuming the individual delivery resource can be properly released when not needed.

The following potential issues are raised during SA2#142E CC#1 with respect to the SMF centric approach:

-	How the SMF gets MB-SMF ID; It was commented that MB-SMF discovery with similar issue also applies for the AMF in the AMF centric approach. Further, allowing new consumers for a service based interface at the UDR would be in line with overall SBA design principles, However, using the PCF to do the UDR lookup can be considered as alternative to the direct SMF UDR interaction in the normative phase.

-	For location dependent MB service: how the additional identifier (e.g. Area Session identifier/Flow ID) will be obtained; It was commented that this relates an insignificant detail of solution 21 for key issue 6 (that is based on solution 3), where the UDR allocated the Area Session identifier, and it could be assessed in the normative phase whether the NEF or MB-SMF could instead allocate the identifier and store it in the UDR.

-	UE joining may get rejected due to MBS Session not configured yet in MB-SMF, there is no way for the UE(s) to know when MB-SMF has been configured, the UE(s) may keep trying causing denial of service attack type situation; It was commented that this discussion is unrelated to the AMF vs SMF centric approach. For instance, solution 32 based on solution 3 also provides a separate session activation for the SMF centric approach.

-	5MBS in Roaming: how the inbound roamer would not be able to join the multicast service; It was commented that roaming requires more study for both the AMF and SMF centric approach, where individual delivery deserves special attention, and that using a LBU PDU session might be a possible solution.

-	Signaling increase unnecessarily due to MBS Session operation performed via PDU Session signaling, examples are:

MBS Session Update will have to be performed “per PDU Session per MBS Session” instead of “per MBS Session”

The proposal of MBS Session deactivation by releasing the MBS QoS Flow or releasing the shared N3 tunnel will have to be performed via PDU Session as well.

It was commented that this relates to setting up associated QoS flows for multicast in the associated PDU session already when the UE joins in preparation of a possible future fallback to individual delivery. This procedure could again be applied in combination with AMF or SMF centric approach and should thus be discussed separately.