3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100126

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3: Conclusion update for the solutions

Document for:	Approval

Agenda Item:	8.4.1

Work Item / Release:	FS_eNS_ph2/ Rel-17

Abstract: This contribution introduces the conclusion of all the solutions of KI#3.

1. Introduction/Discussion

According to the feedback from RAN WGs, RAN could enforce the SMBR in both UL and DL:

For UL, as described in R2-2011104/S2-2100008, “In this solution RAN enforces uplink and downlink SMBR of UEs. This is a similar function as UE-AMBR enforcement at slice level. With proper configuration (LCG and LCH restrictions), the RAN is able to obtain and control the UL data volume of a slice.” And this is further confirmed by R2-2102009.

For DL, as described in R3-207230/ S2-2100041, “For Solution#22, RAN3 acknowledges impact on admission control due to S-MBR apply not only for Non-GBR traffic but also for GBR traffic. It is feasible for the NG-RAN node to perform the DL enforcement of S-MBR.”

Based on the way forward discussion in FS_eNS_ph2, RAN based solution (i.e. Solution #22) is selected as the baseline to resolve KI#3.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-40.

# * * * * First change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.3	Conclusion for Key Issue #3

For Key Issue #3 (Limitation of data rate per network slice in UL and DL per UE):

-	A new QoS parameter, Slice-MBR per S-NSSAI is defined. The UL/DL Slice-MBR limits the aggregate data rate in UL and DL per UE across all GBR and Non-GBR QoS Flows for all PDU sessions associated with a S-NSSAI for the UE. The value of UL/DL Slice-MBR is stored in the UDR.

-  RAN is recommended to perform Slice MBR data rate enforcement as described in Solution #22, i.e. Solution #22 is selected as baseline to resolve KI#3.

# * * * * End of changes * * * *