SA WG2 Meeting #S2-143E	S2-2100251

24 February – 9 March, 2021, Elbonia

# Title:	Reply LS on NAS Non delivery for RRC_INACTIVE state

# Response to:	Reply LS on NAS Non delivery for RRC Inactive state (R3-207170 / S2-2009100)

# Release:	3GPP Rel-15/Rel-16

# Work Item:	NR_newRAT-Core

Source:	SA2

To:	RAN3

Cc:	CT4

Contact Person:	Hannu Hietalahti

E-mail Address:	Hannu (dot) Hietalahti (at) Nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS 23.501 CR 2548

1. Overall Description:

SA2 thanks RAN3 for updating the status of this discussion between SA2 and RAN3 by clarifying question and provides the following answers.

Question 1: For a UE in RRC_INACTIVE state, is there any use case for AMF to piggyback a non-PDU session related NAS PDU in PDU SESSION RESOURE SETUP REQUEST?

SA2 answer: SA2 agrees that the case of Non-PDU session related NAS PDU included in PDU Session Resource Setup does not exist as according to TS 23.501 clause 4.2.3.2-1 step 12 that was already quoted by RAN3.

For the second question related with the Initial Context Setup Request, RAN3 had identified two alternative solutions as follows:

Question 2: Which solution is preferred to inform AMF the non-delivery of the non-PDU session related NAS-PDU in the "Initial Context Setup Request":

- Solution 1: Use NAS NON DELIVERY INDICATION message to indicate the failure of the NAS delivery.

- Solution 2: Use the "Initial Context Setup failure” to implicitly indicate the failure of the NAS delivery.

SA2 answer: Solution 2 applies since SA2 has not specified any other error handling in addition to the Initial Context Setup Failure. SA2 has agreed the attached CR to clarify that AMF error indication does not imply an additional NAS non delivery indication procedure.

2. Actions:

To RAN3 group:

ACTION:   SA2 would like to kindly ask RAN3 to align their specifications with this guidance.

3. Date of Next SA2 Meeting:

SA2 #144E	12 – 16 Apr, 2021	Electronic