3GPP TSG SA WG2 Meeting #143-e		S2-2100630

Electronic, 24 Feb – 09 Mar, 2021

Title:	Reply LS on Clarification request for eNPN features

Response to:	S2-2100143 / R2-2102489

Release:	Rel-17

Work Item:	eNPN; NG_RAN_PRN_enh-Core

Source:	SA2

To:	RAN2, CT1, SA1

Cc:	RAN3

Contact person:	Sašo Stojanovski

saso stojanovski intel com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 thanks RAN2 for their questions related to SNPNs with subscription or credentials by a separate entity (S2-2100143 / R2-2102489).

SA2 would like to provide the following answers:

Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ) , and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area?

A1: The three indications referenced in Q1 need to be uniformly supported across the entire SNPN.

Question 2: Shall Group IDs be broadcasted per SNPN or per cell?

A2: The Group IDs are broadcasted per SNPN in a shared cell.

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?)

A3: It should be possible to have selective support for the “onboardingEnabled” indication e.g. to turn it off in congested RAN cells, while leaving it on in other cells.

Question 4: Is the support of eCall over IMS assumed to be enabled in SNPN cells?

A4: While not deemed essential, SA2 is of the opinion that the specifications should support eCall over IMS in SNPN cells.

.

Question 5: Is the broadcasting of ETWS/CMAS notifications in an SNPN cell enabled in Rel-17?

A5: SA2 is of the opinion that the specification should support ETWS/CMAS notifications in SNPN cells.

# 2	Actions

To RAN2

ACTION: 	SA2 kindly asks RAN2 to take into consideration the information above.

To CT1, SA1

ACTION: 	SA2 kindly asks CT1 and SA1 to take into consideration the information above and provide any feedback, if needed.

# 3	Dates of next TSG SA WG 2 meetings

SA2#144E		12-16 April				Electronic meeting

SA2#145E		17-28 May				Electronic meeting