3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100833

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | draftCR | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE location determination in reference coordinate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the last meeting, UE location determination in local Cartesian coordinate was agreed and a new definition was introduced by S2-2009212 for Local Co-ordinates: co-ordinates relative to a local Cartesian System whose origin may have a known WGS84 location. There is also a remaining issue that Local Co-ordinates are to be defined in TS 23.032. It was observed that RAN WG has discussed the TRP information (location) represented by local coordinate since June 2020, and several conclusions can be found in TS 38.455. It is proposed to refer to RAN conclusion in terms of Relative Cartesian Location, in order to define local Cartesian coordinate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description for local Cartesian coordinate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description for local Cartesian coordinate in baseline specification but use cases agreed for 5G eLCS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 4, 5, 5.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

1	Scope

The present document defines an intermediate universal Geographical Area Description which subscriber applications,  GSM, UMTS, LTE and 5GS services can use and the network can convert into an equivalent radio coverage map.

For GSM, UMTS, LTE and 5GS services which involve the use of an "area", it can be assumed that in the majority of cases the Service Requester will be forbidden access to data on the radio coverage map of a particular PLMN and that the Service Requester will not have direct access to network entities (e.g. BSC/BTS, RNC/Node B, eNode B or NG-RAN).

The interpretation by the PLMN operator of the geographical area in terms of cells actually used, cells that are partly within the given area and all other technical and quality of service aspects are out of the scope of the present document.

This specification also provides a description of velocity that may be associated with a universal Geographical Area Description when both are applied to a common entity at a common time.

# * * * * Second change * * * *

4	Reference system

The reference system chosen for the coding of locations is the World Geodetic System 1984, (WGS 84), which is also used by the Global Positioning System, (GPS), or a local Cartesian coordinate.

In case of WGS 84:

The origin of the WGS 84 co-ordinate system is the geometric centre of the WGS 84 ellipsoid. The ellipsoid is constructed by the rotation of an ellipse around the minor axis which is oriented in the North-South direction. The rotation axis is the polar axis of the ellipsoid, and the plane orthogonal to it and including the centre of symmetry is the equatorial plane.

The relevant dimensions are as follows:

Major Axis (a) = 6378137 m

Minor Axis (b) = 6356752,314 m

First eccentricity of the ellipsoid

Co-ordinates are then expressed in terms of longitude and latitude relevant to this ellipsoid. The range of longitude is -180° to +180°, and the range of latitude is -90° to +90°. 0° longitude corresponds to the Greenwich Meridian, and positive angles are to the East, while negative angles are to the West. 0° latitude corresponds to the equator, and positive angles are to the North, while negative angles are to the South. Altitudes are defined as the distance between the ellipsoid and the point, along a line orthogonal to the ellipsoid.

In case of local Cartesian coordinate:

The origin of a location Cartesian coordinate (aka reference point) may be a Coordinate ID, which is configured by the PLMN operator, or expressed as ellipsoid point (WGS 84) i.e. Reference Point Position or Reference Point High Accuracy Access Position. Examples of reference poin can be found in TS 38.455[x], clause 9.2.51.

# * * * * Third change * * * *

# 5	Shapes

The intention is to incorporate a number of different shapes, that can be chosen according to need.

-	Ellipsoid Point;

-	Ellipsoid point with uncertainty circle;

-	Ellipsoid point with uncertainty ellipse;

-	Polygon;

-	Ellipsoid point with altitude;

-	Ellipsoid point with altitude and uncertainty ellipsoid;

-	Ellipsoid Arc;

-	High Accuracy Ellipsoid point with uncertainty ellipse;

-	High Accuracy Ellipsoid point with altitude and uncertainty ellipsoid;

-	Location under local Cartesian coordinate.

Each shape is discussed individually.

# * * * * Fourth change * * * *

5.X	Location under local Cartesian coordinate

Location under local Cartesian coordinate is expressed by a reference point, and information relative to the reference point either a relative Geodetic location or a relative Cartesian location.

A reference point may be a Coordinate ID, which is configured by the PLMN operator, or expressed as an ellipsoid point (WGS 84) i.e. Reference Point Position or Reference Point High Accuracy Access Position.

A relative geodetic location comprises Milli-Arc-Second Units, Height Units, Delta Latitude, Delta Longitude, Delta Height and Location uncertainty.

A relative Cartesian location comprises XYZ unit (e.g. mm, cm, etc) , X value (represents northing from reference point), Y value (Positive value represents easting from reference), Z value (Positive value represents height above reference point) and Location uncertainty.

# * * * * End of changes * * * *