SA WG2 Meeting #143E e-meeting	S2-2101340

Elbonia, February 24 – March 09, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2586 | rev | 1 | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: NSSF services update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  |  | Date: |  | 2021-02-18 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This is the companion CR to TS 23.501 CR2567 where the Network Slice restriction service provided by NSSF is defined, as agreed in conclusion 8.4 of 23.700-91. In addition, based on the companion CR0201 an update to consumers of Nnssf_NSSelection service is required to allow NWDAF to derive slice load analytics for a network slice instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of subscribe, unsubscribe, notify Network Slice restriction service operations New service consumer for Nnssf_NSSelection - NWDAF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete and inconsistent specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.1, 5.2.16.X (new), 5.2.16.X.Y (new), 5.2.16.X.Y1 (new), 5.2.16.X.Y2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 23.501 CR2567 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS 23.288 CR0201 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS 23.288 CR0202 |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

* * * * Start of Changes * * * *

#### 5.2.16.1	General

The following table illustrates the NSSF Services.

Table 5.2.16.1-1: NF Services provided by NSSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnssf_NSSelection | Get | Request/Response | AMF, NSSF in a different PLMN, SMF, NWDAF |
| Nnssf_NSSAIAvailability | Update | Request/Response | AMF |
|  | Subscribe | Subscribe/Notify | AMF |
|  | Unsubscribe |  | AMF |
|  | Notify |  | AMF |
|  | Delete | Request/Response | AMF |
| Nnssf_ NSRestriction | Subscribe | Subscribe/Notify | AMF |
|  | Notify |  | AMF |

NOTE:	The SMF uses the Nnssf_NSSelection_Get service operation during EPS interworking.

* * * * Next Change (ALL NEW TEXT) * * * *

#### 5.2.16.X	Nnssf_ NSRestriction service

##### 5.2.16.X.Y	Nnssf_NSRestriction_Subscribe service operation

Service operation name: Nnssf_NSRestriction_Subscribe

Description: This service operation enables the NF service consumer to subscribe to Network Slice and/or Network Slice instance restriction notifications.

Inputs, Required: None.

Inputs, Optional: S-NSSAI(s), NSI ID(s).

Outputs, Required: Subscription ID.

Outputs, Optional: None.

##### 5.2.16.X.Y1	Nnssf_NSRestriction_Unsubscribe service operation

Service operation name: Nnssf_NSRestriction_Unsubscribe

Description: This service operation enables the NF service consumer to unsubscribe to Network Slice and/or Network Slice instance restriction notifications.

Inputs, Required: Subscription ID.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.16.X.Y2	Nnssf_NSRestriction_Notify service operation

Service operation name: Nnssf_NSRestriction_ Notify

Description: This service operation enables NSSF to notify the Network Slice and Network Slice instance restrictions to the NF service consumer.

Inputs, Required: Subscription ID, restricted S-NSSAI(s).

Inputs, Optional: restricted NSI ID(s).

Outputs, Required: None.

Outputs, Optional: None.

* * * * End of Changes * * * *