3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100118

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on determination of Registration Area

Document for:	Discussion

Agenda Item:	6.1

Work Item / Release:	5GS_Ph1 / Rel-16

Abstract: This discussion paper aims at discussing the issue of the current Registration Area determination mechanism.

- Discussion

According to the current TS 23.501 and 23.502, the allowed NSSAI and Registration Area are determined in the registration procedure by AMF. It is specified in clause 5.15.5.2.1 of TS 23.501 that:

The serving AMF shall determine a Registration Area such that all S-NSSAIs of the Allowed NSSAI for this Registration Area are available in all Tracking Areas of the Registration Area (and also considering other aspects as described in clause 5.3.2.3) and then return to the UE this Allowed NSSAI and the mapping of the Allowed NSSAI to the Subscribed S-NSSAIs if provided. The AMF may return the rejected S-NSSAI(s) as described in clause 5.15.4.1.

It means that the constraint related to Network Slice is only “all S-NSSAIs of the Allowed NSSAI for this Registration Area are available in all Tracking Areas of the Registration Area”. So all neighbouring Tracking Areas that support the allowed NSSAI may be allocated into the same Registration Area.

It is specified in clause 4.2.2.2.1 of TS 23.502 that:

A UE needs to register with the network to get authorized to receive services, to enable mobility tracking and to enable reachability. The UE initiates the Registration procedure using one of the following Registration types:

-  Initial Registration to the 5GS;

-  Mobility Registration Update upon changing to a new Tracking Area (TA) outside the UE's Registration Area in both CM-CONNECTED and CM-IDLE state, or when the UE needs to update its capabilities or protocol parameters that are negotiated in Registration procedure with or without changing to a new TA, a change in the UE's Preferred Network Behaviour that would create an incompatibility with the Supported Network Behaviour provided by the serving AMF, or when the UE intends to retrieve LADN Information; or

-  Periodic Registration Update (due to a predefined time period of inactivity); or

-  Emergency Registration.

It means that the UE can initiate the registration procedure and update the allowed NSSAI only when it moves out of the Registration Area. Considering a scenario in Figure 1-1:

Figure 1-1

TA1 supports Slice1, and TA2 supports both Slice1 and Slice2. If UE initiate registration procedure in TA1 with Slice1 and Slice2 as requested NSSAI, the allowed NSSAI will be Slice1. The RA1 will include TA1 and TA2 according to the current AMF actions in clause 5.15.5.2.1 of TS 23.501. It means when the UE moves into TA2, the allowed NSSAI will still be Slice1 and the UE cannot use Slice2 even though Slice2 is supported in TA2. When the UE continues to move to TA3, which is out of RA1, it can try to register to Slice2. In other words, UE can register to Slice2 in TA2 only when the UE initiates an initial registration procedure in TA2.

It is proposed that TA support for Rejected S-NSSAIs should also be considered when determining RA. For instance, in this scenario, TA2 should be able to be kept out of RA1 because Slice2 is rejected and Slice2 is supported in TA2.

- Conclusion and proposal(s)

It is proposed that TA support for Rejected S-NSSAIs should be considered by AMF when determining the Register Area for UE. Some corrections should be added in clause 5.15.5.2.1 of TS 23.501.