3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100127

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

Title:	[DRAFT] LS Reply on Response to LS Reply on Enhancement of RAN Slicing

Response to:	LS (S2-2100019/ R3-207236) on Response to LS Reply on Enhancement of RAN Slicing from RAN3

Release:	Release 17

Work Item:	 FS_NR_Slice

Source:	[Huawei to be] SA2

To:	RAN3

Cc:	SA5, RAN2

Contact Person:

Name:	Haiyang, Sun

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

Thanks for checking with SA2 about the candidate solutions in TR 38.832.

According to the discussion, SA2 has evaluated the candidate solutions for service continuity as following:

- Solution 6.2.1, 6.2.3, 6.2.5 and 6.2.6 are configuration based solutions. Such solutions have no impacts on the UE and 5GC except for configuration signalling in Solution 6.2.1. The impacts on OAM is out of scope of SA2.

- Solution 6.2.2, 6.2.4 (the one with CN involvement), 6.2.7 and 6.2.8 are CN-based solutions. All these solutions have impacts on the 5GC and UE. Solution 6.2.2, 6.2.4 (the one with CN involvement), and 6.2.7 will remap the slice during the lifetime of the PDU Session, which is not supported by current SA2 specification. There is also impact on UE as the UE needs to be notified about the new allowed NSSAI and the corresponding S-NSSAI of the PDU Session. Solution 6.2.7 lets 5GC send new type of PDU Session Modification command to trigger UE to establish a new PDU Session with remapping S-NSSAI. This is not SSC Mode 3 as it requires to establish the new PDU Session with new S-NSSAI and DNN. The 5GC needs to update the Allowed NSSAI every time after the release of the original PDU Session. For all the CN-based solutions, it is not guaranteed the application could use the new PDU Session with new NSSAI. Currently, the application selects an S-NSSAI based on NSSP in URSP (see TS 23.503 clause 6.6.2), so these solutions cannot support service continuity and SA2 has no plan to change the current design.

- Solution 6.2.4(the one without CN involvement) is RAN-based solution. This solution has no impact on 5GC, but UE needs to be aware of the target RAN and this solution is only applicable for Xn Handover.

All in all, SA2 prefers to use configuration based solutions as conclusion for the FS_NR_Slice Study Item.

2. Actions:

To RAN3 group.

ACTION: 	SA2 asks RAN3 group to

Kindly take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #144E		April 12 – 16, 2021			Elbonia

TSG-SA2 Meeting #145E		May 17 – 28, 2021			Elbonia