# 3GPP TSG-SA5 Meeting #135e	S5-211153

# 25 Jan to 3 Feb 2021, E-meeting

# Source:	China Telecom

# Title:	Update solution on MDA assisted energy saving to provide necessary information in output

# Document for:	Approval

# Agenda Item:	6.5.4

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

# 3	Rationale

One of the outputs of the potential solution of MDA assisted energy saving is the Recommended NR Cells (ES-Cell) to enter energySaving state. This recommendation may have multiple interpretations based the current energySaving state that the NR Cells:

1. For the NR Cells which is not in the energySaving state:

  - If they are also listed in this recommendation, it suggests asking them to enter the energySaving state (cf. isEnergySaving state in [1]).

  - If they are not listed in this recommendation, it suggests asking them to keep the not-energySaving state (cf. isNotEnergySaving state in [1]).

2. For the NR Cells which had already entered the energySaving state:

  - If they are also listed in this recommendation, it suggests asking them to keep in the energySaving state.

  - If they are not listed in this recommendation, this recommendation suggests asking them to quit the energySaving state. In other words, it is suggested to ask them to enter the not-energySaving state.

Observation 1: Recommended NR Cells (ES-Cell) to enter energySaving state in the output of the potential solution of the MDA assisted energy saving has multiple interpretations based the current NR Cells’ energySaving state.

Observation 2: the meaning of the Recommended NR Cells (ES-Cell) to enter energySaving state is clear when assuming the current energySaving state of the NR Cells are known by this MDA service consumer.

As discussed in the S5-206373, the energySaving state of the cells are currently not available for the NWDAF, which may also be the consumer of the MDA services. In order to get rid of the potential ambiguity, we have the following proposal:

Proposal:  Since the MDA has access to the MOIs of cells and uses the related information as the input, the current Cells EnergySaving State information can be retrieved and provided to the consumer as a part of the output of the solution of MDA assisted energy saving.

# 4	Detailed proposal

1st Modified Section

### 6.6.1	MDA assisted energy saving

#### 6.6.1.1	Use case

Energy saving is a critical issue for the 5G operators. Energy saving is achieved by activating the energy saving mode of the NR capacity booster cell or 5GC NF (e.g. UPF etc.), and the energy saving activation decision making may be based on the various information such as load information of the related cells/UPFs, the energy saving policies set by operators as specified in TS 28.310 [14] and the energy saving instructions provided by MDAS producer as described in TR 28.813 [23].

As the conclusion from clause 7.2 of the TR 21.866 [15], "The EE Control and Coordination Function: a self-managed automated process to control and coordinate system wide power saving operations including the access networks, core network, backhaul/fronthaul transmission networks, backbone networks and other subsystems", the management system has the overall view of network load information and it could also take the inputs from the control plane analysis (e.g. the analytics provided by NWDAF). The management system may provide the network wide analytics and cooperate with Core and RAN domains and decide on which cell/UPFshould move into energy saving mode in a coordinated manner.

There are various performance measurements could be used as inputs by MDA for energy saving analysis, for example, the EE related performance measurements, (e.g. PDCP data volume of cells, PNF temperature, and PNF power consumption etc.) for the gNBs, and the data volume, number of PDU sessions with SSC mode 1 (see TS 23.501 [13]), delay related measurements, and VR usage for UPFs.

The composition of the traffic load could be also considered as inputs for energy saving analysis. (e.g., the percentage of high-value traffic in the traffic load). The variation of traffic load may be related to the network data (e.g., historical handover information of the UEs or network congestion status). Collecting and analysing the network data with machine learning tools may provide predictions related to the trends of traffic load. The composition and the trend of the traffic load may be used as references for making decision on energy saving.

MDAS may also obtain NF location or other inventory information such as energy efficiency and the energy cost of the data centers, while analysing historical network information. Based on the collected information, MDAS producer makes analysis and gives suggestions to network management in optimization suggestion for 5G Core NF deployment options in high-value traffic region (e.g. location of VNF in context of energy saving). The information from control plane data analysis from NWDAF, such as UE Communication analytics (see TS 23.288 [18]), may also be used as input for energy saving analysis and instruction.

The decision of core NF and RAN node energy saving should be coordinated by management system to guarantee the overall network and service performance are not affected as much as possible.

To achieve an optimized balance between the energy consumed and the performance provided by the network, MDAS can be used to provide an analytics report by analysing the above information comprehensively to assist the energy saving.

#### 6.6.1.2	Potential requirements

REQ-MDA_ES-CON-1	The MDAS producer should have a capability to provide the analytics report describing the energy saving instruction.

REQ-MDA_ES-CON-2	The analytics report describing the energy saving instruction should contain the following information:

-	The identifier of the energy saving instruction described in the analytics report;

-	Root cause of the energy consumption issue;

-	Recommended NR Cells (ES-Cell) to enter energySaving state;

-  	Recommended candidate cells with precedence for taking over the traffic of each ES-Cell.

-	Recommended UPFs (ES-UPF) to enter energySaving state;

- 	Recommended candidate UPFs with precedence for taking over the traffic of each ES-UPF.

#### 6.6.1.3	Possible solutions

##### 6.6.1.3.1	Solution description

The MDAS producer correlates and analyses the management data described in the following subclause to assist the management energy saving function to make energy efficiency decisions. As the table in clause 6.6.1.3.3 shows, the analytics report is able to be provided by the MDAS producer to describe the analytics result and recommendations of energy saving. This procedure may be triggered by the request or periodically.

Energy saving activation decision making may be based on the various information such as load information. The prediction result of these information can be used by operators to make energy-saving policies. There are many ML models which may use ML algorithms for predicting these information, such as energy-saving scenarios prediction models and traffic load prediction models.

The prediction models are trained to be able to produce the expected training output from the training input. The data for models training may include RRC connection number, PRB utilization, energy consumption, service experience, etc. After the training process, the pre-trained models for predicting information used to make energy-saving policies can be obtained.

The more accurate the information prediction results are, the better the energy-saving policies based on the information prediction results will be. According to the result of information prediction and the energy-saving benefit, the MDA service can assist the energy saving policy decision making by recommending the optimal ML models which can provide more accurate information prediction results for consumers.

The MDAS producer is informed when the recommended decisions are taken by the MDAS consumer to make energy saving decisions, so that the MDAS producer can get the service experience changes and traffic load changes to evaluate energy-saving benefits.  By maximizing the expected sum of energy-saving benefits, the MDAS producer may update recommended decisions and fine-tune the accuracy of the analytics result.

For example, the MDAS producer may take base station energy saving scenarios prediction model and the traffic load prediction model together as input. Based on the results of energy saving scenarios prediction and traffic load prediction, the MDAS producer may use the ML algorithms to calculate the energy-saving benefit based on base station related information (e.g., service experience changes), the traffic load changes as well as the prediction of the energy saving scenarios. And by maximizing the expected sum of energy-saving benefits, the MDAS producer may provide the optimal prediction models for making recommendation of base station energy saving policies accordingly.

##### 6.6.1.3.2	Data required for MDA assisted energy saving

The following table shows the potential data required to analyse the energy saving issue.

| Data Category | Required Data |
| --- | --- |
| Performance Measurements | PNF Power Consumption: Power consumed over the measurement period, see clause 5.1.1.19.2 of TS 28.552 [8]; PNF Energy consumption: The energy consumed, see clause 5.1.1.19.3 of TS 28.552 [8]; PNF Energy Temperature: The temperature over the measurement period, see clause 5.1.1.19.4 of TS 28.552 [8]; PNF Voltage: The voltage, see clause 5.1.1.19.5 of TS 28.552 [8]; PNF Current: The current, see clause 5.1.1.19.6 of TS 28.552 [8]; PNF humidity consumption: The percentage of humidity during the measurement period, see clause 5.1.1.19.7 of TS 28.552 [8]; PDCP Data Volume of NR cells: The transmitted PDCP data volume, see clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [8]; Virtual resource usage of NF: The resource usage of virtual network functions, see clause 5.7.1 of TS 28.552 [8]. Data volume of UPF: see clause 5.4 of TS 28.552 [8]. Delay related measurements of UPF: see clause 5.4 of TS 28.552 [8]. Number of PDU sessions with SSC mode 1 on UPF. |
| QoE Data | The measurements that are collected are DASH (see TS 26.247 [16]) and MTSI (see TS 26.114 [17]) measurements (see TS 28.406 [24]). |
| NRM | MOIs of the cells, UPFs and SMFs, see TS 28.541 [20]. |
| Alarm information | The alarm information of the cells, UPFs and SMFs. |
| Network analytics data | The control plane analysis result from the NWDAF defined in TS 23.288 [18], e.g., observed service experience related network data analytics. |
| Pre-trained Prediction Models | The pre-trained models, which may be based on ML algorithms and trained to be able to produce the expected training output for consumers, e.g., pre-trained base station energy-saving scenarios prediction models, pre-trained traffic load prediction models. Editor's note: The detailed information about where to obtain the pre-trained prediction models is FFS. |

##### 6.6.1.3.3	Analytics report for MDA assist energy saving

The following table shows the potential information carried in the analytics report of MDA assist energy saving.

| Analytics Report of MDA assisted energy saving | Attribute Name | Description |
| --- | --- | --- |
|  | Energy saving analytics identifier | The identifier of the MDA assisted energy saving |
|  | Location | The geographical area, UPFs or the cells where the unreasonable energy consumption exists |
|  | Root cause | The root cause of the part of the energy consumption that may be conserved, e.g., ultra-low traffic load area with energy consumption, excessive energy consumption |
|  | Recommended prediction models | The optimal prediction models which can provide more accurate information prediction results to assist the energy saving related decision making. |
|  | Current Energy Saving State of the cells | The information of the current energy saving state of the cells, e.g. the cells which are currently in the energy saving state. |
|  | Energy saving recommendations | For ES on NR cells. - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. |

End of Modified Sections