SA WG2 Meeting #143e	S2-2100257

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.682 | CR | 0473 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Roaming status monitoring event clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MONTE, TEI17 |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As specified in TS 23.682 clause 5.6.1.7, it's the responsibility of the HSS to detect the Roaming Status monitoring event type. According to TS 23.682 clause 5.6.1.7, the HSS does this based on the registration information that it receives from the other CN nodes. UE can be registered over 3GPP and N3GPP Access Type. In this case, it would be necessary to report both Roaming status instances. |  |  |  |  |  |  |  |  |
| Summary of change: |  | HSS detects Roaming Status monitoring event type based on the latest HSS registration state information. Multiple instances of Roaming status are added to allow the HSS to report both 3GPP and N3GPP Access Type related roaming status. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the trigger criteria for Roaming Status monitoring event type. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR builds on top of Rel-16 CR 0472 (S2-2100255) assuming that other CR is also agreed. If that other CR is not agreeable in Rel-16, then merging the contents of that Rel-16 CR into this Rel-17 CR should be considered for the sake of completeness. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.6.3.6	Reporting Event: Roaming Status

1b.	This monitoring event is detected as of step 1b of clause 5.6.3.1, which is when the HSS receives from a serving node a serving PLMN ID that is different from the one stored by the HSS.

2b.	Step 2b of clause 5.6.3.1 is executed. If the UE is registered to different PLMNs via 3GPP and N3GPP Access Type, then the HSS includes two instances of Roaming Status in the Monitoring Indication message.

3.	Step 3 of clause 5.6.3.1 is executed. The monitoring information indicates the ID of the serving PLMN and whether it is the home or a roaming PLMN. Operator policies in the SCEF may restrict the report, e.g. to indicate only whether the UE is in the home or in a roaming PLMN. The SCEF includes all Roaming Status instances that it received from the UDM.