SA WG2 Meeting #S2-143E	S2-2100298

24 February - 9 March 2021, Electronic meeting                         	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2555 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G-EIR Discovery and Selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no section to define the discovery and selection aspects of the 5G-EIR |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section for 5G-EIR discovery and selection is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to discover and select 5G-EIR. Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.X (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Applies to earlier Release-15. No impact on stage 3. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### *****START OF CHANGES****

### 6.3.X	5G-EIR discovery and selection

If the NF service consumer of the 5G-EIR (i.e. AMF) performs discovery and selection, the NF consumer shall utilize the NRF to discover the appropriate 5G-EIR instance(s) unless 5G-EIR instance information is available by other means, e.g. locally configured on NF consumer.

The 5G-EIR selection function in NF consumers is applicable to both 3GPP access and non-3GPP access.

The NF consumer or the SCP shall select a 5G-EIR instance within the PLMN of the NF consumer.

In the case of delegated discovery and selection, the SCP shall perform discovery and selection of an appropriate 5G-EIR instance within the PLMN of the NF consumer.

#### *****END OF CHANGES****