3GPP TSG-SA2 Meeting #143-E 	S2-2100335

# Online, 24th Feb 2021 – 9th Mar 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2561 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DCAMP related update of BSF services (23.501) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_DCAMP |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This paper proposes DCAMP related update of BSF services. The purpose of the update is to clarify that there may be both PCF for a PDU Session and PCF for a UE. They may be handled separately and independently by the BSF. (NOTE: In Rel-15 and Rel-16, only PCF for a PDU Session was applicable for the BSF) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a clarification to BSF services to indicate that there may be both PCF for a PDU Session and PCF for a UE. Each of them may separately and independently register itself at the BSF. Each of them may separately and independently be discovered by a consumer of the BSF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may not be clear that in the context of BSF there may be both PCF for a PDU Session and PCF for a UE. It may not be clear that they may be handled separately and independently by the BSF. This may mislead one to believe that the BSF can only handle registration and discovery of PCF for a PDU Session, as it used to be in Rel-15 and Rel-16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

### 7.2.15	BSF Services

The following NF services are specified for BSF as described in TS 23.503 [45]:

Table 7.2.15-1: NF Services provided by BSF

| Service Name | Description | Reference in TS 23.502 [3] |
| --- | --- | --- |
| Nbsf_Management | Allows a PCF to register/deregister itself and to be discoverable by NF service consumers. NOTE: There may be both PCF for a PDU Session and PCF for a UE. Each of them may separately and independently register itself at the BSF. Each of them may separately and independently be discovered by a consumer of the BSF. | 5.2.13.2 |

End of changes