# 3GPP TSG-SA WG2 Meeting #143E	S2-2100355

Elbonia, 24 February - 9 March, 2021	(Revision of S2-21xxxxx)

Title:	[DRAFT] Reply to LS on Clarification request for eNPN features

Response to:	Clarification request for eNPN features (S2-2100143 / R2-2102489)

Release:	Rel-17

Work Item:	NG_RAN_PRN_enh-Core

Source:	SA2

To:	RAN2, SA1

Cc:	RAN3, CT1

Contact Person:

#### Name:	Peter Hedman

E-mail Address:	 peter (dot) hedman (at) ericsson (dot) com

Attachments:	-

SA2 thanks RAN2 for the LS on Clarification request for eNPN features

SA2 discussed the questions and agreed the following answers.

Questions related to SNPNs with subscription or credentials by a separate entity:

Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ), and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area?

SA2 answer: These parameters should be set uniformly. The parameters are used to assist the UE in network selection and once the network selection has been performed the parameters play no role. There is no need to communicate the parameters between the NG-RAN nodes or between NG-RAN and 5GC, but the parameters are to be used by the UE NAS layer.

Question 2: Shall Group IDs be broadcasted per SNPN or per cell?

SA2 answer:  The Group IDs (note the name "Group ID for Network Selection (GIN)" has been used in SA2 CRs) should be broadcasted per SNPN. If not done and cells support multiple SNPNs and not uniform support of GIDs among the SNPNs, then UE will not have enough information to select the right SNPN.

Question related to support UE onboarding and provisioning for NPN:

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?)

SA2 answer: The ”onboardingEnabled” bit can be set/enabled per cell e.g. when onboarding is enabled in only part of the SNPN network and can also be used to avoid the load from onboarding UEs. The parameter is used to assist the UE in network selection. Once the PDU session for remote provisioning has been activated existing 5GS functionality applies for mobility.

Questions related to support IMS and emergency services for SNPN:

Question 4: Is the support of eCall over IMS assumed to be enabled in SNPN cells?

SA2 answer: SA2 would like SA1 to answer.

Question 5: Is the broadcasting of ETWS/CMAS notifications in an SNPN cell enabled in Rel-17?

SA2 answer: SA2 would like SA1 to answer.

2. Actions:

To RAN2 group:

ACTION: SA2 kindly requests RAN2 to take the above answers into account.

To SA1 group:

ACTION: SA2 kindly requests SA1 to answer questions 4 and 5 above.

3. Date of Next TSG-SA2 Meetings:

TSG SA WG2 Meeting 144E		12th – 16th April 2021	Electronic meeting

TSG SA WG2 Meeting 145E		17th – 28th May 2021	Electronic meeting