3GPP TSG-SA2 Meeting #143-e	S2-2100375

# Online, , 24th Feb 2021 - 9th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2566 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Generic secondary authentication/authorization by a DN during the establishment of a PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS |  |  |  |  |  | Date: |  |  | 2021-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the conclusion for KI#2 in TR 23.754 it says: Depending on 3GPP network operator and/or regulatory requirements, the UUAA is performed: - in 5GS: either as a separate procedure during the 5GS registration procedure (see clause 6.5.3.1.1), or when the UAV requests user plane resources for UAV operation (i.e. PDU session establishment) (see clause 8.X.1). From a UE implementation point of view it is mandatory to support UUAA during Registration and PDU session establishment procedure. From a network deployment point of view it is mandatory to support UUAA during PDU session establishment procedure. Clause 8.3.1 of the TR 23.754 further explains the UUAA procedure during PDU Session Establishment, where SMF uses SBI for the authentication and authorization. Currently for secondary authentication/authorization by a DN-AAA only EAP based mechanism is supported as explained in clause 5.6.6 of TS 23.501. The TS 23.501 needs to be updated to support the new SBI mechanism for secondary authentication/authorization by DN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.6.6 is updated to introduce support for Secondary authentication/authorization by DN in PDU session establishment using SBI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An UAV UE can not be authenticated and authorized to use UAS services when UUAA done during PDU session establishment. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

### 5.6.6	Secondary authentication/authorization by a DN during the establishment of a PDU Session

#### 5.6.6.x1	General

At PDU Session Establishment to a DN:

-	The DN-specific identity (TS 33.501 [29]) of a UE may be authenticated/authorized by the DN-AAA using EAP procedures or

-	The DN-specific identity of the UE may be authenticated/authorized by an AF in the DN using API exposed by the AF (via NEF).

NOTE 1:	the DN-AAA server or the authenticating AF may belong to the 5GC or to the DN.

NOTE 2:	All the aspects of Secondary authentication/authorization by a DN-AAA server explained in -in this clause  is also applicable for authentication/authorization using API, unless specified otherwise. The specific aspects of authentication/authorization using API is explained in sub-clause 5.6.6.x2.

Based on the SM policy associated with the DN and SM subscription data, the SMF determines whether to invoke API based mechanism or to use DN-AAA based mechanism for secondary authentication/authorization. For a single PDU session only either DN-AAA based mechanism or API based mechanism can be used for authentication/authorization. A UE subscription can have access to multiple DNs, with one DN requiring secondary authentication/authorization using DN-AAA and API based secondary authentication/authorization for another DN.

Editor's Note: It is FFS how the UE knows which authentication mechanism will be used for a DNN/S-NSSAI

#### At PDU session establishment to a DN, the SMF may use  secondary authentication/authorization as follows:

-	If DN-AAA based mechanism is used and if the UE provides authentication/authorization information corresponding to a DN-specific identity during the Establishment of the PDU Session, and the SMF determines that Secondary authentication/authorization of the PDU Session Establishment is required based on the SMF policy associated with the DN,the SMF passes the authentication/authorization information of the UE to the DN-AAA server via the UPF if the DN-AAA server is located in the DN. If the SMF determines that Secondary authentication/authorization of the PDU Session Establishment is required but the UE has not provided a DN-specific identity as part of the PDU Session Establishment request, the SMF requests the UE to indicate a DN-specific identity using EAP procedures as described in TS 33.501 [29]. If the Secondary authentication/authorization of the PDU Session Establishment fails, the SMF rejects the PDU Session Establishment.

NOTE 3:	If the DN-AAA server is located in the 5GC and reachable directly, then the SMF may communicate with it directly without involving the UPF.

-	If the Secondary authentication/authorization of the PDU Session Establishment fails, the SMF rejects the PDU Session Establishment.

-	The DN-AAA server or the AF (via NEF) may authenticate/authorize the PDU Session Establishment.

-	When DN-AAA server or AF (via NEF) authorizes the PDU Session Establishment, it may send DN Authorization Data for the established PDU Session to the SMF. The DN authorization data for the established PDU Session may include one or more of the following:

-	A DN Authorization Profile Index which is a reference to authorization data for policy and charging control locally configured in the SMF or PCF.

-	a list of allowed MAC addresses for the PDU Session; this shall apply only for PDU Session of Ethernet PDU type and is further described in clause 5.6.10.2.

-	a list of allowed VLAN tags for the PDU Session; this shall apply only for PDU Session of Ethernet PDU type and is further described in clause 5.6.10.2.

-	DN authorized Session AMBR for the PDU Session. The DN Authorized Session AMBR for the PDU Session takes precedence over the subscribed Session-AMBR received from the UDM.

-	Framed Route information (see clause 5.6.14) for the PDU Session.

SMF policies may require DN authorization without Secondary authentication/authorization. In that case, when contacting the DN-AAA or AF (via NEF) server for authorization, the SMF provides the GPSI of the UE if available.

Such Secondary authentication/authorization takes place for the purpose of PDU Session authorization in addition to:

-	The 5GC access authentication handled by AMF and described in clause 5.2.

-	The PDU Session authorization enforced by SMF with regard to subscription data retrieved from UDM.

Based on local policies the SMF may initiate Secondary authentication/authorization at PDU Session Establishment. The SMF provides the GPSI, if available, in the signalling exchanged with the DN-AAA during Secondary authentication/authorization.

If DN-AAA based mechanism is used, after the successful Secondary authentication/authorization, a session is kept between the SMF and the DN-AAA.

The UE provides the authentication/authorization information required to support Secondary authentication/authorization by the DN over NAS SM.

NOTE 4:	The way for the UE to acquire such information is not defined.

SMF policies or subscription information (such as defined in TS 23.502 [3] Table 5.2.3.3.1) may trigger the need for SMF to request the Secondary authentication/authorization and/or UE IP address / Prefix from the DN-AAA server or AF (via NEF).

When SMF adds a PDU Session Anchor (such as defined in clause 5.6.4) to a PDU Session Secondary authentication/authorization is not carried out, but SMF policies may require SMF to notify the DN when a new prefix or address has been added to or removed from a PDU Session or N6 traffic routing information has been changed for a PDU Session.

When SMF gets notified from UPF with the addition or removal of MAC addresses to/from a PDU Session, the SMF policies may require SMF to notify the DN-AAA server or AF (via NEF).

Indication of PDU Session Establishment rejection is transferred by SMF to the UE via NAS SM.

If the DN-AAA or AF (via NEF) sends DN Authorization Data for the authorized PDU Session to the SMF and dynamic PCC is deployed, the SMF sends the PCF the DN authorized Session AMBR and/or DN Authorization Profile Index in the DN Authorization Data for the established PDU Session.

If the DN-AAA or AF (via NEF) sends DN Authorization Profile Index in DN Authorization Data to the SMF and dynamic PCC is not deployed, the SMF uses the DN Authorization Profile Index to refer the locally configured information.

NOTE 5:	DN Authorization Profile Index is assumed to be pre-negotiated between the operator and the administrator of DN-AAA server or AF (via NEF).

If the DN-AAA or AF (via NEF) does not send DN Authorization Data for the established PDU Session, the SMF may use locally configured information.

At any time, a DN-AAA server or AF (via NEF) may revoke the authorization for a PDU Session or update DN Authorization Data for a PDU Session. According to the request from DN-AAA server or AF (via NEF), the SMF may release or update the PDU Session. See clause 5.6.14 when the update involves Framed Route information.

At any time, a DN-AAA server or AF (via NEF) or SMF may trigger Secondary Re-authentication procedure for a PDU Session established with Secondary Authentication as specified in clause 11.1.3 in TS 33.501 [29].

During Secondary Re-authentication/Re-authorization, if the SMF receives from DN-AAA or AF (via NEF) the DN authorized Session AMBR and/or DN Authorization Profile Index, the SMF shall report the received value(s) to the PCF.

The procedure for secondary authentication/authorization by a DN-AAA server during the establishment of a PDU Session is described in TS 23.502 [3] clause 4.3.2.3.

#### 5.6.6.x2	API based secondary authentication/authorization by a DN during the establishment of a PDU Session

Apart from the generic aspects mentioned in clause 5.6.6.x1, below specific aspects apply to API based secondary authentication/authorization.

At PDU session establishment to a DN, the SMF may use API based for authentication/authorization of the PDU session by an AF in the DN by invoking service operations exposed by the NEF.

A UE that is in possession of a DN-specific idenitity shall always include the DN-specific identity in the PDU Session Establishment request for that DN. If the SMF determines that API based secondary authentication/authorization of the PDU Session establishment is required but the UE has not provided a DN-specific identity as part of the PDU Session Establishment request, then the PDU session establishment request is rejected by the SMF.

When invoking the NEF service operation for authentication/authorization, the SMF provides the DN-specific identity, the GPSI of the UE, the IP address reserved for the PDU session if available and PEI of the UE if available.

The procedure for API based secondary authentication/authorization during the establishment of a PDU Session is described in TS 23.502 [3] clause 4.3.2.x.

* * * * End of Changes * * * *