3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100384

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	New TS: On the User plane management of 5MBS

Document for:	Approval

Agenda Item:	8.9.2

Work Item / Release:	5MBS / Rel-17

Abstract: This document adds User plane management to the new TS.

- Background and Introduction

In the skeleton of the new TS, there is a section for "User plane Management".

It is proposed to add the related content in this sub-section.

- Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change* * * *

## 6.7	User plane Management

MBS traffic is delivered from a single data source to multiple UEs. Depending on whether the NG-RAN node supports MBS or not, SMF may determine to use different delivery methods to deliver MBS traffic in the 5GS.

From the viewpoint of 5GC, two delivery methods are possible for MBS multicast service:

-	5GC Individual MBS traffic delivery method: 5GC receives a single copy of MBS data packets and delivers separate copies of those MBS data packets to individual UEs via per-UE PDU sessions, hence for each such UE one PDU session is required to be associated with a multicast session.

-	5GC Shared MBS traffic delivery method: 5GC receives a single copy of MBS data packets and delivers a single copy of those MBS packets packet to a NG-RAN node, which then delivers them to one or multiple UEs

A same received single copy of MBS data packets by the 5GC may be delivered via both 5GC Individual MBS traffic delivery method for some UE(s) and 5GC Shared MBS traffic delivery method for other UE(s), as Figure 6.7-1 shows.

From the viewpoint of RAN, in the case of 5GC Shared MBS traffic delivery method, two delivery methods are available for the transmission of MBS packet flows over radio:

-	Point-to-Point (PTP) delivery method: a RAN node delivers separate copies of MBS data packet over radio to individual UE.

-	Point-to-Multipoint (PTM) delivery method: a RAN node delivers a single copy of MBS data packets over radio to a set of UEs.

A RAN node may use a combination of PTP/PTM to deliver an MBS packet to UEs.

As depicted in Figure 6.7-1, for 5GC Shared MBS traffic delivery method, PTP or PTM delivery and 5GC Individual delivery method may be used at the same time for a multicast MBS session.

MB-UPF

PSA a

PSA b

Incoming MBS traffic over N6

NG-RAN 1

NG-RAN 2

NG-RAN 3

5GC Shared MBS traffic delivery

5GC Individual MBS traffic delivery

UE 1

PDU Session a of UE 3

PDU Session b of UE 4

N9 transmission

N9 transmission

UE 2

UE 3

UE 4

PTP or PTM over radio

Figure 6.7-1: Schematic showing delivery methods

For MBS broadcast service, only 5GC Shared MBS traffic delivery method with PTM delivery is applicable.

# * * * * End of changes * * * *