# SA WG2 Meeting #143-e (e-meeting)	S2-2100450

# 24 February- 9 March 2021, Elbonia

Title:		[DRAFT] Reply LS on 5MBS progress and issues to address

Reply to:	Reply LS on 5MBS progress and issues to address (S2-2100110/ R3-211296)

Release:	Release 17

Work Item:	FS_5MBS

Source:	SA2

To:		RAN3

Cc:		RAN2

Contact person:	Miguel Griot

mgriot@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA2 thanks RAN3 for their Reply LS on 5MBS progress and issues to address (S2-2100110/ R3-211296).

Regarding RAN3 questions for clarification, SA2 would like to provide the following feedback:

RAN3 feedback: To progress this topic, RAN3 would like to ask SA2 to clarify the differences between the following terms: session start/session activation and session deactivation/session stop and their implication to NG-RAN functions.

SA2 answer: Session start and session activation refer to the same event. The implication to NG-RAN functions is that the NG-RAN is asked to notify UEs that session is being activated, and the NG-RAN receives the MBS session information to set up radio resources.

RAN3 also deduces from the TR that a UE may be in any CM/RRC state at MBS Session activation/start.

SA2 answer: SA2 confirms RAN3’s understanding.

Further, RAN3 would like to ask for confirmation from SA2 whether a UE is supposed to receive the MBS Session activation notification also when served by a non-supporting NG-RAN node.

RAN3 would like to ask RAN2 and SA2 to consider how to page the UEs which are not in RRC_CONNECTED state, whether a group notification towards NG-RAN supporting MBS and, if applicable, to NG-RAN nodes not supporting MBS is needed/feasible/beneficial comparing to the legacy paging methods.

SA2 answer: SA2 confirms that UEs which have joined an MBS session should be notified of MBS session activation even when they are camping on a non-supporting cell. SA2 asks RAN3/RAN2 if this can be done in a backward compatible manner. Otherwise, SA2 can consider means for the 5GC to fallback to paging UEs in non-supporting NG-RAN nodes as MBS session activation notification retransmission strategy.

# 2	Actions

To RAN3/RAN2

ACTION: 	SA2 asks RAN3/RAN2 to take SA2 feedback into account and to confirm if MBS session activation notification in non-supporting NG-RAN nodes is feasible in a backward compatible manner.

# 3	Dates of next TSG SA WG2 meetings

3GPPSA2#144-e	12-16 April 2021	Electronic Meeting

3GPPSA2#145-e	17-28 May 2021	Electronic Meeting