3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100478

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0217 | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF discovery per NWDAF service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As stated in the discussion paper, - a NWDAF instance maybe deployed with one or more of the NWDAF services, such as Nnwdaf_AnalyticsSubscription/Info services, Nnwdaf_MLModelProvision/Info services, Nnwdaf_DataRepository service and Nnwdaf_DataCollectionCoordination service. - Each NWDAF service has a service parameter of “Analytics ID” - The “Analytics ID” of each NWDAF service hosted by a NWDAF instance could be different. The current NWDAF discovery mechanism via NRF using Analytics ID at the NWDAF NF profile level is not sufficient to discovery a NWDAF per NWDAF service. 4 Options are listed and compared in the discussion paper i.e. S2-210xxxx to solve this issue. This contribution implements the proposed option 3 in TS23.288 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description of per service NWDAF discovery in clause 5.2.x. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Per service NWDAF discovery is not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information by using the NWDAF discovery principles defined in clause 6.3.13, TS 23.501 [2].

### 5.2.x	NWDAF Discovery and Selection per service

In this Release of the specification, multiple services are defined for an NWDAF (see details in clause 7). One NWDAF instance may deploy only some of these NWDAF services. As shown in Figure 5.2.x-1, an NWDAF offering multiple services does not necessary to support the same Analytics ID for all these services. This section describes the NWDAF Discovery and Selection under consideration of the NWDAF service.

Figure 5.2.x-1: Example for NWDAF support of different NWDAF services per Analytics ID

Each NWDAF instance shall register the supported analytics ID(s) for each of the deployed services in its profile in addition to what is specified in TS 23.501 [2] clause 6.2.6.2. Registering a supported analytics ID in the service parameters indicates that the analytics ID is supported by that service.  Registering a supported analytics ID in the NWDAF profile (outside the service parameters) indicates that the analytics ID is supported by the Nnwdaf_AnalyticsSubscription/Info service.

NOTE 1:	A Release 16 NWDAF does not include an analytics ID in the service parameters during the registration.

For the discovery of an NWDAF instance supporting the Nnwdaf_AnalyticsSubscription/Info service the NWDAF service consumer shall indicate the analytics ID and the NWDAF Serving Area information in the discovery request to the NRF. For the discovery of an NWDAF instance supporting any other NWDAF service, the NWDAF service consumer shall indicate the service name, the NWDAF Serving Area information and the analytics ID in the service parameters in the discovery request to the NRF.

NOTE 2:	Only an NWDAF of this Release can include an analytics ID in the service parameters during the discovery.

# * * * * End of changes * * * *