3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100504

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2517 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network initiated remote provisioning of NPN credentials for PNI-NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | About the conclusion of the study, following conclusion are given: “At least network initiated remote provisioning of credentials to allow access to PNI-NPN services should be supported in Rel-17” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Propose the network initiated remote provisioning procedure using Control Plane for PNI-NPN in TS 23.502. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing feature in constrast with study conclusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change, ALL TEXT ARE NEW* * * *

### 4.X	UE Onboarding

#### 4.X.Y	Onboarding for PNI-NPN

##### 4.X.Y.1	General

This clause specifies the procedures for Onboarding UE for PNI-NPN, which include Control Plane based procedure specified in clause 4.X.Y.2 and User Plane based procedure specified in clause 4.X.Y.3.

##### 4.X.Y.2	Control Plane based procedure

In case UE has registered to PLMN, figure 4.X.Y.2-1 shows the call flows for network initiated remote provisioning of NPN credentials for PNI-NPN.

UE

AMF

UDM

NEF

Provisioning Server

1. Naf_EventExposure_Subscribe

2. Ready to provide the NPN credential

3. Naf_EventExposure_Notify

4. Provisioning procedure

Figure 4.X.Y.2-1: procedure of network initiated remote provisioning of credentials for PNI-NPN

1. 	UDM to PS: Naf_EventExposure_Subscribe.

UDM invokes the Naf_EventExposure_Subscribe service operation via NEF to subscribe the event of provisioning the NPN credential as specified in clause 5.2.19.2 from PS which acts as the AF in the step.

2.	PS is ready to provide the NPN credential and trigger to perform step 3.

3.	PS to UDM: Naf_EventExposure_Notify.

PS provides the event information of provisioning the NPN credential to UDM via NEF which has been subscribed in step 1 via Naf_EventExposure_Notify service operation as specified in clause 5.2.19.2.

4.	Provisioning procedure is performed to provide the NPN credentials and updated subscription to UE.

Editor's note:	Details on the Provisioning procedure to provisioning the NPN credentials to UE is FFS.

Editor's note:	network function service involved in the above procedure need to be revised accordingly.

Editor's note:	whether and how to verify the UE by credential provider is FFS.

Editor's note:	whether and how to support UE initiated remote provisioning of credentials for PNI-NPN over control plane is FFS.

##### 4.X.Y.3	User Plane based procedure

Editor's note:	whether and how to support network initiated remote provisioning of credentials for PNI-NPN over user plane is FFS.

# * * * * End of changes * * * *