3GPP TSG-WG SA2 Meeting #143E e-meeting	S2-2100523

# Elbonia, February 24 – March 9, 2021						       			                                (was xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0222 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#19: Procedures for ML model provisioning and training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is concluded in KI#1 that an NWDAF decomposes into Analytics logical function and Model training logical function where the Model training logical function provides new services. Also, the new services, i.e., MLModelProvision, MLModelInfo, and MLModelUpdate, were endorsed in KI#19 as supporting services of Model training logical function. Thus, this CR proposes to add the procedures for the new services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to add the procedures for Model training logical function to expose and train an ML model(s) to the consumer NF with introduction of new services, i.e., Nnwdaf_MLModelProvision, Nnwdaf_MLModelInfo, Nnwdaf_MLModelUpdate, and Nnwdaf_MLTraining. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is uncertain how Model training logical function train and expose an ML model to the consumer NF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x, 6.x.1, 6.x.2, 6.x.3, 6.x.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change (all new text) * * * *

6.x	Procedure for ML Model Provisioning and Training

6.x.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.x.1-1 is used by NWDAF service consumer to subscribe/unsubscribe at NWDAF which supports Model Training logical function to be notified for an ML model on the related Analytics, using Nnwdaf_MLModelProvision Services as defined in clause 7.2.

Figure 6.x.1-1: ML Model for analytics subscribe/unsubscribe

1.	The NWDAF service consumer (e.g., other NWDAF which supports Analytics logical function) subscribes to or cancels subscription for an (set of) ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation.

When a subscription to an ML model for the Analytics is received, the NWDAF determines a specific ML model for each Analytics indicated in the subscription, and whether triggering further training for ML models is needed. In case the NWDAF determines further training is needed, the NWDAF may collect data by invoking  Nnf_EventExposure_Subscribe or train the ML models by invoking Nnwdaf_MLModelUpdate_Subscribe or Nnwdaf_MLModelTraining_Request.

2.	If the NWDAF service consumer subscribes to ML model for an Analytics, the NWDAF notifies the NWDAF service consumer with the ML model by invoking Nnwdaf_MLModelProvision_Notify service operation, based on the request from the NWDAF service consumer, e.g., Analytics ID, target reporting time, Area of Interest.

When a notification of an ML model is received, the NWDAF service consumer may determine whether triggering further training for the ML model is needed.

NOTE:	This procedure is only used within a single vendor environment in this release of the specification.

6.x.2	ML Model Request

The procedure in Figure 6.x.2-1 is used by NWDAF service consumer to request and get an ML model on the related Analytics from NWDAF which supports Model Training logical function, using Nnwdaf_MLModelInfo Services as defined in clause 7.2.

Figure 6.x.2-1: ML Model for analytics request

1.	The NWDAF service consumer (e.g., other NWDAF which supports Analytics logical function) requests an (set of) ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelInfo_Request service operation.

When a request for an ML model for the Analytics is received, the NWDAF determines a specific ML model for each Analytics indicated in the request, and whether triggering further training for ML models is needed. In case the NWDAF determines further training is needed, the NWDAF may collect data by invoking  Nnf_EventExposure_Subscribe or train the ML models by invoking Nnwdaf_MLModelUpdate_Subscribe or Nnwdaf_MLModelTraining_Request.

2.	The NWDAF responses the NWDAF service consumer with the ML model by invoking Nnwdaf_MLModelInfo_Response service operation, based on information in the request from the NWDAF service consumer, e.g., Analytics ID and Area of Interest.

When a response for an ML model is received, the NWDAF service consumer may determine whether triggering further training for the ML model is needed.

NOTE:	This procedure is only used within a single vendor environment in this release of the specification.

6.x.3	ML Model Update Subscribe/Unsubscribe

The procedure in Figure 6.x.3-1 is used by NWDAF service consumer to subscribe/unsubscribe at NWDAF which supports Model Training logical function to be notified for a trained ML model, using Nnwdaf_MLModelUpdate Services as defined in clause 7.2.

Figure 6.x.3-1: ML Model Update subscribe/unsubscribe

1.	The NWDAF service consumer (e.g., other NWDAF) subscribes to or cancels subscription for training an (set of) ML Model(s) by invoking the Nnwdaf_MLModelUpdate_Subscribe / Nnwdaf_MLModelUpdate_Unsubscribe service operation.

When a subscription to training an ML model is received, the NWDAF determines whether triggering new data collection is needed.

2.	If the NWDAF service consumer subscribes to training an ML model, the NWDAF notifies the NWDAF service consumer with the trained ML model by invoking Nnwdaf_MLModelUpdate_Notify service operation, based on the request from the NWDAF service consumer, e.g., target reporting time, Area of Interest, and the requested parameters for update ML model.

NOTE:	This procedure is only used within a single vendor environment in this release of the specification.

6.x.4	ML Model Training Request

The procedure in Figure 6.x.4-1 is used by NWDAF service consumer to request and get a trained ML model from NWDAF which supports Model Training logical function, using Nnwdaf_MLModelTraining Services as defined in clause 7.2.

Figure 6.x.4-1: ML Model Training request

1.	The NWDAF service consumer (e.g., other NWDAF) requests training an (set of) ML Model(s) by invoking the Nnwdaf_MLModelTraining_Request service operation.

When a request for training an ML model is received, the NWDAF determines whether triggering new data collection is needed.

2.	The NWDAF responses the NWDAF service consumer with the trained ML model by invoking Nnwdaf_MLModelTraining_Response service operation, based on information in the request from the NWDAF service consumer, e.g., target reporting time, Area of Interest, and the requested parameters for update ML model.

NOTE:	This procedure is only used within a single vendor environment in this release of the specification.

# * * * * End of changes * * * *