# SA WG2 Meeting #S2-143E	S2-2100539

# 24 February – 09 March, 2021, Electronic, Elbonia

Title:	[Draft] Reply LS on Response on Enhancement of RAN Slicing

Response to:	Response to LS Reply on Enhancement of RAN Slicing (S2-2100019/ R3-207236)

Release:	Rel-17

Work Item:	FS_NR_Slice

Source:	SA2

To:	RAN3

Cc:	RAN2, SA5

Contact Person:

Name:	Seokjung Kim

Tel. Number:

E-mail Address:	seokjung.kim@lge.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks RAN3 for the Response to LS Reply on Enhancement of RAN Slicing (S2-2100019 / R3-207236).

SA2 has discussed the candidate solutions in TR 38.832 v0.5.0 and would like to inform RAN3 about system level requirements as followings:

- If the NG-RAN node performs the slice re-mapping, the NG-RAN node shall inform the slice re-mapping decision and re-mapped slice to the CN e.g., for slice quota check, charging.

- Slice re-mapping shall consider Subscribed S-NSSAIs

- Service continuity shall be supported

- Allowed NSSAI shall not include the S-NSSAI which is not supported by the NG-RAN.

From CN impact/system level impact point of view, each solution is evaluated based on the system level requirements.

- Solutions 6.2.1.1.1 (Slice Re-mapping policy configured by OAM) and 6.2.1.1.2 (Slice Re-mapping policy configured by CN (during NG setup)) are not feasible because the re-mapping policy does not take into account the UE subscription information.

- Solution 6.2.1.1.3 (Slice Re-mapping policy configured by CN (during PDU session setup)) is feasible because the UE subscription information can be used to properly determine the re-mapping policy.

- Solution 6.2.1.2.1.4 (5GC Solution based on SSC-mode 3) is not feasible because service continuity is not supported if the application layer does not support IP address change.

- In Solution 6.2.2, the solution without CN involvement cannot cover all deployment scenario because it requires Xn connectivity between NG-RAN nodes.

- Solution 6.2.4 (Slice Remapping decision in 5GC) is feasible because it builds on the existing NG-based handover with the enhancement to change the slice of the on-going PDU session.

2. Action:

To RAN3 group:

ACTION: 	SA2 kindly requests RAN3 to take the above information into account.

3. Dates of Next TSG-SA2 Meetings:

3GPP TSG SA2#144E			12 – 16 April 2021		eMeeting

3GPP TSG SA2#145E			17 – 28 May 2021		eMeeting