3GPP TSG-SA WG2 Meeting #143-e	S2-2100554

# E-Meeting, 24th February – 09th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2600 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS voice over PS Session Supported Indication taking into account UE E-UTRA capability status |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_Ph1 |  |  |  |  |  | Date: |  |  | 2021-02-14 |
|  |  |  |  |  |  |  |  |  |  | 7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 24.301, there are scenarios that a UE supporting N1 and S1 mode may register to 5GS after disabling its S1 mode. Since its S1 mode is disabled, it is not possible for the UE to conduct EPS Fallback or RAT fallback even if 5GS supports them. But the pocedures of UE radio capability report, UE capability matching and how AMF set IMS voice over PS Session Supported Indication over 3GPP access do not take into account the status of S1 mode. When the 5GS only supports EPS Fallback or RAT Fallback, the AMF indicates to the UE IMS voice of PS Session is supported even if the S1 mode of the UE is disabled, which cause all the voice calls for the UE fails. Moreover, the UE has no chance to select to GERAN, UTRAN. It has already happened in real deployment and needs to be solved in specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When the AMF set indicate IMS voice over PS session supported over 3GPP access, AMF takes into account the status of UE E-UTRA capability. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS voice call fails when the network only supports EPS Fallback and the UE disables its S1 mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 5.16.3.2	IMS voice over PS Session Supported Indication over 3GPP access

The serving PLMN AMF shall send an indication toward the UE during the Registration procedure over 3GPP access to indicate if an IMS voice over PS session is supported or not supported in 3GPP access and non-3GPP access. A UE with "IMS voice over PS" voice capability over 3GPP access should take this indication into account when performing voice domain selection, as described in clause 5.16.3.5.

The serving PLMN AMF may only indicate IMS voice over PS session supported over 3GPP access in one of the following cases:

-	If the network and the UE are able to support IMS voice over PS session in the current Registration Area with a 5G QoS Flow that supports voice as specified in clause 5.7.

-	If the network or the UE are not able to support IMS voice over PS session over NR connected to 5GC, but is able for one of the following:

-	If the network and the UE are able to support IMS voice over PS session over E-UTRA connected to 5GC, and the NG-RAN supports a handover or redirection to E-UTRA connected to 5GC for this UE at QoS Flow establishment for IMS voice, and the UE E-UTRA capability is not disabled;

-	If the UE supports handover to EPS, the EPS supports IMS voice, and the NG-RAN supports a handover to EPS for this UE at QoS Flow establishment for IMS voice, and the UE E-UTRA capability is not disabled; or

-	If the UE supports redirection to EPS, the EPS supports IMS voice, and the NG-RAN supports redirection to EPS for this UE at QoS Flow establishment for IMS voice, and the UE E-UTRA capability is not disabled.

-	If the network is not able to provide a successful IMS voice over PS session over E-UTRA connected to 5GC, but is able for one of the following:

-	If the UE supports handover to EPS, the EPS supports IMS voice, and the NG-RAN supports a handover to EPS for this UE at QoS Flow establishment for IMS voice, and the UE E-UTRA capability is not disabled; or

-	If the UE supports redirection to EPS, the EPS supports IMS voice, and the NG-RAN supports redirection to EPS for this UE at QoS Flow establishment for IMS voice, and the UE E-UTRA capability is not disabled.

The serving PLMN provides this indication based e.g. on local policy, UE capabilities, HPLMN, whether IP address preservation is possible, whether NG-RAN to UTRAN SRVCC is supported and how extended NG-RAN coverage is, and the Voice Support Match Indicator from the NG-RAN (see TS 23.502 [3] clause 4.2.8a), UE E-UTRA capability status. The AMF determines UE E-UTRA capability status as specified in clause 4.5 of TS 24.301 [13]. The AMF in serving PLMN shall indicate that IMS voice over PS is supported only if the serving PLMN has a roaming agreement that covers support of IMS voice with the HPLMN. This indication is per Registration Area.

NOTE:	If the network supports EPS fallback for voice the 5GC can be configured not to perform the Voice Support Match Indicator procedure in order to set the IMS voice over PS session Supported Indication.

END OF CHANGES