3GPP TSG-WG SA2 Meeting #143E e-meeting  	S2-2100564

# Elbonia, Feb 24 – Mar 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2603 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | clarification on the determination of the egress port for DL traffic |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For downlink stream, the PDU Session for transmitting the DL stream could not be derived just from ingress port number. It needs to derive the egress port number for the DL stream. The ReplyLS (S2-2008498) from IEEE 802.1 responses that: - By default, a Stream is defined by its VLAN ID and Destination MAC Address without reference to the Source MAC address. It is possible to retrieve the egress bridge port for a given Destination MAC and VLAN ID pair from the FDB. Static filtering entries are used for Streams; therefore, the FDB is populated with egress port(s) for a given Destination MAC and VLAN ID pair. - The CNC configures static filtering entries for each Stream. Therefore, the corresponding management information for egress ports is available. - Static filtering entries (without PSFP) are suitable to retrieve the egress port for a Stream. So the egress port number for a stream could be derived from the static filtering entry e.g., ieee8021QBridgeStaticUnicastStaticEgressPorts or ieee8021QBridgeStaticMulticastStaticEgressPorts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use pre-configuration or static filtering entry to determine the egress port number for the TSN stream |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Functionality could not be properly implemented for transmitting DL TSN stream |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.2, I.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.28.2	5GS Bridge configuration

The configuration information of 5GS Bridge as defined in IEEE Std 802.1Q [98] clause 8.6.8.4, includes the following:

-	Bridge ID of 5GS Bridge.

-	Configuration information of scheduled traffic on ports of DS-TT and NW-TT:

-	Egress ports of 5GS Bridge, e.g., ports on DS-TT and NW-TT;

-	Traffic classes and their priorities.

NOTE 1:	In this Release of the specification, scheduled traffic (clause 8.6.8.4 in IEEE 802.1Q-2018 [98]) is only supported with protected windows, (see Annex Q.2 in IEEE 802.1Q [98]), therefore, it is enough to support AdminControlList, AdminBaseTime, AdminCycleTime and TickGranularity for the configuration of the 5GS.

The configuration information of 5GS Bridge as defined in IEEE Std 802.1Q [98], includes the following:

-	Chassis ID of 5GS Bridge;

-	Traffic forwarding information as defined in IEEE Std 802.1Q [98] clause 8.8.1:

-	Destination MAC address and VLAN ID of TSN stream;

-	Port number in the Port MAP as defined in IEEE Std 802.1Q [98] clause 8.8.1.

NOTE 2:	In this Release of the specification, the 5GS Bridge only supports traffic forwarding information for uplink TSN streams.

-	Configuration information per stream according to IEEE Std 802.1Q [98] clause 8.6.5.1 including:

-	Stream filters.

-	Stream gates.

NOTE 3:	In order to support IEEE Std 802.1Q [98] clause 8.6.5.1, it is required to support the Stream Identification function as specified by IEEE Std 802.1CB [83].

The SMF report the MAC address of the DS-TT port of the related PDU Session to TSN AF via PCF. The association between the DS-TT MAC address, 5GS Bridge ID and port number on DS-TT is maintained at TSN AF and further used to assist to bind the TSN traffic with the UE's PDU session.

Two models are supported to configure 5GS QoS for TSN traffic:

-	Based on the assumption that PSFP information is always provided by CNC: In this case the QoS Flows are setup based on the PSFP information provided by CNC;

NOTE 4:	PSFP information may be provided by CNC if TSN AF has declared PSFP support to CNC. TSN AF indicates the support for PSFP to CNC only if each DS-TT and NW-TT of the 5GS bridge has indicated support of PSFP.

-	Without requiring PSFP information provided by the CNC.: In this case, pre-configured QoS flows are used and configured e.g. during PDU session establishment as described in clause 5.28.4. Additional QoS flows are setup as necessary based on the PSFP, if available, as described in this clause.

TSN AF determines the DS-TT port MAC address(es) identifying the corresponding PDU session(s) carrying the TSN stream as follows: for uplink, it determines the DS-TT port MAC address corresponding to ingress port number derived from PSFP information as described in Annex I; for downlink, it determines the DS-TT port MAC address corresponding to egress port number derived from PSFP information and traffic forwarding information as described in Annex I. Flow direction of a TSN stream is determined as follows: if the ingress port is a DS-TT port, then the Flow direction is UL; otherwise if the ingress port(s) is (are) NW-TT port, the Flow direction is DL. Flow direction is part of the TSCAI as defined in clause 5.27.2.

The TSN AF uses the stream filter instances of PSFP information to derive the service data flow for TSN streams. The TSN AF uses the Priority values in the stream filter instances in PSFP information (if available) as defined in IEEE Std 802.1Q [98] clause 8.6.5.1, the 5GS bridge delay information (see clause 5.27.5) and may additionally use scheduled traffic information as defined in IEEE Std 802.1Q [98] clause 8.6.8.4, to derive the TSN QoS information (i.e. priority and delay) for a given TSN stream or flow of aggregated TSN streams.

When CNC configures the PSFP information to the TSN AF, TSN AF determines the TSC Assistance Container as described in clause 5.27.2. The TSN AF associates the TSN QoS information and TSC Assistance Container (if available) with the corresponding service data flow description and provides to the PCF and the SMF as defined in TS 23.503 [45] clause 6.1.3.23.

NOTE 5:	When the TSN stream priority information from PSFP is not available (priority value in stream filters is set to wild card), in certain configurations it can be possible to use the scheduled traffic information as defined in IEEE Std 802.1Q [98] clause 8.6.8.4 to derive the Priority of the TSN stream. For example, when there is a single downlink stream for a given DS-TT port, it can be possible to determine the affected DS-TT port in the downlink and the associated TSN stream priority based on the scheduled traffic information of the affected egress port, and to derive an estimated MDBV based on the gate open interval and the assumed ingress port bitrate.

# * * * * Second change * * * *

I.1	Determination of traffic pattern information

As described in clause 5.27.2, the calculation of the TSCAI relies upon mapping of information for the TSN stream(s) based upon certain IEEE standard information.

Additional traffic pattern parameters such as maximum burst size and maximum flow bitrate can be mapped to MDBV and GFBR.

The traffic pattern parameter determination based on PSFP (IEEE Std 802.1Q [98]), when available, is as follows:

-	Periodicity of a TSN stream is set equal to PSFPAdminCycleTime if there is only one PSFPGateControlEntry with a PSFPgateStatesValue set to Open in the PSFPAdminControlList. If there is more than one PSFPGateControlEntry with a PSFPgateStatesValue set to Open in the PSFPAdminControlList, then the Periodicity of the TSN Stream is set equal to sum of the timeIntervalValues from the first gate open instance to a next gate open instance in the PSFPAdminControlList. For aggregated TSN streams with same periodicity and compatible Burst Arrival Times, the periodicity of the aggregated flow of these TSN Streams is set equal to PSFPAdminCycleTime received from CNC for one of the TSN streams that are aggregated.

NOTE 1:	Given that only TSN streams that have the same periodicity and compatible Burst Arrival Time can be aggregated, the PSFPAdminCycleTime for those TSN streams is assumed to be the same.

-	Burst Arrival time of a TSN stream at the ingress port is determined based on the following conditions:

-	The Burst Arrival Time of a TSN Stream should be set to PSFPAdminBaseTime plus the sum of the timeIntervalValues for which the PSFPgateStatesValue is Closed in the PSFP AdminControlList until the first gate open time (i.e. until PSFPgateStatesValue set to Open is found). If the PSFPgateStatesValue is Open for the first timeIntervalValue, then the Burst Arrival time is set to PSFPAdminBaseTime. For aggregated TSN streams, the arrival time is calculated similarly, but using the time interval to the first PSFPgateStatesValue that is Open from the aggregated TSN streams.

-	Burst Size of a TSN stream at the ingress port (which is useful to map to MDBV) is determined based on the following conditions:

-	The Burst Size may be determined from TSN Stream gate control operations in the PSFPAdminControlList. If in the PSFPAdminControlList, IntervalOctetMax is provided for a PSFPGateControlEntry with an "open" PSFPgateStatesValue, the Burst Size is set to the IntervalOctetMax for that control list entry. If IntervalOctetMax is not provided, the Burst Size is set to the timeIntervalValue (converted from ns to s) of the PSFPGateControlEntry with an "open" PSFPgateStatesValue multiplied by the port bitrate.

-	When multiple compatible TSN Streams are aggregated, the Burst Size is set to the sum of the Burst Sizes for each TSN stream as determined above.

-	Maximum Flow Bitrate of a TSN stream (which is useful to map to GBR) is determined as follows:

-	The Maximum Flow Bitrate of a TSN Stream is equal to the summation of all timeIntervalValue (converted from ns to s) with PSFPgateStatesValue = Open, multiplied by the bitrate of the corresponding port, and divided by PSFPAdminCycleTime. For aggregated TSN streams, the same calculation is performed over the burst of aggregated streams (calculated using superposition, i.e., timeIntervalValue with PSFPgateStatesValue = Open of every stream is summed up, as they are assumed to have same periodicity, compatible Burst arrival time, and same traffic class if they are to be aggregated.

When CNC configures the PSFP information to the TSN AF, the TSN AF may use local information (e.g. local configuration) to map the PSFP information to an ingress port and/or egress port of the 5GS bridge. Further, the TSN AF can use the traffic forwarding information to map the PSFP information to an egress port of the 5GS bridge for TSN stream described via its destination address and VLAN identifier that matches the static filtering entry: one method to identify the egress port of the 5GS bridge for the TSN stream is to exploit the information element ieee8021QBridgeStaticUnicastStaticEgressPorts (or ieee8021QBridgeStaticMulticastStaticEgressPorts) (as specified in IEEE Std 802.1Q [98]) of a Static Filtering Database entry. This information element can be set equal to the egress port for which the static filtering rule applies. For TSN stream described via its destination address and VLAN identifier that matches the static filtering rule, the ieee8021QBridgeStaticUnicastStaticEgressPorts parameter can be used to infer the egress port for that TSN stream.

NOTE 2:	As an example, for the local configuration, the PSFP can use either the destination MAC address and VLAN identifier, or the source MAC address and VLAN identifier for stream identification. The TSN AF is pre-configured with either the MAC address of Ethernet hosts behind a given DS-TT port (identified by the DS-TT port MAC address), or the VLAN identifier used over a given DS-TT port, or both. When the TSN AF determines that one of the known Ethernet host's MAC address appears as a source or destination MAC address, it can identify that the ingress or egress port is the associated DS-TT port.

# * * * * End of changes * * * *