3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100569

Elbonia, Feb 24 – Mar 09, 2021	(revision of S2-2008410)

Source:	Huawei, HiSilicon

Title:	KI#3A, Conclusion update for N6 jitter eliminating

Document for:	Approval

Agenda Item:	8.5.1

Work Item / Release:	FS_IIoT / Rel-17

Abstract: Update of conclusion for N6 jitter eliminating.

1. Introduction/Discussion

It is concluded to support Hold and forward functionality, the Burst Spread should be supported at least when calculating TSCAI and Qbv parameters.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-20 V1.3.0.

# * * * * First change * * * *

8.5	Key Issue #3A: Exposure of deterministic QoS

The following is taken as the basis for the way forward:

Take solution#5 as basis for KI#3A:

-	The AF provides traffic related description and QoS requirement:

-	UE related Identification used to determine target UE PDU Session, AF Identification, a Traffic Description, a 5GS delay, Bandwidth, which are used to identify the target traffic and related QoS requirement.

-	Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Size, Burst Periodicity, and Burst Spread, a Timing Domain, which are used for efficient scheduling in RAN for Ethernet PDU sessions.

-	Burst Arrival Time on UPF is provided to the UPF. The UPF sends a notification to the AF via SMF/PCF for TSCAI adjustment when bursts arrive later or earlier than indicated by thresholds.

NOTE:	It is up to UPF implementation to use the Burst Arrival time on UPF, e.g., to detect the bursts arrive later or earlier than indicated by thresholds, execute the hold and buffer.

-	For ETH PDU Sessions, in order to reuse hold and forward functionality in the DS-TT and NW-TT, Qbv parameters can be derived by NEF/PCF based on AF request (with no impact to nodes other than NEF/PCF) and provided to NW-TT/DS-TT. It is assumed that Rel-16 hold and forward functionality in DS-TT and NW-TT is re-used.

Editor's note:	Whether a requirement exists that hold and forward functionality is needed for VIAPA services needs to be confirmed by SA1.

Editor's note:	Need for Jitter measurement is FFS.

# * * * * End of changes * * * *