3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100577

Elbonia, Feb 24 – Mar 09, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2: evaluations update

Document for:	Approval

Agenda Item:	8.2.1

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution introduces evaluations updates for solutions to KI#2.

1. Introduction

Evaluations updates taking into account the impacts of solution #50 and #55.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-07 V1.2.0.

# * * * * First change * * * *

7.2	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

The evaluation is based on major principles from the KI#2 solutions. The evaluation separates to different areas for each principle as below table:

Table 7.2-1: Evaluation of KI#2 related principles

| Principle | What it solves | Benefit, drawback | System impact | Open issues | Reference |
| --- | --- | --- | --- | --- | --- |
| N3IWF architecture | Service continuity aspect. Simultaneous data service aspect. | Benefit: Drawback: | Based on the existing release 16 architecture, No new architecture impact |  | Solution #13, #14, #15, #17, #18, #46, #47, #48, #49, #50, #51, #52 |
| N3IWF architecture with dual radio capability (2RX/2TX) UE enhancement. | Same as N3IWF architecture. | Benefit: Drawback: |  |  | Solution #13, #15, #48, #50 |
| N3IWF architecture with application layer service continuity support | Same as N3IWF architecture | Benefit: Drawback: | Solution#48: Network triggered UE registration to target network via N3IWF. Solution #49: The handover or the deregistration procedure is triggered by the AF; | It is FFS regarding which NF and which procedure is used by the network to signal to the UE to active non-3GPP access in another network. It is FFS regarding how the information is obtained by the 3GPP network from the application layer regarding the mobility decision. | Solution #48, #49 |
| N3IWF architecture with enhancement to keep UE in CM-Connected state in overlay network. | Paging aspect. | Benefit: Drawback: | UE logic to keep one PDU session in underlay network. Existing liveness check tool to be used over NWu interface to keep the NWu connection open. Apply to single radio UE |  | Solution #13, #14, #50 |
| N3IWF architecture with enhancement to keep UE in CM-Connected state in both overlay network and underlay network. | Paging aspect. | Benefit: Drawback: | RAN and CN impact to keep UE always in RRC-Inactive in the underlay network. PDU session in underlay network remains active. Impact to be able to select certain UE to be released to RRC-Inactive. Support single radio UE | Whether it is necessary to keep UE in CM-Connected in underlay network is FFS. | Solution #14 |
| N3IWF architecture with enhancement to introduce "inactive" mode on NWu interface. | Paging aspect. | Benefit: Drawback: |  | IKEv2 impact is FFS. How does the DL IPSec data reach UE when NAT timeout is FFS. | Solution #17 |
| N3IWF architecture with enhancement to split DL traffic to Uu interface and UL traffic to NWu interface for 2RX/1TX UE. | Simultaneous data service aspect. | Benefit: Drawback: | RAN impact for uplink control communication with single Tx UE/AMF impact to support split indication. | It will be up to RAN WG2 and RAN WG3 to evaluate and decide what the RAN impact will be or if reusing the existing RAN mechanism is feasible | Solution #18 |
| N3IWF architecture with enhancement to assist UE to choose either SNPN or PLMN to use Uu interface for better QoS. | Simultaneous data service aspect. | Benefit: Drawback: | Solution #46: The USRP rule is extended or a new policy is used for the UE to select the subscription. Solution #47: The solution provides supported PLMN list to the UE to decide whether to access SNPN service via SNPN N3IWF using PLMN PDU Session |  | Solution #46, #47, #50 |
| Inter-PLMN like architecture between SNPN and PLMN. | Service continuity aspect. | Benefit: Drawback: | To be concluded from KI#1 evaluation. Only the service mobility between NPN and PLMN from the outcome of KI#1 is considered by KI#2. |  | Solution #1, #2, #50. |
| Inter-PLMN like architecture between SNPN and PLMN with enhancements to enable SNPN operator to control access to PLMN services | Simultaneous data service aspect | Benefit: Drawback: | Network impacts for control the access to PLMN services using a HR PDU session or a secondary registration at PLMN. For secondary registration at PLMN, the 5GC can indicate to UE whether the secondary registration at PLMN is allowed and further the PLMN UDM will determine whether to accept UE's secondary registration. |  | Solution #50 |
| N3IWF architecture with MUSIM like implementation | Concurrent paging and data service for single radio UE. | Benefit: Drawback: | Solution#52: Impact due to support temporary leave. | Solution#52, it is not clear how the backup UP is maintained without interrupting ongoing service. | Solution #15, #50, #52. |
| N3IWF architecture with QoS degradation subscription/notification between overlay and underlay network. | Because of the traffic characteristics of VIAPA (e.g. High QoS KPI, potentially bringing heavy load to the networks, etc) the overlay network need to get QoS notification from the underlay network and adjust its QoS accordingly. | Benefit: Drawback: | UE to provide overlay network with information about the underlay network, so the overlay network could find the underlay network to make a QoS degradation subscription request. |  | Solution #51 |
| N3IWF architecture with paging/downlink data event subscription/notification between overlay and underlay network | Paging aspect | Benefit: Drawback: | UE, AMF, SMF, NEF impacts to enable UE receive the paging message from another network | How the NEF in SNPN/PLMN finds the NEF in PLMN/SNPN based on PLMN ID/SNPN ID is FFS. | Solution #55 |

# * * * * End of changes * * * *