SA WG2 Meeting #143E (e-meeting)	S2-2100591

February 24 – March 09, 2020, Elbonia

Source:	CATT

Title:	Establishment of a PDU Session associated with multicast session(s)

Document for:	Approval

Agenda Item:	8.9.2

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes the procedures for establishment of a PDU Session associated with multicast session(s).

- Introduction

As described in the conclusion of FS_5MBS, "The AMF shall select an SMF that supports 5MBS for multicast session join during PDU session establishment, which is used for sending join (i.e. handling of join requests for 5MBS and/or fallback to individual delivery)." Accordingly, this contribution proposes the procedures for establishment of a PDU Session associated with multicast session(s) to the TS.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change (all new text)********************

7.1.1.x	Establishment of a PDU Session associated with multicast session(s)

A PDU Session associated with multicast session(s) is established, if not involving multicast session join, using the procedures as specified in TS 23.502 [W] clause 4.3.2.2 with the following differences:

- In step 1, in the NAS Message to the AMF, the UE includes an indication of establishing a PDU Session associated with multicast session(s);

- In step 2, based on the indication of establishing a PDU Session associated with multicast session(s), the AMF selects an SMF capable of handling multicast sessions in the Serving PLMN as described in clause XX.

******************** 2nd Change (all new text)********************

7.1.1.y	Multicast Session Join during PDU Session Establishment without involving MBSF

The UE may initiate multicast session join during the PDU Session Establishment procedure, as shown in Figure 7.1.1.y-1.

Figure 7.1.1.y-1: Multicast Session Join during PDU Session Establishment procedure

1.	The UE initiates multicast session join by sending an NAS message (S-NSSAI(s), DNN, PDU Session ID, Request type, MBS Session ID, N1 SM container (PDU Session Establishment Request)) to the AMF. The MBS Session ID identifies the multicast session that the UE wants to join.

2.	The AMF selects an SMF capable of handling multicast sessions as described in clause XX. The AMF invokes the Nsmf_PDUSession_CreateSMContext Request (SUPI, S-NSSAI(s), DNN, PDU Session ID, AMF ID, Request Type, MBS Session ID, UE location information, N1 SM container (PDU Session Establishment Request)) towards the selected SMF.

3.	The SMF selects an MB-SMF for the multicast session as described in clause YY. The SMF invokes the Nmbsmf_MBSSession_Join Request (MBS Session ID, SUPI, S-NSSAI, DNN, UE location information, AMF ID) towards the selected MB-SMF.

4.	The MB-SMF may interact with the UDM/UDR to check whether the UE is authorized to join the multicast session.

The MB-SMF checks whether an MBS Session context exists for the requested MBS Session ID, and if so, updates the MBS Session context. Otherwise, the MB-SMF creates an MBS Session context with the MBS Session ID requested by the UE. The MB-SMF also updates its NF profile with the MBS Session ID for the newly created MBS Session context towards the NRF.

5.	The MB-SMF may perform an SM Policy Association Establishment or Modification procedure by invoking the Npcf_SMPolicyControl_Create or Npcf_SMPolicyControl_Update Request towards the PCF and get the MBS Session related PCC Rules.

6.	The MB-SMF selects an MB-UPF as described in clause ZZ. The MB-SMF sends an N4 Session Establishment/Modification Request to the MB-UPF and provides packet detection, enforcement and reporting rules to be installed on the MB-UPF for the MBS Session.

The MB-UPF acknowledges by sending an N4 Session Establishment/Modification Response containing the CN Tunnel Info.

7.	The MB-SMF sends the Nmbsmf_MBSSession_Join Response to the SMF.

8.	The MB-SMF sends the Namf_Communication_N1N2MessageTransfer (MBS Session ID, N2 SM information (MBS Session ID, MBS Session context information, QFI(s), QoS Profile(s), CN Tunnel Info, S-NSSAI, DNN)) to the AMF.

9.	The AMF sends the N2 MBS Session Request to the NG-RAN.

10.	If the NG-RAN supports MBS and no tunnel is established for the multicast session towards the MB-UPF, the NG-RAN allocates the AN Tunnel Info if configured to use N3 unicast transport for multicast sessions. If the AN resources for MBS session is not established yet, the NG-RAN establishes the AN resources for the MBS session.

11.	The NG-RAN sends the N2 MBS Session Response to the AMF.

12-14.	The AMF invokes the Nmbsmf_MBSSession_UpdateSMContext Request to the MB-SMF. The MB-SMF initiates the N4 Session Modification procedure towards the MB-UPF to provide/update the AN Tunnel Info. The MB-SMF sends the Nmbsmf_MBSSession_UpdateSMContext Response to the AMF.

15.	The PDU Session establishment procedure continues and completes as specified in TS 23.502 [W] clause 4.3.2.2.

NOTE:	Step 15 can be performed in parallel with step 3-14.

7.1.1.z	Multicast Session Join during PDU Session Establishment involving MBSF

Editor's note: Details are FFS.

******************** End of Change ********************