SA WG2 Meeting #143E (e-meeting)	S2-2100592

February 24 – March 09, 2020, Elbonia

Source:	CATT

Title:	MB-SMF discovery and selection

Document for:	Approval

Agenda Item:	8.9.2

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes description of MB-SMF discovery and selection for multicast/broadcast session management.

- Introduction

MB-SMF discovery and selection for multicast session management is needed, e.g. by the SMF when the UE requests to join an multicast session during a PDU Session establishment or modification procedure, or by the NEF/MBSF during MBS Session configuration procedures.

Based on TR 23.757, this contribution proposes the following solution of MB-SMF discovery and selection for multicast/broadcast session management:

-	The MB-SMF registers its capability related to multicast/broadcast session management (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), service area) as part of its profile to the NRF. In addition, when an multicast session is established, the MB-SMF creates the multicast session context and updates its profile towards the NRF with the multicast session information, i.e. information that uniquely identifies the multicast session (i.e. TMGI and/or source specific IP multicast address).

-	When the UE joins the multicast session via PDU session establishment or modification procedures, the SMF serving the PDU session queries the NRF and selects the MB-SMF currently serving the multicast session based on the MB-SMF information provided by the NRF.

-	When the multicast session context is deleted from the MB-SMF, e.g. due to multicast session release, the MB-SMF updates its profile towards the NRF, i.e., removing the multicast session information which is no longer served by the MB-SMF.

-	During multicast/broadcast session configuration procedures, unless the MB-SMF information is available by other means, e.g. locally configured in the NEF/MBSF, the NEF/MBSF queries the NRF with information of the multicast/broadcast session to be configured (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), service area), and selects the MB-SMF based on the MB-SMF information provided by the NRF.

A comparison between the proposed solution and some other solutions for MB-SMF discovery and selection via the NRF is given in Table 1.

Table 1: Comparison of solutions for MB-SMF discovery and selection via the NRF

| Solution | Description | Advantage | Disadvantage |
| --- | --- | --- | --- |
| 1 | MB-SMF registers its profile with TMGI range towards the NRF. | 1. One-step selection. 2. Dynamic update of MB-SMF profile upon multicast session establishment/release is not required. | 1. MBS Session may be identified by a source specific IP multicast address, instead of TMGI. 2. Different MB-SMFs have different TMGI ranges, thus multiple MB-SMFs in different service areas (/regions) cannot serve one MBS session of the same TMGI. |
| 2 | MBSF registers its profile with TMGI range towards the NRF, and selects the MB-SMF based on region. SMF discovers the MBSF via the NRF and then gets the MB-SMF ID from the MBSF. | 1. Multiple MB-SMFs in different service areas (/regions) can be selected by the MBSF to serve one MBS session of the same TMGI. 2. Dynamic update of MB-SMF profile upon multicast session establishment/release is not required. | 1. MBSF may not be available, e.g. for 5G MBS basic service (or function) level. 2. Two-step selection, i.e. SMF selects MBSF via the NRF, then gets MB-SMF ID from the MBSF. |
| 3 | MB-SMF registers its profile towards the NRF and updates its profile with current served multicast session information (TMGI/source specific IP multicast address). | 1. One-step selection. 2. Different MB-SMFs in the same PLMN can have the same TMGI ranges/values, thus multiple MB-SMFs in different service areas (/regions) can serve one MBS session of the same TMGI/source specific IP multicast address. 3. Multiple MB-SMFs in different service areas (/regions) can be selected according to the combination of TMGI/source specific IP multicast address and service area. | 1. Dynamic update of MB-SMF profile upon multicast session establishment/release is required. |

Taking into account the advantages and disadvantages of the solutions as listed above, it is suggested that the solution proposed in the present document (i.e. solution 3) is taken as a way forward.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change (all new text)********************

7.1.x	MB-SMF discovery and selection for multicast/broadcast session

To facilitate the MB-SMF discovery/selection for one multicast/broadcast session, the following mechanism is used:

-	The MB-SMF registers its capability related to multicast/broadcast session management (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), service area) as part of its profile to the NRF. In addition, when an multicast session is established, the MB-SMF creates the multicast session context and updates its profile towards the NRF with the multicast session information, i.e. information that uniquely identifies the multicast session (i.e. TMGI and/or source specific IP multicast address).

-	When the UE joins the multicast session via PDU session establishment or modification procedures, the SMF serving the PDU session invokes the Nnrf_NFDiscovery_Request including the multicast session information provided by the UE and possibly other information, i.e. the S-NSSAI and the associated NSI ID (if available), DNN, service area, etc., to query the NRF for MB-SMF information. Based on the multicast session information and other information for query, the NRF decides whether an MB-SMF serving the multicast session exists. If so, the NRF provides in Nnrf_NFDiscovery_Request response the information of the MB-SMF currently serving the multicast session. The SMF serving the PDU session selects the MB-SMF currently serving the multicast session, based on the MB-SMF information provided by the NRF.

-	When the multicast session context is deleted from the MB-SMF, e.g. due to multicast session release, the MB-SMF updates its profile towards the NRF, i.e., removing the multicast session information which is no longer served by the MB-SMF.

-	During multicast/broadcast session configuration procedures, unless the MB-SMF information is available by other means, e.g. locally configured in the NEF/MBSF, the NEF/MBSF queries the NRF with information of the multicast/broadcast session (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), service area), and selects the MB-SMF based on the MB-SMF information provided by the NRF.

******************** End of Change ********************