SA WG2 Meeting #S2-143E	S2-2100595

24 February - 09 March, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0223 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multiple NWDAF instances architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Multiple NWDAF instances architecture needs to be specified based on the conclusion of TR 23.700-91. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description of multiple NWDAF instances architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multiple NWDAF instances architecture is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 4.X (new) |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

4.1	General

The NWDAF (Network Data Analytics Function) is part of the architecture specified in TS 23.501 [2] and uses the mechanisms and interfaces specified for 5GC in TS 23.501 [2] and OAM services (see clause 6.2.3.1).

The NWDAF interacts with different entities for different purposes:

-	Data collection based on subscription to events provided by AMF, SMF, PCF, UDM, AF (directly or via NEF), and OAM;

-	Retrieval of information from data repositories (e.g. UDR via UDM for subscriber-related information);

-	Retrieval of information about NFs (e.g. from NRF for NF-related information);

-	On demand provision of analytics to consumers, as specified in clause 6.

A single instance or multiple instances of NWDAF may be deployed in a PLMN. If multiple NWDAF instances are deployed, the architecture supports deploying the NWDAF as a central NF, as a collection of distributed NFs, or as a combination of both. The multiple NWDAF instances architecture is define in clause 4.X.

NOTE 1:	When multiple NWDAFs exist, not all of them need to be able to provide the same type of analytics results, i.e., some of them can be specialized in providing certain types of analytics. An Analytics ID information element is used to identify the type of supported analytics that NWDAF can generate.

NOTE 2:	NWDAF instance(s) can be collocated with a 5GS NF.

******************** 2nd Change (all new text)********************

4.X	Multiple NWDAF instances architecture

The architecture of multiple NWDAF instances with an Aggregator NWDAF is shown in Figure 4.X-1, where an NWDAF can act as an aggregate point to collect data, analytics information and/or models from multiple other NWDAFs, which may have different Serving Areas, to produce the expected analytics information or models.

The architecture of multiple NWDAF instances without an Aggregator NWDAF is shown in Figure 4.X-2, where no NWDAF exists as the aggregated point and the NWDAFs may or may not exchange data, analytics information and/or models with each other.

A combination of both architectures for multiple NWDAF instances is also supported in a PLMN.

The architecture of multiple NWDAF instances with an Aggregator NWDAF enables the support of analytics aggregation as specified in clause Y.Z, and the support of Federated Learning based model sharing as specified in clause U.V.

NOTE:	This architecture allows for a hierarchy/tree with flexible number of layers/branches. The number and organisation of the layers, as well as the capabilities of each NWDAF instance, are deployment specific.

Figure 4.X-1: Multiple NWDAF instances architecture - with Aggregator NWDAF

Figure 4.X-2: Multiple NWDAF instances architecture - without Aggregator NWDAF

******************** End of Change ********************