SA WG2 Meeting #S2-143E	S2-2100600

24 February - 09 March, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2615 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Triggers for network analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Trigger conditions for the AMF and SMF to request for or subscribe to the analytics information from the NWDAF need to be specified, based on TR 23.700-91 clause 8.13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add trigger conditions for the AMF and SMF to request for or subscribe to the analytics information from the NWDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Triggers for the AMF and SMF to request analytics is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.x (new), 5.6.y (new) |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change (all new text)********************

5.3.x	Triggers for network analytics

Triggers for the AMF to request for or subscribe to the analytics information from the NWDAF may include:

-	UE access and mobility related event subscription by other NFs (e.g. SMF, NEF);

-	notifications received from UDR or CHF on UE subscription change;

-	local events;

-	analytics information received.

The trigger conditions depend on operator policy and may be configured locally in the AMF. When a trigger condition happens, the AMF may check local configuration or evaluate operator policy to decide if any analytics information is needed and if so, request for or subscription to the analytics information from the NWDAF.

The AMF may, upon receiving event subscription for a given UE by other NFs for notification about an Event ID, e.g. "UE moving in or out of Area of Interest", decide that mobility related abnormal behaviour analytics of the UE is needed and therefore subscribe to notifications of network analytics related to "Abnormal behaviour" of the UE from the NWDAF as described in TS 23.288 [86].

Upon notifications received from UDR or CHF on UE subscription change, the AMF may decide that mobility related abnormal behaviour analytics of the UE is needed and therefore subscribe to notifications of network analytics related to "Abnormal behaviour" of the UE from the NWDAF as described in TS 23.288 [86]. Alternatively, the AMF may initiate AM Policy association establishment or modification request towards the PCF, which in turn triggers the PCF to subscribe to network analytics related to "Abnormal behaviour" of the UE from the NWDAF as described in TS 23.503 [45].

The AMF may, upon detection of certain local events, e.g. frequent mobility re-registration of one or more UEs, subscribe to mobility related abnormal behaviour analytics of the UE(s) as described in TS 23.288 [86] in order to trace UE mobility trend and take appropriate actions.

The AMF may, upon reception of analytics information, e.g. mobility related abnormal behaviour analytics which indicates unexpected UE location for a UE, decide to request for or subscribe to UE mobility analytics from the NWDAF as described in TS 23.288 [86] to further track the UE's mobility behaviour.

******************** 2nd Change (all new text)********************

5.6.y	Triggers for network analytics

Triggers for the SMF to request for or subscribe to the analytics information from the NWDAF may include:

-	UE PDU Session related event subscription by other NFs (e.g. AMF, NEF);

-	notifications received from UDR or CHF on UE subscription change;

-	UE access and mobility event reports from the AMF;

-	local events;

-	analytics information received.

The trigger conditions depend on operator policy and may be configured locally in the SMF. When a trigger condition happens, the SMF may check local configuration or evaluate operator policy to decide if any analytics information is needed and if so, request for or subscription to the analytics information from the NWDAF.

The SMF may, upon receiving event subscription for a given UE by other NFs for notication about an Event ID, e.g. "UP path change", decide that communication related abnormal behaviour analytics of the UE is needed and therefore subscribe to notifications of network analytics related to "Abnormal behaviour" of the UE from the NWDAF as described in TS 23.288 [86].

Upon notifications received from UDR or CHF on UE subscription change, the SMF may decide that communication related abnormal behaviour analytics of the UE is needed and therefore subscribe to notifications of network analytics related to "Abnormal behaviour" of the UE from the NWDAF as described in TS 23.288 [86]. Alternatively, the SMF may initiate SM Policy association establishment or modification request towards the PCF, which in turn triggers the PCF to subscribe to network analytics related to "Abnormal behaviour" of the UE from the NWDAF as described in TS 23.503 [45].

The SMF may, upon receiving event report from the AMF, e.g. "UE moving in or out of Area of Interest", subscribe to network analytics related to "UE communication" or "Network Performance" with Analytics Filter including the Area of Interest as described in TS 23.288 [86], to observe communication performance of the UE(s) or the network in the specific area.

The SMF may, upon detection of certain local events, e.g. number of PDU session establishment or release reaches a threshold in a specific area, request for or subscribe to network analytics related to "Abnormal behaviour" as described in TS 23.288 [86] to detect whether there are any exceptional UE behaviours in this area.

The SMF may, upon reception of analytics information, e.g. communication related abnormal behaviour analytics which indicates "Unexpected long-live/large rate flows", decide to request for or subscribe to UE communication analytics from the NWDAF as described in TS 23.288 [86] to further track the UE's communication behaviour.

******************** End of Change ********************