SA WG2 Meeting #143e	S2-2100656

# Feb 24th – March 9th, 2021 ; Elbonia             				(revision of S2-210xxxx)

# Title:	(DRAFT) Reply LS on clarification request for eNPN features

# Response to:	S2-2100143/ R2-2102489

# Release:	Rel-17

# Work Item:	eNPN, NG-RAN_PRN_enh-Core

Source:	SA2

To:	RAN2

Cc:	RAN3, CT1, SA1

Contact Person:

Name:	Devaki Chandramouli

Tel. Number:

E-mail Address:	Devaki.chandramouli@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none.

1. Overall Description:

SA2 thanks RAN2 for their LS on Enhancement for Private Network Support for NG-RAN.

SA2 would like to offer the following answers for the questions raised by RAN2:

Questions related to SNPNs with subscription or credentials by a separate entity:

Question 1: Can RAN2 assume uniform support of external authentication related parameters (i.e., indicator for "access using credentials from a separate entity is supported", GID(s) ) , and indicator for "whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN") across a network or a registration area?

[SA2 answer] Yes, uniform support of access using credentials from separate entity can be assumed across an SNPN, and thus No GID based mobility restrictions are needed.

Question 2: Shall Group IDs be broadcasted per SNPN or per cell?

[SA2 answer] It is assumed to be broadcast per SNPN. Different GIDs may be supported by the different SNPNs sharing a cell.

Question related to support UE onboarding and provisioning for NPN:

Question 3: Can RAN2 assume uniform support of onboarding in all cells in an O-SNPN? (I.e. can RAN2 assume that all cells of an O-SNPN broadcasts the support for onboarding or can some cells not set the ”onboardingEnabled” bit to e.g. control RAN congestion?)

[SA2 answer] Yes, uniform support of onboarding in all cells in an O-SNPN can be assumed.

Questions related to support IMS and emergency services for SNPN:

Question 4: Is the support of eCall over IMS assumed to be enabled in SNPN cells?

[SA2 answer] yes.

Question 5: Is the broadcasting of ETWS/CMAS notifications in an SNPN cell enabled in Rel-17?

[SA2 answer] yes.

2. Actions:

To RAN2.

ACTION: 	SA2 kindly asks RAN2 to take the above answers into account for their work.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #144E   		12th April to 16th April 2021

TSG-SA2 Meeting #145E   		17th May to 28th May 2021