SA WG2 Meeting #143e	S2-2100663

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-210)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2540 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AF Support for TSC QoS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provide normative text to implement the Conclusion in TR23.700-20 clause 8.5 for Exposure of deterministic QoS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new sub-section describing Exposure for TSC QoS is proposed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed approach for exposure of deterministic QoS for TSC is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.33.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.6.6	Setting up an AF session with required QoS procedure

Figure 4.15.6.6-1: Setting up an AF session with required QoS procedure

1.	The AF sends a request to reserve resources for an AF session using Nnef_AFsessionWithQoS_Create request message (UE address, AF Identifier, Flow description(s), QoS reference, (optional) Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order)) to the NEF. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The NEF assigns a Transaction Reference ID to the Nnef_AFsessionWithQoS_Create request. For TSC QoS the AF request may in addition specify a 5GS delay, TSC Burst Size, a Guaranteed Flow Bit Rate, a Time Domain or Traffic Pattern Parameters (Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Periodicity).

2.	The NEF authorizes the AF request and may apply policies to control the overall amount of pre-defined QoS authorized for the AF. If the authorisation is not granted, steps 3 and 4 are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

3.	The NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Create request and provides UE address, AF Identifier, Flow description(s), the QoS reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order). Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.203 [24]).

For TSC QoS, the NEF sends the 5GS delay, TSC Burst Size, a Guaranteed Flow Bit Rate a Time Domain or Traffic Pattern Parameters to the PCF if received from the AF. If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2. If the PDU Session Type is Ethernet, for hold and forward buffer configuration at the UE/DS-TT or UPF/NW-TT, the NEF creates a Port Management Information Container containing Gate control information (see clause 5.28.3 of TS23.501 [2]) and sends it to the PCF. The PCF transparently forwards the container to the UE/DS-TT or UPF/NW-TT as described in Annex F.2.

Editor’s Note: Support for Hold and Forward Buffers is under discussion and may be revised.

4.	The PCF determines whether the request is authorized and notifies the NEF if the request is not authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration for this AF), and notifies the result to the NEF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]).

For TSC QoS, as authorized the PCF sets the PDB and/or MDBV according to the received 5GS Delay and Burst Size received from the AF, and the GFBR according to the AF requested Guaranteed Flow Bit Rate. AF specified parameter values are used to over-ride default values for the 5QI corresponding to the AF provided QoS Reference. If an AF request for a parameter value exceeds an authorization, the PCF may assign the highest authorized value and indicate the assigned value in the response to the NEF.

If the request is not authorized, or the required QoS is not allowed, NEF responds to the AF in step 5 with a Result value indicating the failure cause.

5.	The NEF sends a Nnef_AFsessionWithQoS_Create response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not. For TSC QoS, the response includes the assigned value of parameters if they did not meet the requested values.

6.	The NEF shall send a Npcf_PolicyAuthorization_Subscribe message to the PCF to subscribe to notifications of Resource allocation status and may subscribe to other events described in clause 6.1.3.18 of TS 23.503 [20].

7.	When the event condition is met, e.g. that the establishment of the transmission resources corresponding to the QoS update succeeded or failed, the PCF sends Npcf_PolicyAuthorization_Notify message to the NEF notifying about the event.

8.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the event reported by the PCF to the AF.

The AF may send Nnef_AFsessionWithQoS_Revoke request to NEF in order to revoke the AF request. The NEF authorizes the revoke request and triggers the Npcf_PolicyAuthorization_Delete and the Npcf_PolicyAuthorization_Unsubscribe operations for the AF request.

#### 4.15.6.6a	AF session with required QoS update procedure

Figure 4.15.6.6a-1: AF session with required QoS update procedure

1.	For an established AF session with required QoS, the AF may send a Nnef_AFsessionWithQoS_Update request message (AF Identifier, Transaction Reference ID, [Flow description(s)], [QoS reference], [Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order)]) to NEF for updating the reserved resources. Optionally, a period of time or a traffic volume for the requested QoS can be included in the AF request. The Transaction Reference ID provided in the AF session with required QoS update request message is set to the Transaction Reference ID that was assigned, by the NEF, to the Nnef_AFsessionWithQoS_Create request message. For TSC QoS the AF request may in addition specify a 5GS delay, TSC Burst Size, a Guaranteed Flow Bit Rate and a Time Domain or Traffic Pattern Parameters (Flow Direction, Burst Arrival Time at UE (uplink) or UPF (downlink), Burst Periodicity).

2.	The NEF authorizes the AF request of updating AF session with required QoS and may apply policies to control the overall amount of pre-defined QoS authorized for the AF. If the authorisation is not granted, steps 3 and 4 are skipped and the NEF replies to the AF with a Result value indicating that the authorisation failed.

3.	The NEF interacts with the PCF by triggering a Npcf_PolicyAuthorization_Update request and provides UE address, AF Identifier, Flow description(s), the QoS reference and the optional Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order). Any optionally received period of time or traffic volume is also included and mapped to sponsored data connectivity information (as defined in TS 23.203 [24]).

For TSC QoS hold and forward buffer configuration. the NEF creates a Port Management Information Container containing Gate control information (see clause 5.28.3 of TS23.501 [2]) and sends it to the PCF. The PCF transparently forwards the container to the UE/DS-TT or UPF/NW-TT as described in Annex F.2. If received from the AF, the NEF also sends the 5GS delay, TSC Burst Size, a Guaranteed Flow Bit Rate, a Time Domain or Traffic Pattern Parameters to the PCF. If the PCF determines that the SMF needs updated policy information, the PCF issues a Npcf_SMPolicyControl_UpdateNotify request with updated policy information about the PDU Session as described in the PCF initiated SM Policy Association Modification procedure in clause 4.16.5.2.

4.	The PCF determines whether the request is authorized.

If the request is authorized, the PCF derives the required QoS parameters based on the information provided by the NEF and determines whether this QoS is allowed (according to the PCF configuration for this AF), and notifies the result to the NEF. In addition, if the Alternative Service Requirements are provided, the PCF derives the Alternative QoS parameter set(s) from the one or more QoS reference parameters contained in the Alternative Service Requirements in the same prioritized order (as defined in TS 23.503 [20]).

For TSC QoS. as authorized the PCF sets the PDB and/or MDBV according to the received 5GS Delay and Burst Size received from the AF, and the GFBR according to the AF requested Guaranteed Flow Bit Rate. AF specified parameter values are used to over-ride default values for the 5QI corresponding to the AF provided QoS Reference. If an AF request for a parameter value exceeds an authorization, the PCF may assign the highest authorized value and indicate the assigned value in the response to the NEF. The PCF transparently forwards the Port Management Information Container to the UE/DS-TT or UPF/NW-TT as described in Annex F.2

If the request is not authorized or the required QoS is not allowed, NEF responds to the AF in step 5 with a Result value indicating the failure cause.

5.	The NEF sends a Nnef_AFsessionWithQoS_Update response message (Transaction Reference ID, Result) to the AF. Result indicates whether the request is granted or not. For TSC QoS, the response includes the Assigned TSC Parameter Values if they did not equal the requested values.

6.	The PCF sends Npcf_PolicyAuthorization_Notify message to the NEF when the modification of the transmission resources corresponding to the QoS update succeeded or failed.

7.	The NEF sends Nnef_AFsessionWithQoS_Notify message with the event reported by the PCF to the AF.

Second Change (all new text)

#### 4.15.6.x	TSC Capability Exposure

This procedure exposes 5GS information to aid the AF in formulating a request to establish or modify an AF session with required QoS as described in clauses 4.15.6.6, 4.14.6.6a and Annex G.

Figure 6.5.3-2: TSC Capability exposure towards external AF

1.	The AF requests the 5GS Capability to support Determinists QoS. The AF includes one or more MAC addresses of devices, GPSIs or IP addresses in the Nnef_TSCCapability_Request. It also includes an Application Identifier.

2.	The NEF finds the PCF (e.g: by using the BSF) that serves the PDU session. The NEF sends a Npcf_TSCCapability_Request to the PCF.

NOTE 1:	The NEF sends the Npcf_ TSCCapability_Request for devices where UPF MAC learning has detected the AF requested MAC address as a Source Address in an uplink frame (and hence the device is connected via the associated PDU session), or where the IP address or GPSI is associated with a UE. In both cases the BSF should find a PCF for the UE address/GPSI.

3.	If the UE-DS-TT has reported a residence time to the PCF (see clause 4.16.5.1: SMF initiated SM Policy Association Modification), the PCF is configured with at least one 5QI that supports TSC QoS and TSC QoS is allowed for the Application Identifier, then the PCF provides the UE-DS-TT Residence time to the NEF in the Npcf_ TSCCapability_Request response. Otherwise the PCF response to the NEF may indicate:

-	 “TSC QoS is not supported” if there is no 5QI that supports TSC QoS

-	 “UE delay is unknown” if no Residence Time has been reported by the UE

-	 “TSC QoS not supported for AF-ID” if TSC QoS is not allowed for the AF

4.	For PDU Sessions for which a Residence Time is received from the PCF, the NEF aggregates UE-DS-TT residence time received from the PCF and per QoS configured UE to UPF/NW-TT delay to determine the 5GS delays for TSC.

For each PDU Session, the NEF responds to the AF with TSC QoS Capability, which comprises one of:

- The 5GS Delays for TSC if available (indicating TSC QoS is supported).

- The response from the PCF indicated in step 3.

- An indication that no PDU session could be found for the requested MAC addresses of devices, GPSIs or IP addresses

NOTE 2:	The PDU Session is not found if a MAC address provided by the AF is not known (eg: because it has not been detected by the UPF as a Source Address in an uplink frame), or where the AF provided IP address or GPSI is not recognized.

Third Change

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TSCCapability | Get | Request/Response | AF |

Fourth Change

#### 5.2.6.9	Nnef_AFsessionWithQoS service

##### 5.2.6.9.1	General

See clause 4.15.6.6.

This service is also used to support subscription and notification of QoS Monitoring for URLLC, as described in TS 23.501 [2] clause 5.33.3.2.

##### 5.2.6.9.2	Nnef_AFsessionWithQoS_Create service operation

Service operation name: Nnef_AFsessionWithQoS Create

Description: The consumer requests the network to provide a specific QoS for an AF session.

Inputs, Required: AF Identifier, UE address (i.e. IP address or MAC address), Flow description(s), QoS Reference.

Inputs, Optional: time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting as described in clause 6.1.3.21 of TS 23.503 [20], 5GS delay, TSC Burst Size, a Guaranteed Flow Bit Rate, a Time Domain and Traffic Pattern Parameters.

Outputs, Required: Transaction Reference ID, result.

Output (optional): Assigned TSC Parameter Values.

##### 5.2.6.9.3	Nnef_AFsessionWithQoS_Notify service operation

Service operation name: Nnef_AFsessionWithQoS Notify

Description: NEF reports the QoS Flow level event(s) to the consumer.

Inputs, Required: Reports of the events as defined in clause 6.1.3.18 of TS 23.503 [20].

Inputs, Optional: When the event report is for QoS Monitoring for URLLC, includes Packet delay for UL, DL, or round trip of the single UP path or two UP paths in the case of redundant transmission, as defined in clause 5.33.3.2 of TS 23.501 [2].

Outputs, Required: None.

Output (optional): None.

##### 5.2.6.9.4	Nnef_AFsessionWithQoS_Revoke service operation

Service operation name: Nnef_AFsessionWithQoS Revoke

Description: The consumer requests the network to revoke the AF session with requested QoS or the AF session with requested QoS including Alternative Service Requirements.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, result.

Output (optional): None.

##### 5.2.6.9.5	Nnef_AFsessionWithQoS_Update service operation

Service operation name: Nnef_AFsessionWithQoS Update

Description: The consumer requests the network to update the Service Requirement(s) and/or additional Alternative Service Requirement(s) for an AF session.

Inputs, Required: Transaction Reference ID.

Inputs, Optional: Flow description, QoS reference, time period, traffic volume, Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order), QoS parameter(s) to be measured, Reporting frequency, Target of reporting as described in clause 6.1.3.21 of TS 23.503 [20], 5GS delay, TSC Burst Size, a Guaranteed Flow Bit Rate, a Time Domain and Traffic Pattern Parameters.

Outputs, Required: Result.

Output (optional): Assigned TSC Parameter Values.

Fifth Change (all new text)

#### 5.2.5.x	Nnef_TSC_Capability service

##### 5.2.5.x.1	General

This service is used to learn whether TSC QoS can be supported for an application and UE.  See clause 4.15.6.x.

##### 5.2.5.x.2	Nnef_TSCCapability_Request service operation

Service operation name: Nnef_TSCCapability_Request

Description: The consumer requests the network to provide information on support for TSC QoS for a UE(s) and AF(s).

Inputs, Required: AF Identifier, UE identifier(s) (i.e. IP address, MAC address or GPSI), Transaction Reference ID.

Inputs, Optional: None

Outputs, Required: TSC QoS Capability, Transaction Reference ID

Output (optional): None.

Sixth Change

### 5.2.5	PCF Services

#### 5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Notify | Subscribe/Notify | AF, NEF, NWDAF |
|  | Subscribe |  | AF, NEF, NWDAF |
|  | Unsubscribe |  | AF, NEF, NWDAF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, V-PCF |
|  | Update | Request/Response | AMF, V-PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Npcf_TSCCapability | Get | Request/Response | NEF |

Seventh Change

#### 5.2.5.3	Npcf_PolicyAuthorization Service

##### 5.2.5.3.1	General

Service description: This service is to authorise an AF request and to create policies as requested by the authorized AF for the PDU Session to which the AF session is bound. Additionally this service allows an AF/NEF to exchange Ethernet port management information with DS-TT and NW-TT and to send TSC QoS capability information to an NEF. This service allows the NF consumer to subscribe/unsubscribe the notification of events, which are defined in clause 6.1.3.18 of TS 23.503 [20].

##### 5.2.5.3.2	Npcf_PolicyAuthorization_Create service operation

Service operation name: Npcf_PolicyAuthorization_Create

Description: Authorize the request, and optionally determines and installs SM Policy Control Data according to the information provided by the NF Consumer or provides Ethernet Port Management Information Container for Ethernet ports on DS-TT or NW-TT.

Inputs, Required: UE (IP or MAC) address, identification of the application session context.

Inputs, Optional: UE identity if available, DNN if available, S-NSSAI if available, Media type, Media format, bandwidth requirements, sponsored data connectivity if applicable, flow description, Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, emergency indicator, Application service provider, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition and spatial validity condition as described in clause 5.6.7 in 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 in TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 in TS 23.503 [20], Port Management Information Container and related port number, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.2.23 of TS 23.503 [20], QoS parameter(s) to be measured, Reporting frequency, Target of reporting as described in clause 6.1.3.21 of TS 23.503 [20], Alternative Service Requirements (containing one or more QoS reference parameters in a prioritized order).

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in TS 23.503 [20] clause 6.1.3.16, clause 6.1.3.10).

Outputs, Optional: The service information that can be accepted by the PCF, Assigned TSC Parameter Values.

##### 5.2.5.3.3	Npcf_PolicyAuthorization_Update service operation

Service operation name: Npcf_PolicyAuthorization_Update

Description: Provides updated information to the PCF.

Inputs, Required: Identification of the application session context.

Inputs, Optional: Media type, Media format, bandwidth requirements, sponsored data connectivity if applicable, flow description, Application Identifier, AF Communication Service Identifier, AF Record Identifier, Flow status, Priority indicator, Application service provider, resource allocation outcome, AF Application Event Identifier, a list of DNAI(s) and corresponding routing profile ID(s) or N6 traffic routing information, AF Transaction Id, Early and/or late notifications about UP path management events, temporal validity condition and spatial validity condition as described in clause 5.6.7 in 23.501 [2], Background Data Transfer Reference ID, priority sharing indicator as described in clause 6.1.3.15 in TS 23.503 [20], pre-emption control information as described in clause 6.1.3.15 in TS 23.503 [20], Port Management Information Container and related port number, TSN AF parameters provided by the TSN AF to the PCF as described in clause 6.1.2.23 of TS 23.503 [20], QoS parameter(s) to be measured, Reporting frequency, Target of reporting as described in clause 6.1.3.21 of TS 23.503 [20].

Outputs, Required: Success or Failure (reason for failure, e.g. as defined in TS 23.503 [20] clause 6.1.3.16).

Outputs, Optional: The service information that can be accepted by the PCF, Assigned TSC Parameter Values.

Provides updated application level information and communicates with Npcf_SMPolicyControl service to determine and install the policy according to the information provided by the NF Consumer. Updates an application context in the PCF.

##### 5.2.5.3.4	Npcf_PolicyAuthorization_Delete service operation

Service operation name: Npcf_PolicyAuthorization_Delete

Description: Provides means for the NF Consumer to delete the context of application level session information.

Inputs, Required: Identification of the application session context.

Inputs, Optional: None.

Outputs, Required: None.

Outputs, Optional: None.

##### 5.2.5.3.5	Npcf_PolicyAuthorization_Notify service operation

Service operation name: Npcf_PolicyAuthorization_Notify

Description: provided by the PCF to notify NF consumers of the subscribed events.

Inputs, Required: Event ID.

The events that can be subscribed are defined in clause 6.1.3.18 of TS 23.503 [20].

Inputs, Optional: Event information (defined on a per Event ID basis) are defined in clause 6.1.3.18 of TS 23.503 [20], Notification Correlation Information (information to identify the application session).

Notification Correlation Information is mandatory except in the case of the Manageable Ethernet Port detected event if no AF session exists between the PCF and the AF.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Port Management Information Container, MAC address and related port number, information of the detected bridge, information of the detected port(s) in a bridge.

##### 5.2.5.3.6	Npcf_PolicyAuthorization_Subscribe service operation

Service operation name: Npcf_PolicyAuthorization_Subscribe

Description: provided by the PCF for NF consumers to explicitly subscribe the notification of events.

Inputs, Required: (Set of) Event ID(s) as specified in Npcf_PolicyAuthorization_Notify service operation, target of PCF event reporting (defined below), NF ID, Event Reporting Information defined in Table 4.15.1-1 (only the Event Reporting mode and the inmediate reporting flag when applicable), Notification Target Address (+ Notification Correlation ID).

The target of PCF event reporting the subscription for an individual AF session: An UE IP address (IPv4 address or IPv6 prefix) optionally together with a (DNN, S-NSSAI) or with a UE ID (SUPI or GPSI).

Inputs, Optional: Event Filter, Subscription Correlation ID (in the case of modification of the event subscription).

Outputs, Required: When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

##### 5.2.5.3.7	Npcf_PolicyAuthorization_Unsubscribe service operation

Service operation name: Npcf_PolicyAuthorization_Unsubscribe

Description: Enable NF consumers to explicitly unsubscribe the notification of PCF events related to Npcf_PolicyAuthorization_Subscribe operation.

Inputs, Required: Subscription Correlation.

Inputs, Optional: None.

Outputs, Required: Success or Failure.

Outputs, Optional: None.

Eighth Change

#### 5.2.6.x	Npcf_TSC_Capability service

##### 5.2.6.5.1	General

This service is used to learn whether TSC QoS can be supported for an application and UE.  See clause 4.15.6.x.

##### 5.2.6.x	Npcf_TSCCapability_Request service operation

Service operation name: Npcf_TSCCapability_Request

Description: The consumer requests the PCF to provide information on support for TSC QoS for a UE(s) and AF(s).

Inputs, Required: AF Identifier, UE identifier(s) (i.e. IP address, MAC address or GPSI), Transaction Reference ID.

Inputs, Optional: None

Outputs, Required: TSC QoS Capability, Transaction Reference ID

Output (optional): None.

Ninth Change

Annex G (informative):
Information flows for TSC QoS

This annex defines the procedures for 5GS QoS.

# G.1	AF Request for TSC QoS

This procedure is used by the AF to request TSC QoS, including setup of Hold and Forward Buffers in the UE/DS-TT (for downlink follows) or UPF/NW-TT (for uplink flows) for Ethernet PDU Sessions and configuration of TSCAI.

Figure G.1-1: AF Request for TSC QoS

1.	PDU Session Establishment is performed as specified in clause 4.3.2.2.1. If the AF is located inside the operator's domain, the PCF exposes the UE-DS-TT residence time to the AF as defined in TS 23.502 [3] and the AF is configured with the UE to UPF/NW-TT delay per Type of Service.

2.	If the AF is located outside the operator's domain, the NEF exposes Deterministic QoS Capability to AF when it sends a query to the NEF as shown in clause 4.15.6.x. If the NEF learns the UE-DS-TT residence time from the PCF and the NEF is configured with the UE to UPF/NW-TT delay, the NEF computes 5GS delay per Type of Service.

3.	Setting up an AF session with required QoS procedure specified in clause 4.15.6.6. is executed. If the PDU Session Type is Ethernet, the NEF formulates a Port Management Information Container containing Gate Control Information used to configure Hold and Forward Buffers at the UE/DS-TT or UPF/NW-TT and sends traffic pattern parameters and TSC QoS information to the PCF.

4.	The PCF creates the PCC rule and provides it to the SMF. The PCF also includes the TSC traffic pattern parameters from the NEF/AF and the Port Management Information Container.

5.	The SMF may calculate the TSCAI similar to TS 23.501 [2] clause 5.27.2. The SMF assigns the TSCAI with the Time Domain as received from the PCF to a QoS Flow. If the SMF receives clock drift information from UPF for that Time Domain (i.e. clock drift between 5G timing and AF supplied Time Domain), then SMF may adjust the TSCAI information of the QoS Flow so that it reflects the 5GS Clock.

6.	The SMF configures the U-plane and sends the N2 message with TSCAI to NG-RAN. The SMF sends the Port Management Information Container containing Gate Control Information for Hold and Forward Buffering to the UE or UPF.

Annex H (informative):
Change history

End of CHANGEs