3GPP TSG-SA2 Meeting #143E	S2-200xxxx

# Elbonia, 24 February – 9 March 2021	(Revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2631 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Localization accuracy and 5QI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-02-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Discussion paper titled - Positioning Error impact on 5QI - 3D localization and orientation accuracy affect the data-rate requirements of XR services. In particular, the lower the localization and orientation accuracy is, the larger is the amount of 3D information, which needs to be transmitted to the HMD to compensate localization and orientation errors. - Localization and orientation accuracy for XR services should be defined in terms of maximum positioning error on the X, Y and Z axes and maximum rotation angles around the X, Y and Z axes with respect to the actual position and orientation, respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend Table 5.7.4-1 of TS 23.501 - Section 5.7.4 with one additional column marked as Maximum 6DoF Error for XR related 5QI |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Suboptimal support of 5QI characteristics for XR services |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 5.7.3.8	Maximum 6DoF error

6DoF error can be used to estimate and change QoS parameters of the XR QoS Flow like the guaranteed bitrate amd the dependency between 6DoF error and QoS parameters is bidirectional. Packet Delay Budget (PDB) and Packet Error Rate (PER) affect the 6DoF error. For example, a low PER can limit the maximum 6DoF error since the additional used bandwidth to send the extra-information used to compensate the 6DoF error might not be guaranteed in certain radio and network conditions. Thus, the maximum 6DoF error is defined as a new QoS characteristic for interactive XR and gaming services.

The 6DoF error can be represented considering maximum horizontal and/or vertical distances. Typical units of measure for the 6DoF error are meters for positioning errors and degrees or radians for rotation angles.

### 5.7.4	Standardized 5QI to QoS characteristics mapping

Standardized 5QI values are specified for services that are assumed to be frequently used and thus benefit from optimized signalling by using standardized QoS characteristics. Dynamically assigned 5QI values (which require a signalling of QoS characteristics as part of the QoS profile) can be used for services for which standardized 5QI values are not defined. The one-to-one mapping of standardized 5QI values to 5G QoS characteristics is specified in table 5.7.4-1.

Table 5.7.4-1: Standardized 5QI to QoS characteristics mapping

| 5QI Value | Resource Type |  | Default Priority Level | Packet Delay Budget (NOTE 3) | Packet Error Rate | Default Maximum Data Burst Volume (NOTE 2) | Default Averaging Window | Maximum 6DoF error | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR |  | 20 | 100 ms (NOTE 11, NOTE 13) | 10-2 | N/A | 2000 ms |  | Conversational Voice |
| 2 | (NOTE 1) |  | 40 | 150 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms |  | Conversational Video (Live Streaming) |
| 3 |  |  | 30 | 50 ms (NOTE 11, NOTE 13) | 10-3 | N/A | 2000 ms |  | Real Time Gaming, V2X messages (see TS 23.287 [121]). Electricity distribution – medium voltage, Process automation monitoring |
| 4 |  |  | 50 | 300 ms (NOTE 11, NOTE 13) | 10-6 | N/A | 2000 ms |  | Non-Conversational Video (Buffered Streaming) |
| 65 (NOTE 9, NOTE 12) |  |  | 7 | 75 ms (NOTE 7, NOTE 8) | 10-2 | N/A | 2000 ms |  | Mission Critical user plane Push To Talk voice (e.g., MCPTT) |
| 66 (NOTE 12) |  |  | 20 | 100 ms (NOTE 10, NOTE 13) | 10-2 | N/A | 2000 ms |  | Non-Mission-Critical user plane Push To Talk voice |
| 67 (NOTE 12) |  |  | 15 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | 2000 ms |  | Mission Critical Video user plane |
| 75 (NOTE 14) |  |  |  |  |  |  |  |  |  |
| 71 |  |  | 56 | 150 ms (NOTE 11, NOTE 13, NOTE 15) | 10-6 | N/A | 2000 ms |  | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 72 |  |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms |  | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 73 |  |  | 56 | 300 ms (NOTE 11, NOTE 13, NOTE 15) | 10-8 | N/A | 2000 ms |  | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 74 |  |  | 56 | 500 ms (NOTE 11, NOTE 15) | 10-8 | N/A | 2000 ms |  | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 76 |  |  | 56 | 500 ms (NOTE 11, NOTE 13, NOTE 15) | 10-4 | N/A | 2000 ms |  | "Live" Uplink Streaming (e.g. TS 26.238 [76]) |
| 5 | Non-GBR |  | 10 | 100 ms NOTE 10, NOTE 13) | 10-6 | N/A | N/A |  | IMS Signalling |
| 6 | (NOTE 1) |  | 60 | 300 ms (NOTE 10, NOTE 13) | 10-6 | N/A | N/A |  | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) |
| 7 |  |  | 70 | 100 ms (NOTE 10, NOTE 13) | 10-3 | N/A | N/A |  | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  |  | 80 | 300 ms (NOTE 13) | 10-6 | N/A | N/A |  | Video (Buffered Streaming) TCP-based (e.g., www, e-mail, chat, ftp, p2p file sharing, progressive |
| 9 |  |  | 90 |  |  |  |  |  | video, etc.) |
| 69 (NOTE 9, NOTE 12) |  |  | 5 | 60 ms (NOTE 7, NOTE 8) | 10-6 | N/A | N/A |  | Mission Critical delay sensitive signalling (e.g., MC-PTT signalling) |
| 70 (NOTE 12) |  |  | 55 | 200 ms (NOTE 7, NOTE 10) | 10-6 | N/A | N/A |  | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 79 |  |  | 65 | 50 ms (NOTE 10, NOTE 13) | 10-2 | N/A | N/A |  | V2X messages (see TS 23.287 [121]) |
| 80 |  |  | 68 | 10 ms (NOTE 5, NOTE 10) | 10-6 | N/A | N/A | (0, 0, 0) (0, 0, 0) | Low Latency eMBB applications Augmented Reality |
| 82 | Delay-critical GBR |  | 19 | 10 ms (NOTE 4) | 10-4 | 255 bytes | 2000 ms |  | Discrete Automation (see TS 22.261 [2]) |
| 83 |  |  | 22 | 10 ms (NOTE 4) | 10-4 | 1354 bytes (NOTE 3) | 2000 ms |  | Discrete Automation (see TS 22.261 [2]); V2X messages (UE - RSU Platooning, Advanced Driving: Cooperative Lane Change with low LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 84 |  |  | 24 | 30 ms (NOTE 6) | 10-5 | 1354 bytes (NOTE 3) | 2000 ms |  | Intelligent transport systems (see TS 22.261 [2]) |
| 85 |  |  | 21 | 5 ms (NOTE 5) | 10-5 | 255 bytes | 2000 ms |  | Electricity Distribution- high voltage (see TS 22.261 [2]). V2X messages (Remote Driving. See TS 22.186 [111], NOTE 16, see TS 23.287 [121]) |
| 86 |  |  | 18 | 5 ms (NOTE 5) | 10-4 | 1354 bytes | 2000 ms |  | V2X messages (Advanced Driving: Collision Avoidance, Platooning with high LoA. See TS 22.186 [111], TS 23.287 [121]) |
| 100 |  |  | 68 | 10 ms (NOTE 5, NOTE 10) | 10-6 | N/A | N/A | (0.1 m, 0.1 m, 0.1 m) (5°, 5°, 5°) | Low Latency eMBB applications Augmented Reality with Error |
|  |  | NOTE 1: A packet which is delayed more than PDB is not counted as lost, thus not included in the PER. NOTE 2: It is required that default MDBV is supported by a PLMN supporting the related 5QIs. NOTE 3: The Maximum Transfer Unit (MTU) size considerations in clause 9.3 and Annex C of TS 23.060 [56] are also applicable. IP fragmentation may have impacts to CN PDB, and details are provided in clause 5.6.10. NOTE 4: A static value for the CN PDB of 1 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 5: A static value for the CN PDB of 2 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 6: A static value for the CN PDB of 5 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. When a dynamic CN PDB is used, see clause 5.7.3.4. NOTE 7: For Mission Critical services, it may be assumed that the UPF terminating N6 is located "close" to the 5G_AN (roughly 10 ms) and is not normally used in a long distance, home routed roaming situation. Hence a static value for the CN PDBof 10 ms for the delay between a UPF terminating N6 and a 5G_AN should be subtracted from this PDB to derive the packet delay budget that applies to the radio interface. NOTE 8: In both RRC Idle and RRC Connected mode, the PDB requirement for these 5QIs can be relaxed (but not to a value greater than 320 ms) for the first packet(s) in a downlink data or signalling burst in order to permit reasonable battery saving (DRX) techniques. NOTE 9: It is expected that 5QI-65 and 5QI-69 are used together to provide Mission Critical Push to Talk service (e.g., 5QI-5 is not used for signalling). It is expected that the amount of traffic per UE will be similar or less compared to the IMS signalling. NOTE 10: In both RRC Idle and RRC Connected mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 11: In RRC Idle mode, the PDB requirement for these 5QIs can be relaxed for the first packet(s) in a downlink data or signalling burst in order to permit battery saving (DRX) techniques. NOTE 12: This 5QI value can only be assigned upon request from the network side. The UE and any application running on the UE is not allowed to request this 5QI value. NOTE 13: A static value for the CN PDB of 20 ms for the delay between a UPF terminating N6 and a 5G-AN should be subtracted from a given PDB to derive the packet delay budget that applies to the radio interface. NOTE 14: This 5QI is not supported in this Release of the specification as it is only used for transmission of V2X messages over MBMS bearers as defined in TS 23.285 [72] but the value is reserved for future use. NOTE 15: For "live" uplink streaming (see TS 26.238 [76]), guidelines for PDB values of the different 5QIs correspond to the latency configurations defined in TR 26.939 [77]. In order to support higher latency reliable streaming services (above 500ms PDB), if different PDB and PER combinations are needed these configurations will have to use non-standardised 5QIs. NOTE 16: These services are expected to need much larger MDBV values to be signalled to the RAN. Support for such larger MDBV values with low latency and high reliability is likely to require a suitable RAN configuration, for which, the simulation scenarios in TR 38.824 [112] may contain some guidance. |  |  |  |  |  |  |  |

NOTE:	It is preferred that a value less than 64 is allocated for any new standardised 5QI of Non-GBR resource type. This is to allow for option 1 to be used as described in clause 5.7.1.3 (as the QFI is limited to less than 64).