3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100674

Elbonia, Feb 24 – Mar 09, 2021	(revision of S2-200xxxx)

Source:	CATT

Title:	Discussion on UE positioning capabilities storage in core network

Document for:	Discussion

Agenda Item:	8.14

Work Item / Release:	5G_eLCS_Ph2 / Rel-17

Abstract: This contribution analyses the UE positioning capabilities storage in core network.

- Introduction

The work tasks of FS_eLCS_ph2 SID in SP-191370 are shown in Table 1.

Table 1: Work Tasks for FS_eLCS_ph2

| Work Task ID | Work Task Title | Work Task Description | RAN Dependency |
| --- | --- | --- | --- |
| WT#1.1 | High accuracy and Low latency | Investigate solution to support SA1 requirements e.g. very low latency and very high accuracy positioning, including horizontal and vertical positioning service levels, 5G positioning service area; | Yes |
| WT#2.1 | IIoT and etc | Investigate necessary system function/procedure modifications to satisfy specific new use cases identified by RAN Rel-17 NR positioning study and potential MCX UE positioning requirement which impacts existing 5G_eLCS defined in TS23.273. | Yes |

To support the low latency in WT#1.1 in Table 1, one method discussed by RAN2 is to store the UE positioning capabilities in core network. Because the method is relevant to SA2, this paper is proposed to analyse the benefit and feasibility of the method from SA2 point of view.

- Discussion

2.1	Latency analyses

Based on analyses in R2-2102095, the latency of UE Assisted and UE Based Positioning Procedure is shown in Figure 1.

Figure 1: Latency analyses of UE Assisted and UE Based Positioning Procedure

The method discussed in RAN2 is to store the UE positioning capabilities in core network to avoid the LMF to retrieve the capabilities from UE, i.e. the steps 1 – 2 in Figure 1 are skipped.

As shown in Figure 1, the latency of steps 1 – 2 is 43 – 89ms, so from this point of view, the benefit of the method is to reduce 43 – 89ms latency of the positioning procedure.

Proposal 1: The positioning latency is reduced by storing positioning capabilities in core network before the positioning request is received by network.

2.2	UE positioning capabilities analyses

The UE positioning capabilities described in TS 37.355 is summarized in Table 1.

Table 1: Summary of UE Positioning Capabilities

| Positioning capability |  | Functionality | Positioning method | Static or dynamic |
| --- | --- | --- | --- | --- |
| NR-UL-SRS-Capability |  | Specifies UE uplink SRS capability | UL-TDOA/UL-AOA/Multi-RTT | Dynamic |
| Positioning Modes related capability | agnss-Modes | Indicates Positioning mode supported by the target service | GNSS/OTOOA/Sensor/TBS/WLAN/BT/NR-DL-AOD/NR-DL-TDOA | Static |
|  | otdoa-Mode |  |  |  |
|  | sensor-Modes |  |  |  |
|  | tbs-Modes |  |  |  |
|  | wlan-Modes |  |  |  |
|  | bt-Modes |  |  |  |
|  | nr-DL-AoD-Mode |  |  |  |
|  | nr-DL-TDOA-Mode |  |  |  |
| NR-DL-PRS-ResourcesCapability |  | Specifies DL-PRS resources capability for each positioning method | DL-TDOA/DL-AOD/Multi-RTT |  |
| NR-DL-PRS-ProcessingCapability |  | Specifies DL-PRS Processing capability | DL-TDOA/DL-AOD/Multi-RTT |  |
| NR-DL-PRS-QCL-ProcessingCapability |  | Specifies UE DL-PRS QCL Processing capability | DL-TDOA/DL-AOD/Multi-RTT |  |
| nr-DL-TDOA-MeasurementCapability |  | Specifies DL-TDOA measurement capability | DL-TDOA |  |
| additionalPathsReport |  | Indicates whether the UE supports additional paths reporting | DL-TDOA/Multi-RTT/OTDOA |  |
| periodicalReporting |  | Indicates which positioning mode UE supports periodical Reporting | NR ECID/DL-AoD/Multi-RTT/DL-TDOA/ECID/OTDOA |  |
| NR-DL-AoD-MeasurementCapability |  | Specifies DL-AoD measurement capability | DL-AOD |  |
| NR-Multi-RTT-MeasurementCapability |  | Specifies Multi-RTT measurement capability | Multi-RTT |  |
| triggeredReporting |  | Indicates whether the UE supports triggered Reporting | NR ECID/ECID |  |
| nr-ECID-MeasSupported |  | Indicates the supported NR ECID measurements | NR E-CID |  |
| bt-MeasSupported |  | Specifies the Bluetooth measurements supported by the target device | BT |  |
| idleStateForMeasurements |  | Indicates the target device requires idle state to perform RSTD measurements. | BT/WLAN/Sensor/ECID/OTDOA/GNSS |  |
| wlan-MeasSupported |  | Specifies the measurements supported by the target device when accessing a WLAN | WLAN |  |
| mbs-IdleStateForMeasurements |  | Indicates that the target device requires idle state to perform MBS measurements. | TBS |  |
| periodicalReportingSupported |  | Indicates which positioning mode the UE supports periodical Reporting | TBS/Sensor |  |
| periodicalReportingNotSupported |  | Specifies the positioning modes for which the target device does not support periodicalReporting | GNSS |  |
| mbs-AssistanceDataSupportList |  | Indicate the capability to support MBS Assistance Data and to provide capability to the location server | TBS |  |
| mbs-ConfigSupport |  | Specifies the MBS configurations supported by the target device | TBS |  |
| sensor-AssistanceDataSupportList |  | Specifies a list of sensor assistance data supported by the target device | Sensor |  |
| sensor-MotionInformationSup |  | Indicates that the target device supports displacement reporting | Sensor |  |
| ecid-MeasSupported |  | Specifies the E CID measurements supported by the target device | E-CID |  |
| additional-prs-config |  | Indicates that the target device supports additional PRS configurations | OTDOA |  |
| prs-based-tbs |  | Indicates that the target device supports RSTD measurements for PRS-only TPs. | OTDOA |  |
| maxSupportedPrsBandwidth |  | Indicates the maximum PRS bandwidth supported by the target device | OTDOA |  |
| densePrsConfig |  | Indicates that the target device supports a subset of the additional PRS configurations associated with capability additional-prs-config | OTDOA |  |
| prsOccGroup |  | Indicates that the target device supports PRS occasion groups | OTDOA |  |
| prsFrequencyHopping |  | Indicates that the target device supports PRS occasion frequency hopping | OTDOA |  |
| maxSupportedPrsConfigs |  | Indicates that the target device supports multiple PRS configurations per cell | OTDOA |  |
| multiPrbNprs |  | Indicates that the target device supports NPRS configuration in more than one resource block | OTDOA |  |
| motionMeasurements |  | Indicates that the target device supports reporting of motion measurements | OTDOA |  |
| interRAT-RSTDmeasurement |  | Indicates that the target device supports inter-RAT RSTD measurements | OTDOA |  |
| interFreqRSTDmeasurement |  | Indicates that the target device supports inter-frequency RSTD measurements within and between the frequency bands indicated in SupportedBandEUTRA | OTDOA |  |

Based on the summary in Table 1, most of the UE positioning capabilities are static, except the NR-UL-SRS-Capability which may change but not very frequently.

For the static capabilities, it is valuable to store them in core network to reduce positioning latency.

For the dynamic capability (i.e. NR-UL-SRS-Capabilities), because it is not changed very frequently, storing the capability in core network will not introduce more latency in NAS layer. Furthermore, based on description in clause 6.3.3 in TS 38.331, the SRS capabilities (i.e. OLPC-SRS-Pos and SpatialRelationsSRS-Pos) are already stored in AMF, so it is feasible to store the capability in core network.

Proposal 2: It is feasible to store the UE positioning capabilities in core network.

2.3	NF in core network to store the UE positioning capabilities

To store UE positioning capabilities in core network, the candidate NFs in core network include AMF and LMF. Table 2 illustrates 5GS system impacts for these candidates.

Table 2: Impacts of storing UE positioning capabilities in different 5GC NF

|  | UE impact | RAN impact | 5GC impact |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LMF selection | UE context stored in AMF | UE mobility support | positioning capabilities delivery in 5GC |
| Storing UE positioning capabilities in AMF | UE sends UE positioning capabilities to AMF, e.g. via NAS or via NG-RAN | Transfer UE positioning capabilities to AMF | The existing LMF selection mechanism is not impacted | A new item is added, i.e. UE positioning capabilities | The existing mobility mechanism is not impacted. | AMF sends UE positioning capabilities to LMF along with positioning request received from GMLC. |
| Storing UE positioning capabilities in LMF | Same | Same | New trigger(s) of LMF selection needs to be defined, e.g. when UE positioning capabilities is received by AMF, UE mobility events. | A new item is added, i.e. the serving LMF | Not supported. Several questions needs to be considered, e.g. AMF should decide whether LMF is changed during UE mobility procedure, target LMF selection, UE positioning capabilities transmission between source and target LMFs, how to release UE positioning capabilities in source LMF. | AMF sends UE positioning capabilities to LMF when received from UE. |

Based on the impacts described in Table 2, storing UE positioning capabilities in AMF has less impact to 5G system than in LMF. So it is proposed that the UE positioning capabilities is stored in AMF. The UE sends the UE positioning capabilities to AMF via NAS or via NG-RAN, the AMF stores the capabilities. When positioning request is received by AMF, AMF sends the request along with the UE positioning capabilities to LMF.

Proposal 3: The UE positioning capabilities is stored in AMF.

3. Proposals for way forwards

Proposal 1: The positioning latency is reduced by storing positioning capabilities in core network before the positioning request is received by network.

Proposal 2: It is feasible to store the UE positioning capabilities in core network.

Proposal 3: The UE positioning capabilities is stored in AMF.