3GPP TSG-SA WG2 Meeting #143-e	S2-2100822

# E-Meeting, 24th February – 09th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2652 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | O-PLMN selection and registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on DP in S2-2100821, the issue is left to be resolved. - UE with default PLMN credentials unable to find a suitable cell of an O-PLMN to camp on The DP in S2-2100821 proposes to resolve the above issue |  |  |  |  |  |  |  |  |
| Summary of change: |  | - A UE with default PLMN credentials unable to find a suitable cell of an O-PLMN to camp on may camp on an acceptable cell of any available and allowable O-PLMN and attempt an onboarding registration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE onboarding is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.X.2.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

##### 5.30.X.2.7	Network selection and Initial Registration when ON is a PLMN

This clause applies only when the UE is not in SNPN access mode.

When the UE is using PLMN credentials for accessing the Onboarding Network (ON), then regular network selection, as per TS 23.122 [17] and regular initial registration procedures apply, as per TS 23.502 [3].

If a UE with default PLMN credentials is unable to find a suitable cell of an ON to camp on, the UE may camp on an acceptable cell of any available and allowable ON (i.e. which broadcasts onboarding enabled indication) and attempt an onboarding registration.

NOTE X:	In this scenario it is assumed that the Onboarding Network is a PLMN.

***** End of changes *****