3GPP TSG-SA2 # 143E (e-meeting)	S2-2100883

# Febuarary.24 – March 1, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 245 | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reselection of NWDAF due to mobility change |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of TR23700-91 of FS_eNA_Ph2, “ KI#2, Specific aspects of Re-selection of NWDAF, it is conclude that: “d) Support the transfer of analytics context: - NF consumer was not changed, i.e. NWDAF re-selection was initiated by the source NWDAF: the target NWDAF initiates the collection of information related to the subscriptions handed over.”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR: [Change-1] Clause 6.2y Add a procedure to TS 23.288, which describes the scenario that the serving NWDAF of one consumer NF may need to change due to UE mobility event without Consumer NF change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | R17 TS 23.288 does not involve the conclusion for KI#2: Specific aspects of NWDAF interactions of FS_eNA_Ph2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2y.x (all new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

6.2y.x Reselection of NWDAF due to mobility change

6.2y.x.1	Description

If there are multiple NWDAFs deployed in the network, there may be some cases that serving NWDAF of one consumer NF may need to change due to UE mobility event. Upon the change of UE location,  AMF is required to inform the serving NWDAF which has subscribed the UE mobility event notification from AMF, and the serving NWDAF decides whether to continue analysis service or start NWDAF re-selection. An analytics context relocation is indicated from the source NWDAF to the target NWDAF. It is also required to inform the related NWDAF consumer NF the change of NWDAF  to update subscriptions between the NWDAF and the consumer NF.

6.2y.x.2	Procedures

6.2y.x.2.1	Re-selection of NWDAF without AMF change

Figure 6.57.2.1-1: Procedure for re-selection of NWDAF without AMF change

1.	Consumer NF sends an analytics subscription to the NWDAF1 by invoking an Nnwdaf_AnalyticsSubscription_Subscribe service; the NWDAF1 subscribes to fetch the network data or management data from the NF(s) or the OAM and the NF(s) or the OAM deliver the network data or management data, as defined in TS 23.288 [5].

2.	The NWDAF1 subscribes the UE mobility event notification from AMF by invoking an Namf_EventExposure_Subscribe service.

3.	UE mobility event is detected by AMF and the AMF notifies the UE mobility event to the NWDAF1 by invoking the Namf_EventExposure_Notify service if the location (e.g. TAI, or cell list) of the UE changes. NWDAF1 determines if the UE location is in the serving area of NWDAF1. If yes, the NWDAF1 could continue the analytics service, otherwise, the NWDAF1 performs step 4-6 to trigger the re-selection of NWDAF.

4.	The NWDAF1 interacts with NRF by sending Nnrf_NFDiscovery_Request to discover an appropriate NWDAF2 for the UE.

5a.	[OPTIONAL] The NWDAF1 invokes the Nnwdaf_AnalyticsRelocation_Request or enhanced Nnwdaf_AnalyticsInfo_Request service to the NWDAF2 to trigger relocation and negotiate the elements need to  be transferred. The NWDAF2 responds to the NWDAF1 with a Nnwdaf_AnalyticsRelocation_Response or Nnwdaf_AnalyticsInfo_Response service operation which includes information on the subscription selected by the NWDAF2.

5b.	The NWDAF1 invokes the Nnwdaf_AnalyticsRelocation_Request or enhanced Nnwdaf_AnalyticsInfo_Request service to the NWDAF2 to transfer subscribed information in analytics context consisting of Subscription ID, Analytics ID, Target of Reporting and Event Reporting Information. The NWDAF2 responds to the NWDAF1 with a Nnwdaf_AnalyticsRelocation_Response or Nnwdaf_AnalyticsInfo Response service operation which confirms the selected subscribed information has been received by the NWDAF2.

6.	The NWDAF1 sends Nnf_EventExposure_Unsubscribe to the NF(s) or the OAM to unsubscribe the fetching the network data.

7.	The NWDAF2 invokes the Nnwdaf_AnalyticsSubscription_Notify service to the consumer NF to notify the release of the subscriptions of NWDAF1 and take over the analytics subscription for the consumer NF.

8.	The NWDAF2 invokes the Nnf_EventExposure_Subscribe service to the source NF(s) or OAM to subscribe the data in the source NF(s) or OAM.

6.2y.x.2.2	Re-selection of NWDAF with AMF change

Figure 6.57.2.2-1: Procedure for re-selection of NWDAF with AMF change

1~2.	Steps 1~2 are executed as defined in clause 6.57.2.1.

3.	The AMF2 invokes the Namf_Communication_UEContextTransfer service operation to the AMF1 to fetch the UE context which includes the subscription of the UE mobility event from AMF1.

4~9.	Steps 3~8 are executed as defined in clause 6.57.2.1.

* * *End of Changes * * *