3GPP TSG-WG SA2 Meeting #143E e-meeting  	S2-2100886

# Elbonia, February 24 - March 09, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 247 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for ML Model registration, discovery and provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the TR 23.700-91, it is concluded for Key Issue #1 that the NWDAF could be decomposed into two logical functions i.e. Model Training logical function and Analytics logical function. Also, Model Training logical function may register the supported ML model information to NRF. Based on the conclusion above, TS 23.288 needs to be updated correspondingly to define the procedures for ML Model registration, discovery and provisioning.. |
| Summary of change: |  | Define the procedures for ML Model registration, discovery and provisioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack the procedures for ML Model registration, discovery and provisioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2x (new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (All new text)

6.2x	Procedure for ML Model registration, discovery and provisioning

Figure 6. 2x-1: Procedure for ML Model registration, discovery and provisioning

1a-1b.	Training service provided by Training function in NWDAF 1 or NWDAF 2 sends registration request by invoking Nnrf_NFManagement_NFRegister to register its own service in the NRF, including some parameters, e.g. IP address, location, load, algorithm information (Algorithm ID, supported accuracy, supported convergence time).

2.	The data analytics for an Analytics ID in Inference Function in NWDAF 3 is triggered by e.g. operator policies.

3.	Inference Function in NWDAF 3 invokes Nnrf_NFDiscovery_Request service operation using parameters such as algorithm information (Algorithm ID, supported accuracy, supported convergence time).

4.	NRF matches the requested query and sends this information via Nnrf_NFDiscovery_Request response service to Inference Function in NWDAF 3 including one or more Training services provided by the Training Function in NWDAF 1 or NWDAF 2.

5.	Inference Function in NWDAF 3 selects Training Function in NWDAF 1 based on the output of NRF.

6.	Inference Function in NWDAF 3 sends request including some parameters, e.g. Analytics ID for a machine learning model to invoke the Training service provided by the Training Function in NWDAF 1 for machine learning model generation.

7.	Training Function in NWDAF 1 performs data collection and machine learning model training.

8.	Inference Function in NWDAF 3 receives the response from the Training Function in NWDAF 1including machine learning model response and some parameters, e.g. machine learning model identification, input parameter and output parameter.

9.	Inference Function in NWDAF 3 completes the machine learning model deployment. In this step, the Inference Function in NWDAF 3 is prepared to use the received trained model to perform inference.

10.	Infernce Function in NWDAF 3 registers the Analytics ID and address to NRF by using the current NF service registration mechanism as defined in clause 4.17.1, TS 23.502 [3].

11.	Consumer NF discovers the Inference Function in NWDAF 3 by using the current NWDAF discovery and selection mechanism as defined in clause 6.3.13, TS 23.501 [2].

12a.	The Consumer NF invokes the Nnwdaf_AnalyticsSubscripion_Subscribe or Nnwdaf_AnalyticsInfo_Request service provided by Inference Function in NWDAF 3 for analytics.

12b.	Inference Function in NWDAF 3 sends analytics response to the requested NF.

END OF CHANGES