SA WG2 Meeting #143E (e-meeting)	S2-2100963

Elbonia, Feb 24 – Mar 9, 2021

Source:	Qualcomm Incorporated

Title:	TS 23.304 clause 5.10: Support for ProSe for UEs in limited service state

Document for:	Discussion/Approval

Agenda Item:	8.8.2

Work Item / Release:	5G_ProSe/Rel-17

Abstract of the contribution: This contribution adds the description of Support for ProSe for UEs in limited service state to TS 23.304 as clause 5.10.

# Discussion

According to the work plan, this contribution adds the description of Support for ProSe for UEs in limited service state to TS 23.304 as clause 5.10

# Text Proposal

It is proposed to add the following to TS 23.304.

>>>>Start Changes<<<<

## 5.10	Support for ProSe for UEs in limited service state

For UE in limited service state, as defined in TS 23.122 [13], ProSe can be used over PC5 reference point with the following considerations.

UEs that are authorized to use ProSe over PC5 reference point according to clause 5.1 shall be able to use the corresponding services following the principles defined in clause 5.1.2.2 for ProSe Direct Discovery, clause 5.1.3.2 for ProSe Direct Communication, clause 5.1.4.2 for ProSe UE-to-Network Relay, and clause 5.1.5.2 for ProSe UE-to-UE Relay, when the UE enters in limited service state in 5GS:

-	because UE cannot find a suitable cell of the selected PLMN as described in TS 23.122 [13]; or

-	as the result of receiving one of the following reject reasons defined in TS 23.122 [13]:

-	a "PLMN not allowed" response to a registration request or;

-	a "5GS services not allowed" response to a registration request or service request.

A UE in limited service state shall only use the radio resources and procedure available in CM-IDLE mode for ProSe over PC5 reference point, for details see TS 36.300 [x] and TS 38.300 [y].

UEs shall not use ProSe over PC5 reference point using the "operator-managed" radio resources, as specified in clause 5.1.2.1, 5.1.3.1, 5.1.4.1, and 5.1.5.1, if the UE has entered in limited service state due to all other situations (e.g. no SIM in the MS, an "illegal MS" or "illegal ME" response to a registration request, or an "IMSI unknown in HLR" response to a registration request) defined in TS 23.122 [13], where the UE is unable to obtain normal service from a PLMN. The UEs may use ProSe over PC5 reference point using the "non-operator-managed" radio resources, as specified in clause 5.1.2.1, 5.1.3.1, 5.1.4.1, and 5.1.5.1, according to the corresponding principles defined in clause 5.1.2.2, 5.1.3.2, 5.1.4.2, and 5.1.5.2.

>>>>End Changes<<<<