3GPP TSG-SA WG2 Meeting #143E e-meeting	S2-2100975

# Elbonia, February 24 – March 09, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2590 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.502 KI#1 Network Slice Admission Control Function (NSACF) services and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Apple, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Further to: - the eNS-Ph2 conclusions in the TR23.700-40 1.2.0, clause 8.1 for KI#1; and - a decision from SA2#143E meeting preparation telco on eNS-Ph2 to submit CRs based on the so-called push method. this contribution proposes the new feature addition to TS23.502. The changes are based on TR solution ‘6.2 Solution #2: Max number of UEs per Network Slice control at registration’ which is fully compatible with the conclusions in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New eNS_Ph2 feature addition for KI#1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNS_Ph2 KI#1 conclusions are not implemented in TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new), 5.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 ... CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

### 4.2.x	Network Slice Admission Control Function (NSACF) procedures

The NSACF is configured by the O&M with the maximum number of UEs that is allowed to be served by the network slice that is subject to NSAC. The NSACF controlls the number of the UEs registered per network slice so that the maximum number of UEs per network slice is not exceeded.

#### 4.2.x.1	Number of UEs per network slice availability check procedure

The number of UEs per network slice availability check procedure is triggered by the AMF to check whether the maximum number of UEs registered with a network slice subject to NSAC has already been reached.  The AMF is configured with the information indicating which network slice is subject to NSAC.

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceAvailabilityCheck_Request

4. Nnsacf_NumberOfUEsPerSliceAvailabilityCheck_Response

3. The number of UEs per network slice availability check.

1. A trigger to perform number of UEs per network slice availability check during UE registration

5. Continue with the UE registration procedure

Figure 4.2.x.1-1: Number of UEs per network slice availability check procedure

- During the UE registration procedure as per clause 4.2.2.2.2, after the UE was successfully authenticated and authorised, after step 12 the AMF triggers a request to the NSACF to perform NSAC for each network slice the UE is not registered yet but for which the UE requires registration and which is subject to NSAC.

- The AMF sends Nnsacf_NumberOfUEsPerSliceAvailabilityCheck_Request message to the NSACF. The AMF includes in the message the UE ID and the S-NSSAI(s) which are subject to NSAC.

- The NSACF checks for each S-NSSAI whether the number of UEs registered with that network slice has already reached the maximum number of UEs per network slice threshold. The NSACF maintains a list of registered UE IDs per network slice for each S-NSSAI that is subject to NSAC. The NSACF checks for each S-NSSAI if the UE ID is already in the list of UEs registered with that S-NSSAI. If the UE ID is in the list of UEs registered with that S-NSSAI, then there is no check if the maximum number of registered UEs has been reached as the UE is already counted as registered with that S-NSSAI. If the UE_ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns maximum number of UEs per network slice reached result. Otherwise, The NSACF returns maximum number of UEs per network slice not reached result.

- The NSACF returns the Nnsacf_NumberOfUEsPerSliceAvailabilityCheck_Response in which the NSACF includes the S-NSSAI(s) for which the number of UEs per network slice availability check was done along with a result parameter per S-NSSAI which indicates whether the maximum number of UEs per network slice has already been reached or not reached.

- The AMF continues with the UE registration procedure. If all the S-NSSAI(s) that the UE requested to register for were subject to NSAC and for all of them the NSACF returned the maximum number of UEs per network slice has been reached, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list, a reject cause set to 'maximum number of UEs per network slice reached' for each S-NSSAI and optionally a back-off timer.

Otherwise, the AMF continues with the registration procedure and in the Registration Accept message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached, a reject cause set to 'maximum number of UEs per network slice reached' for each rejected S-NSSAI and optionally a back-off timer.

#### 4.2.x.2	Number of UEs per network slice update procedure

The number of UEs per network slice update procedure is to update (i.e. increase or decrease) the number of UEs registered with a network slice which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

NSACF

AMF

2. Nnsacf_NumberOfUEsPerSliceUpdate_Request

4. Nnsacf_NumberOfUEsPerSliceUpdate_Response

3. The number of UEs per network slice update.

1. A trigger to perform number of UEs per network slice update

Figure 4.2.x.2-1: Number of UEs per network slice update procedure

- The AMF triggers the Number of UEs per network slice update procedure to update the number of UEs registered with a network slice for the newly registered networks slices that are subject to NSAC as a last step of:

- UE Registration procedure, as per clause 4.2.2.2.2;

- UE Deregistration procedure, as per clause 4.2.2.3;

- Network Slice-Specific Authentication and Authorisation, as per clause 4.2.9.2;

- AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure, as per clause 4.2.9.3; or

- AAA Server triggered Slice-Specific Authorization Revocation, as per clause 4.2.9.4.

- The AMF sends Nnsacf_NumberOfUEsPerSliceUpdate_Request message to the NSACF. The AMF includes in the message the UE ID, the S-NSSAI(s) for which the number of UEs registered per network slice update is required and the number of UEs update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased (after UE registration and NSSAA) or the number of UEs registered with the S-NSSAI(s) is to be decreased (after UE deregistration or S-NSSAI(s) status change from allowed to rejected after AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure (clause 4.2.9.3) or AAA Server triggered Slice-Specific Authorization Revocation (clause 4.2.9.4)).

- The NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs per network slice based on the information provided by the AMF in the number of UEs update flag parameter.

If the number of UEs update flag parameter from the AMF indicates increase the current number of UEs per network slice, the NSACF checks if the UE ID is already in the list of UEs registered with the network slice and if not in the list, the NSACF adds the UE ID to the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the NSACF increases the number of UEs per network slice that is maintained by the NSACF for each of these network slices.

If the number of UEs update flag parameter from the AMF indicates decrease the current number of UEs per network slice, the NSACF removes the UE ID from the list of UEs registered with a network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the NSACF decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices.

- The NSACF acknowledges the number of UEs per network slice update for each S-NSSAI(s) to the AMF.

* * * * end of 1st change * * * *

* * * * start of 2nd change * * * *

### 5.2.xx	Network Slice Admission Control Function (NSACF) services

#### 5.2.xx.1	General

The following table illustrates the NSACF services.

Table 5.2.xx.1-1: List of NSQCF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NumberOfUEsPerSlice | AvailabilityCheck | Request/Response | AMF |
|  | Update | Request/Response | AMF |

#### 5.2.xx.2	Nnsacf_NumberOfUEsPerSlice services

##### 5.2.xx.2.1	General

Service Description: The Nnsacf_NumberOfUEsPerSlice services control the number of UEs registered with a network slice for the network slices subject to NSAC. The AMF can request the NSACF to check whether the number of UEs registered with a network slice has reached the maximum number of UEs per network slice and the AMF can also request the NSACF to update the number of UEs registered with a network slice.

##### 5.2.xx.2.2	Nnsacf_NumberOfUEsPerSliceAvailabilityCheck service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceAvailabilityCheck

Description: Checks whether the number of UEs registered with a network slice has reached the maximum number of UEs per network slice.

Inputs, Required: S-NSSAI(s), UE ID

The S-NSSAI(s) input parameter is a list of one or more network slices for which the NSACF checks whether the number of UEs registered with a network slice has reached the maximum number of UEs per network slice.

The UE ID is used by the NSACF to check whether the UE ID is in the list of UEs registered with a network slice. If the UE ID is found in the list of UEs already registered with the network slice or the maximum number of UEs per that network slice has not been reached yet, then the operation returns maximum number of UEs per network slice not reached result. Otherwise, it returns maximum number of UEs per network slice reached result.

Outputs, Required: maximum number of UEs per network slice reached or maximum number of UEs per network slice not reached result for each input S-NSSAI.

##### 5.2.xx.2.3	Nnsacf_NumberOfUEsPerSliceUpdate service operation

Service Operation name: Nnsacf_NumberOfUEsPerSliceUpdate

Description: Updates the number of UEs registered with a network slice.

Inputs, Required: S-NSSAI(s), UE ID, number of UEs update flag.

The S-NSSAI(s) parameter is a list of one or more network slices for which the number of UEs registered per network slice is to be updated.

The UE ID is used by the NSACF to check whether the UE ID is already in the list of UEs registered with the network slice.

The number of UEs update flag input parameter indicates whether the number of UEs registered with a network slice is to be increased, for example at registration or whether the number of UEs registered with a network slice to be decreased, for example at deregistration. However, if the number of UEs update flag indicates the number of UEs registered with a network slice is to be increased, the UE ID is found in the list of the UEs already registered with the network slice, then the number of UEs registered with that network slice is not increased as the UE already is counted as registered with that network slice.

Outputs, Required: none

* * * * end of 2nd change * * * *