3GPP TSG-SA WG2 Meeting #143E e-meeting	S2-2100980

# Elbonia, February 24 – March 09, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2591 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS23.502 KI#2 Network Slice Admission Control Function (NSACF) services and procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Apple, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Further to: - the eNS-Ph2 conclusions in the TR23.700-40 1.2.0, clause 8.2 for KI#2; and - a decision from SA2#143E meeting preparation telco on eNS-Ph2 to submit CRs based on the so-called push method. this contribution proposes the new feature addition to TS23.502. The changes are based on TR solution ‘6.10 Solution #10: Max number of PDU Sessions per Network Slice control via NSQ function’ which are fully compatible with the conclusions in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New eNS_Ph2 feature addition for K#2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eNS_Ph2 K#2 conclusions are not implemented in TS 23.502 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.x (new), 5.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR xxxx |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * start of 1st change * * * *

### 4.2.x	Network Slice Admission Control Function (NSACF) procedures

The NSACF is configured by the O&M with the maximum number of PDU Sessions that is allowed to be served by the network slice that is subject to NSAC. The NSACF controlls the number of the PDU Sessions per network slice so that the maximum number of PDU Sessions per network slice is not exceeded.

#### 4.2.x.1	Number of PDU Sessions per network slice availability check procedure

The number of PDU Sessions per network slice availability check procedure is triggered by the SMF during the PDU Session Establishment procedure to check whether the maximum number of PDU Sessions per network slice subject to NSAC has already been reached. The SMF is configured with the information indicating which network slice is subject to NSAC.

NSACF

SMF

2. Nnsacf_NumberOfPDUsPerSliceAvailabilityCheck_Request

4. Nnsacf_NumberOfPDUsPerSliceAvailabilityCheck_Response

3. The number of PDU Sessions per network slice availability check.

1. A trigger to perform number of PDU Sessions per network slice availability check during PDU Session Establishment procedure

5. Continue with the PDU Session Establishment procedure

Figure 4.2.x.1-1: Number of PDU Sessions per network slice availability check procedure

- During the PDU Session Establishment procedure, as per clause 4.3.2, if the request from the AMF is for PDU Session Establishment on a network slice that is subject to NSAC, after step 3 in clause 4.3.2.2.1 and 4.3.2.2.2, the SMF triggers a request to the NSACF for number of PDU Sessions per network slice availability check, i.e. to check whether the current number of PDU Sessions on the network slice has already reached the maximum number of PDU Sessions allowed for that network slice.

- The SMF sends Nnsacf_NumberOfPDUsPerSliceAvailabilityCheck_Request message to the NSACF. The SMF includes in the message the S-NSSAI which is subject to NSAC.

- The NSACF checks whether the current number of PDU Sessions established on the network slice has already reached the maximum number of PDU Sessions per network slice treshold for that network slice. If the maximum number of PDU Sessions for that S-NSSAI has not been reached, the NSACF returns maximum number of PDU Sessions per network slice not reached result. Otherwise, it returns maximum number of PDU Sessions per network slice reached result.

- The NSACF returns the Nnsacf_NumberOfPDUsPerSliceAvailabilityCheck_Response in which the NSACF includes the maximum number of PDU Sessions per network slice reached or not reached result parameter.

- The SMF continues with the PDU Session Establishment procedure. If the NSACF returned maximum number of PDU Sessions per network slice reached result, the SMF rejects the PDU Session establishment request with maximum number of PDU Sessions per network slice reached reject cause. If the NSACF returned maximum number of PDU Sessions per network slice not reached result, the SMF continues with the PDU Session Establishment procedure.

#### 4.2.x.2	Number of PDU Sessions per network slice update procedure

The number of PDU Sessions per network slice update procedure is triggered by the SMF during the PDU Session Establishment procedure to update (i.e. increase or decrease) the number of PDU Sessions per network slice subject to NSAC. The SMF is configured with the information indicating which network slice is subject to NSAC.

NSACF

SMF

2. Nnsacf_NumberOfPDUsPerSliceUpdate_Request

4. Nnsacf_NumberOfPDUsPerSliceUpdate_Response

3. The number of PDUs per network slice update.

1. A trigger to perform number of PDUs per network slice update

Figure 4.2.x.2-1: Number of PDU Sessions per network slice update procedure

- The AMF triggers the Number of PDU Sessions per network slice update procedure for the network slices that are subject to NSAC as a last step of a successful PDU Session Establishment procedure (clause 4.3.2.2.1 and clause 4.3.2.2.2) and as a last step of successful PDU Session Release procedure (clause 4.3.4.2 and clause 4.3.4.3).

- The SMF sends Nnsacf_NumberOfPDUsPerSliceUpdate_Request message to the NSACF. The SMF includes in the message the S-NSSAI for which the number of PDU Sessions per network slice update is required and the number of PDUs update flag which indicates that the numver of PDUs established on the S-NSSAI is to be increased if the procedure is triggered at the end of PDU Session Establishment procedure or indicates that the number of PDU Sessions on the S-NSSAI is to be decreased if the procedure is triggered at the end of PDU Sessions Release procedure.

- The NSACF updates the current number of PDU Sessions established on the S-NSSAI, i.e. increase or decrease the number of PDU Sessions per network slice based on the information orivide by the SMF.

If the number of PDUs update flag from the SMF indicates increase the current number of PDU Sessions per network slice, the NSACF increases the number of PDU Sessions for that network slice.

If the number of PDUs update flag from the SMF indicates decrease the current number of PDU Sessions per network slice, the NSACF decreases the number of PDU Sessions for that network slice.

- The NSACF acknowledges the update to the SMF.

* * * * end of 1st change * * * *

* * * * start of 2nd change * * * *

### 5.2.xx	Network Slice Admission Control Function (NSACF) services

#### 5.2.xx.1	General

The following table illustrates the NSACF services.

Table 5.2.xx-1: List of NSQCF services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnsacf_NumberOfPDUsPerSlice | AvailabilityCheck | Request/Response | SMF |
|  | Update | Request/Response | SMF |

##### 5.2.xx.2.1	Nnsacf_NumberOfPDUsPerSliceAvailabilityCheck service operation

Service Operation name: Nnsacf_NumberOfPDUsPerSliceAvailabilityCheck

Description: Checks whether the number of PDU Sessionss established on a network slice has reached the maximum number of PDU Sessions per network slice.

Inputs, Required: S-NSSAI

The S-NSSAI input parameter is the network slice for which the NSACF checks whether the number of the PDU Sessions already established on that network slice has reached the maximum number of PDU Sessions per network slice. If the maximum number of PDU Sessions per that network slice has not been reached yet, then the operation returns maximum number of PDU Sessions per network slice not reached result. Otherwise, it returns maximum number of PDU Sessions per network slice reached result.

Outputs, Required: result (mux number of PDU Sessions per network slice reached or mux number of PDU Sessions per network slice not reached result for the input S-NSSAI.)

##### 5.2.xx.2.2	Nnsacf_NumberOfPDUsPerSliceUpdate service operation

Service Operation name: Nnsacf_NumberOfPDUsPerSliceUpdate

Description: Updates the number of PDU Sessions established on a network slice.

Inputs, Required: S-NSSAI, number of PDUs update flag.

The S-NSSAI parameter is the network slice for which the number of PDU Sessions established on a network slice is to be updated.

The number of PDUs update flag input parameter indicates whether the number of the PDU Sessions established on that network slice is to be increased, for example at PDU Session Establishment procedure or the PDU Sessions established decreased, for example at PDU Session Release procedure.

Outputs, Required: none

* * * * end of 2nd change * * * *