3GPP TSG-WG SA2 Meeting #143e 	S2-2101040

# Feb 24th – March 9th, 2021 ; Elbonia                   							   (revision of S2-20XXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0159 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF Change Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_ph2 |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 6.4 on LMF change procedure allows AMF and LMF to determine new LMF. However, currently it is simply mentioned this is based on operator configuration and policy. Thus, it does not clarify and provide any details on how AMF and/or LMF may determine and select a different LMF i.e. one which is more suitable to the UE new location due to UE mobilty. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to LMF change procedure in clause 6.4, enabling AMF and/or LMF to query NRF and to select most suitable LMF in case of UE mobility e.g. based on LMF serving area, etc. Corresponding changes in clause 5.1 adding theses as additional LMF selection factors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AMF and/or LMF may not be able to select most suitable LMF e.g. in case of UE mobility |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN FIRST CHANGE *****

## 5.1	LMF Selection

LMF selection functionality is used by the AMF to determine an LMF for location estimation of the target UE. If an LMF ID is available in the UE location context or provided by the UE, the AMF should first evaluate if the LMF identified by the LMF ID can be used. The LMF selection functionality is also supported by the LMF if it determines that it is unsuitable or unable to support location for the current UE access network or serving cell for the deferred 5GC-MT-LR procedure for periodic, or triggered location events.

LMF reselection is a functionality supported by AMF when necessary, e.g. due to UE mobility.

The AMF selects a new LMF if:

-	AMF determines that LMF identified by the LMF ID cannot be used; or

-	the LMF ID is not available in the UE location context; or

-	the LMF ID is not provided by the UE.

The LMF selection may be performed at the AMF or LMF based on the locally available information i.e. LMF profiles are configured locally at AMF or LMF, or by querying NRF.

LMF selection is performed when:

-	a location request is received at the AMF and the AMF determines to use the LMF for UE position estimation; or

-	the subscribed UE event reporting is received.

The following factors may be considered during the LMF selection:

-	LCS client type.

-	Requested Quality of Service information, e.g.:

-	LCS accuracy,

-	Response time (latency),

-	Access Type (3GPP /N3GPP).

NOTE:	Location methods may differ depending on the Access Type, e.g. in the case of WLAN Access Location determination may just correspond to retrieval of IP addressing information from the N3IWF/TNGF; As another example, for Wireline access, Location determination may just correspond to retrieval of geo coordinates corresponding to a GLI as defined in TS 23.316 [21] clause 4.7.8 or a HFC Node ID.

-	RAT type (i.e. 5G NR or eLTE) and/or the serving AN node (i.e. gNB or NG-eNB) of the target UE.

-	RAN configuration information.

-	LMF capabilities.

-	LMF load.

-	LMF location.

-	Indication of either a single event report or multiple event reports.

-	Duration of event reporting.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

- LMF Service Area consisting of one or more TA(s)

***** END FIRST CHANGE *****

***** BEGIN SECOND CHANGE *****

## 6.4	LMF Change Procedure

The LMF Change procedure supports change of a serving LMF during a deferred 5GC-MT-LR procedure for periodic, or triggered location events as defined in clause 6.3.1. When a serving LMF is used for the procedure in clause 6.3.1, mobility of the target UE may lead to a change of serving AMF for which the original serving LMF is not suitable. For example, the serving LMF may be very remote from the new serving AMF leading to higher resource utilisation for AMF to LMF signalling or the LMF may not be configured with information (e.g. a cell database) for the current access network for the UE to enable location. In such a case, the serving LMF may need to change. Figure 6.4-1 shows a procedure to enable change of the serving LMF when a UE sends an event report as at step 25 in clause 6.3.1.

Figure 6.4-1: Change of serving LMF for periodic and triggered UE location events

Precondition:	Steps 1-23 in clause 6.3.1 have already been performed and steps 22-30 may have been performed or repeated. A serving LMF is used and the current serving LMF is LMF1.

1.	The UE performs a service request if needed as for step 24 in clause 6.3.1.

2.	The UE sends a NAS Transport message containing a supplementary services event report message to the serving AMF. The NAS Transport message includes a deferred routing identifier indicating LMF1. Step 2 corresponds to part of step 25 for clause 6.3.1.

3.	Based on operator configuration and policy, the AMF may evaluate and determine that LMF1 is unsuitable or unable to support location for the current UE access network or serving cell and determines LMF2 as being a more suitable LMF. AMF may already have LMF2 information e.g. from previous NRF discovery or locally configured, otherwise AMF queries NRF and in response may get a set of LMF profiles. AMF selects new LMF (i.e. LMF2 in this case) for the current UE location based on LMF service area (consisting of one or more TA(s)) and also other information as listed in clause 5.1.

4.	The AMF invokes the Namf_Communication_N1MessageNotify service operation towards LMF1. The service operation includes the event report received in step 2. If the AMF determined in step 3 that a new LMF2 should be used, it indicates that to the LMF1 as well.

5.	If the AMF did not indicate a new LMF in step 4, based on the operator configuration and policy, LMF1 may evaluate and determine that it is unsuitable or unable to support location for the current UE access network or serving cell and determines LMF2 as being a more suitable LMF. LMF may already have LMF2 information e.g. from previous NRF discovery or locally configured, otherwise AMF queries NRF and in response may get a set of LMF profiles. LMF selects new LMF (i.e. LMF2 in this case) for the current UE location based on LMF service area (consisting of one or more TA(s)) and other information as listed in clause 5.1.

6.	LMF1 invokes an Nlmf_Location_LocationContextTransfer Request service operation towards LMF2 to provide the current location context of the UE and includes the event report message received in step 4. The service operation includes the AMF identity and all the information originally received by LMF1 for the periodic or triggered location request either from the AMF according to the procedure in clause 6.3.1 or from an earlier serving LMF according to this procedure. The service operation may also include the current status of event reporting (e.g. the number of event reports so far received from the UE and/or the duration of event reporting so far) and may include location related information for the UE such a previous location estimate or location measurements.

7.	LMF2 informs LMF1 of the location context transfer operation results. LMF1 then releases all resources for the procedure.

8.	LMF2 invokes the Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request the transfer of a supplementary services Event Report Acknowledgment message to the UE. The Event Report Acknowledgment indicates a change of LMF and includes a deferred routing identifier indicating LMF2.

9.	The AMF forwards the Event Report Acknowledgment to the UE in a NAS Transport message. The AMF also informs the LMF2 of the result of the delivery of the Event Report Acknowledgement.

10.	If a location estimate is needed for event reporting, LMF2 may perform positioning of the UE and determines the UE location as at step 27 in clause 6.3.1. The rest of the procedure in clause 6.3.1 then continues from step 28 with LMF2 retaining state information to enable support of subsequent event reports from the UE.

***** END SECOND CHANGES *****