3GPP TSG-SA2 # 143E (e-meeting)	S2-2101386

# Febuarary 24 – March 9, 2021                                                       (revision of S2-2100483r02)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2513 | rev | 1 | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UP path selection enhancement based on analytics info provided by NWDAF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-03-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of TR23700-91 of FS_eNA_Ph2, “ KI#16: UP optimization for edge computing”, the Analytics IDs for “Observed service experience” is proposed for the normative work to optimize UP path selection. It is extended to include UPF information, DNAI and even AS instance information in order to allow NWDAF to be able to provide Service Experience per UP path. And the extended Observed service experience analytics may be provided to consumer SMF, AF and LDNSR to (re)select UPF or relocate DNAI or to determine the best AS instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [Change-1] Clause 4.3.6.2 Clarify the SMF can also take the Service Experience analytics per UP path into account to do UP path selection and traffic steering. [Change-2] Clause 4.3.6.4 Clarify the SMF can also take the Service Experience analytics per UP path into account to do UP path selection and traffic steering. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedures for UP path selection don’t incorporate the conclusion for FS_eNA_Ph2 KI#16. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.2, 4.3.6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

4.3.6.2	Processing AF requests to influence traffic routing for Sessions not identified by an UE address

Figure 4.3.6.2-1: Processing AF requests to influence traffic routing for Sessions not identified by an an UE address

NOTE 1:	The 5GC functions used in this scenario are assumed to all belong to the same PLMN (HPLMN in non-roaming case or VPLMN in the case of a PDU Session in LBO mode).

NOTE 2:	Nnef_TrafficInfluence_Create or Nnef_TrafficInfluence_Update or Nnef_TrafficInfluence_Delete service operations invoked from an AF located in the HPLMN for local breakout and home routed roaming scenarios are not supported.

1.	To create a new request, the AF invokes an Nnef_TrafficInfluence_Create service operation. The content of this service operation (AF request) is defined in clause 5.2.6.7. The request contains also an AF Transaction Id. If it subscribes to events related with PDU Sessions the AF indicates also where it desires to receive the corresponding notifications (AF notification reporting information).

To update or remove an existing request, the AF invokes an Nnef_TrafficInfluence_Update or Nnef_TrafficInfluence_Delete service operation providing the corresponding AF Transaction Id.

2.	The AF sends its request to the NEF. If the request is sent directly fom the AF to the PCF, the AF reaches the PCF selected for the existing PDU Session by configuration or by invoking Nbsf_management_Discovery service.

The NEF ensures the necessary authorization control, including throttling of AF requests and, as described in clause 4.3.6.1, mapping from the information provided by the AF into information needed by the 5GC.

3.	(in the case of Nnef_TrafficInfluence_Create or Update): The NEF stores the AF request information in the UDR (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = AF Transaction Internal ID, S-NSSAI and DNN and/or Internal Group Identifier or SUPI).

NOTE 3:	Both the AF Transaction Internal ID and, S-NSSAI and DNN and/or Internal Group Identifier or SUPI are regarded as Data Key when the AF request information are stored into the UDR, see Table 5.2.12.2.1-1.

(in the case of Nnef_TrafficInfluence_delete): The NEF deletes the AF requirements in the UDR (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = AF Transaction Internal ID).

The NEF responds to the AF.

4.	The PCF(s) that have subscribed to modifications of AF requests (Data Set = Application Data; Data Subset = AF traffic influence request information, Data Key = S-NSSAI and DNN and/or Internal Group Identifier or SUPI) receive(s) a Nudr_DM_Notify notification of data change from the UDR.

5.	The PCF determines if existing PDU Sessions are potentially impacted by the AF request. For each of these PDU Sessions, the PCF updates the SMF with corresponding new PCC rule(s) by invoking Npcf_SMPolicyControl_UpdateNotify service operation as described in steps 5 and 6 in clause 4.16.5.

If the AF request includes a notification reporting request for UP path change, the PCF includes in the PCC rule(s) the information required for reporting the event, including the Notification Target Address pointing to the NEF or AF and the Notification Correlation ID containing the AF Transaction Internal ID.

The PCF may, optionally, use service experience analytics per UP path, as defined in clause 6.4.3, TS 23.288 [50], to provide an updated list of DNAI(s) to the SMF.

6.	When a PCC rule is received from the PCF the SMF may take appropriate actions, when applicable, to reconfigure the User plane of the PDU Session. The SMF may consider service experience analytics per UP path (i.e. including UPF and/or DNAI and/or AS instance) as defined in clause 6.4.3, TS 23.288[50] before taking such actions. Examples of actions are:

-	Determing a target DNAI and adding, replacing or removing a UPF in the data path to e.g. act as an UL CL or a Branching Point e.g. as described in clause 4.3.5.

-	Allocate a new Prefix to the UE (when IPv6 multi-Homing applies).

-	Updating the UPF in the target DNAI with new traffic steering rules.

-	Subscribe to notifications from the AMF for an Area of Interest via Namf_EventExposure_Subscribe service operation.

* * * Next Changes * * *

4.3.6.4	Transferring an AF request targeting an individual UE address to the relevant PCF

Figure 4.3.6.4-1: Handling an AF request targeting an individual UE address to the relevant PCF

Depending on the AF deployment (see TS 23.501 [2], clause 6.2.10), the AF may send the AF request to PCF directly, in which case step 1 is skipped, or via the NEF.

1.	[Conditional] If the AF sends the AF request via NEF, the AF sends Nnef_TrafficInfluenceCreate/Update/Delete Request targeting an individual UE address to the NEF. This request corresponds to an AF request to influence traffic routing that targets an individual UE address.

When NEF receives an AF request from AF, the NEF ensures the necessary authorization control and, as described in clause 4.3.6.1, mapping from the information provided by the AF into information needed by the 5GC. The NEF responds to the AF.

2.	[Conditional] AF/NEF consumes Nbsf_Management_Discovery service operation (providing at least the UE address) to find out the address of the relevant PCF if the PCF address is not available on the NEF based on local configuration, otherwise step 1 is skipped.

NOTE:	The AF/NEF finds the BSF based on local configuration or using the NRF.

3.	BSF provides the PCF address in the Nbsf_Management_Discovery response to AF/NEF.

4.	If step 1 was performed, NEF invokes the Npcf_PolicyAuthorization service to the PCF to transfer the AF request. If an AF sends the AF request directly to the PCF, AF invokes Npcf_PolicyAuthorization service and the PCF responds to the AF.

5.	The PCF updates the SMF with corresponding new PCC rule(s) with PCF initiated SM Policy Association Modification procedure as described in clause 4.16.5.2.

The PCF may, optionally, use service experience analytics per UP path, as defined in clause 6.4.3, TS 23.288 [50], to provide a an updated list of DNAI(s) to the .

When a PCC rule is received from the PCF, the SMF may take appropriate actions, when applicable, to reconfigure the User plane of the PDU Session. The SMF may consider service experience analytics per UP path (i.e. including UPF and/or DNAI and/or AS instance) as defined in clause 6.4.3, TS 23.288 [50], before taking such actions. Examples of actions are:

-	Determing a target DNAI and adding, replacing or removing UPF(s) in the data path, e.g. to act as UL CL, Branching Point, and/or PDU Session Anchor e.g. as described in clause 4.3.5.

-	Allocate a new Prefix to the UE (when IPv6 multi-Homing applies).

-	Updating the UPF regarding the target DNAI with new traffic steering rules.

-	Subscribe to notifications from the AMF for an Area of Interest via Namf_EventExposure_Subscribe service operation.

* * *End of Changes * * *