SA WG2 Meeting #143E e-meeting	S2-2101427

Elbonia, February 24 – March 09, 2021

Source:	Samsung

Title:	Service level definitions

Document for:	Approval

Agenda Item:	8.9.2

Work Item / Release:	5MBS / Rel-17

Abstract: This document adds service level definitions to the new TS based on KI#2 conclusion.

# Introduction

This document adds service level definitions to the new TS based on KI#2 conclusion of TR 23.757.

# Proposal

It is proposed to capture the following changes in TS23.247.

* * * * Start of 1st Change * * * *

# 2	References

[X]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

* * * * Start of 2nd Change * * * *

## 4.1	Principles of multicast and broadcast communication

Editor’s Note:	It is FFS whether the text below will be moved to a different clause of the TS.

The following service levels for the multicast communication service are defined:

NOTE 1:	Transport Only mode and Full-Service mode of operation as defined in TS 23.246 [X] differ from the service levels defined here.

-	Basic service level. The following requirements are defined:

-	Media transported transparently through the 5GS.

-	Request to receive the multicast service.

-	Packet distribution from the 5GS ingress to (R)AN node(s).

-	Data delivery from (R)AN node(s) to the UE.

-	Enhanced service level, with additional requirements on top of basic service level. Different requirements out of the set below may be necessary to address each use case:

-	Local MBS service.

-	User authentication and authorization.

NOTE 2:	User authentication and authorization can be done by 5GS or AF or both, or even not needed for a multicast communication service.

-	Explicit configuration of multicast session by network function external to the 5GS including Group member management.

-	Enhanced QoS support.

NOTE 3:	5G MBS can provide different QoS other than default QoS for different multicast groups.

The 5GC Shared MBS traffic delivery method is required in all 5G MBS deployments. The 5GC Individual MBS traffic delivery method is required to enable mobility when there is an NG-RAN deployment with non-homogeneous support of 5G MBS.

Editor’s Note:	The above description of requirements may change during the normative phase.

* * * * End of Changes * * * *